| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0179-v1 |
| --- | --- |

| Title: | AHG15: On auxiliary information for the Unity Gaming sequences proposed by InterDigital |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Thiebaud, S. Puri, T. Poirier, F. Galpin | Email: | sylvain.thiebaud@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Auxiliary information associated with gaming content may be used to improve the compression efficiency. These improvements may be obtained in term of BD-rate, or in term of encoder speed for instance.

This contribution proposes to provide some of these auxiliary information provided by the game engine Unity, such as camera parameters, depth map and motion vector maps. This information may be provided in associated text files and EXR files, synchronized with the gaming sequences. To understand the meaning of these auxiliary information, a brief introduction to what they represent in the game engine, and how they can be interpreted is provided. The process to compute the 3D depth of the scene from the depth map and some camera parameters is also described.

# Introduction

Associated with the gaming content, this contribution proposes to provide:

- Some game engine’s camera parameters.

- Depth maps.

- Motion vectors maps.

The camera parameters may be provided in a text file. They are synchronized with the gaming sequence, to provide game engine’s camera parameters per frame. To understand the meaning of these parameters, paragraph 1 briefly introduces how the game engine works, and how its virtual camera performs the projection.

Depth map and the motion vectors maps can be provided by the game engine in floating point representation. This contribution proposes to store these maps in EXR files, one file per gaming frame. Since the depth map provided by the game engine does not represent the real depth of the 3D scene, paragraph 2 describes the way this 3D information may be computed from the map and parameters provided by the game engine.

## 3D model and matrices in the game engine

A 3D engine (Unity for instance in our case) uses a virtual camera to project the 3D scene onto a plane, a 2D image. In the pinhole camera model Figure 1, the physical characteristics of the camera (focal length, sensor size, field of view, …) can be used to compute a projection matrix. This matrix, also called the intrinsic matrix of the camera, defines where a point in the 3D space is projected in the 2D image. For instance, the 3D point P (X, Y, Z) of the 3D object is projected in the 2D image at position Pi (x, y). The way this matrix is built is described below. More details are available in OpenGL, DirectX, or generic documentation about projection matrices [1].

Camera

optical center

Xcam

Ycam

Zcam

P(X,Y,Z)

Pi(x,y)

Image plane

Focal length f

Camera

coordinate system

3D world

coordinate system

3D object

Figure 1 : The pinhole camera model.

Instead of projecting the 3D points located between 0 and the infinity like a physical camera does, a game engine camera only projects the 3D points located between two planes, the near and the far planes in Figure 2. It means that these two planes represent the minimal and maximal depth used for the rendering. The near plane (ZN) is usually mapped to depth 0 and the far plane (ZF) to depth 1. These 2 planes represent the limits of a truncated pyramid of the viewing volume. Then, this volume is mapped to Normalized Device Coordinates (NDC), from coordinates (-1,-1,-1) to (1,1,1). We will see later that to be used properly, the projection matrix must be applied to pixels expressed in this NDC representation.

Xcam

Ycam

Zcam

Camera

coordinate system

3D world

coordinate system

Far plane

Near plane

ZF

ZN

Viewing volume

Viewing volume in NDC coordinates

(-1,-1,-1)

(1,1,1)

NDC mapping

Figure 2 : Game engine’s camera Near and Far planes, viewing volume in NDC coordinates.

The camera projection matrix performs its projection relatively to its own coordinate system (camera coordinate system in Figure 1 and Figure 2). Since the camera is not placed at the origin of the 3D world coordinate system, another matrix is required to pass from the 3D world coordinate system to the camera coordinate system. This is the world to camera matrix, used to represent the rotation and the translation of the camera relatively to the 3D world coordinate system. The inverse transformation is the camera to world matrix used to pass from the camera coordinate system to the 3D world coordinate system.

## Projection matrix

The 4 x 4 projection matrix (also called the intrinsic matrix of the camera) can be extracted from Unity or can be computed thanks to camera parameters.

The property “projectionMatrix” of the Unity Camera API [2] returns the 16 coefficients of the current projection matrix.

This matrix can also be computed with some camera parameters: the Field of View (FoV), the Znear (ZN) and the Zfar (ZF) plane positions (cf. Figure 1 and Figure 2), and the aspect ratio (or the Width (H) and Height (V) of the image). These parameters correspond respectively to the Unity camera properties “fieldOfView”, “nearClipPlane”, “farClipPlane”, and “aspect” (or “pixelWidth” and “pixelHeight”) provided by the Unity camera API.

The projection matrix may be expressed as:

1tanFoV2.HV00001tanFoV20000-ZF+ZNZF-ZN-2ZFZNZF-ZN00-10

This matrix defines the way a point in the 3D world is projected in the 2D image provided by the virtual camera.

This contribution proposes to provide this projection matrix as auxiliary information.

## World to Camera matrix

The camera can be placed anywhere in the 3D world (not especially at its origin) and with any orientation. The translation and rotation of the camera in the 3D world coordinate system are represented by the World to Camera matrix, or its inverse matrix the Camera to World matrix.

The properties “worldToCameraMatrix” and “cameraToWorldMatrix” of the Unity Camera API [2] represent the 16 coefficients of these matrices.

The way these matrices are computed is not detailed here, but the following 4 x 4 Translation Scale Rotation (TSR) matrix indicates how it is composed.

TSR matrix composition : 		R0R1R2T0R3R4R5T1R6R7R8T20001

The 9 Ri coefficients represent the rotations of the camera around the 3 axes. They can be computed with the 3 rotation parameters of the camera (one per axis). The 3 Ti coefficients represent the translation relatively to the 3D world origin.

This contribution proposes to provide this worldToCamera matrix as auxiliary information.

# Real Depth computation

## Depth buffer in Unity

The depth buffer in Unity does not directly represent the distance between an object and the camera. It contains information representing the non-linear depth, obtained by the transformation of a 3D world point by the projection matrix. So, it can be used as the third coordinate of a 2D image point on which we would apply a de-projection matrix.

Note that the de-projection matrix must be applied to data expressed in Normalized Device Coordinates (NDC viewing volume Figure 2), in the range [-1,1]. It means that the depth buffer provided by Unity, expressed in the range [0, 1], must be converted in NDC, range [-1, 1].

In the same way, the x and y coordinates of the image point Pi(x, y) must be mapped in the range [-1,1], to be expressed in NDC coordinates.

Usually, in the depth buffer, 0 represents the Znear plane position and 1 represents the Zfar plane position. But depending on the platform, the depth buffer may be represented in the inverse way. Unity provides the macro UNITY_REVERSED_Z to determine the way the depth buffer is coded. In our case, the depth buffer is reversed: 1 - Zbuf must be applied to obtain a Z buffer in the range [0, 1], where Znear is 0 and Zfar is 1.

In short, the Unity reversed Z buffer ZbufUnity must be expressed in the range [0, 1] to map Znear and Zfar to the correct values, and expressed in NDC coordinates to be used for de-projection:

zbuf=1- zbufUnity 	(To map Znear to 0, Zfar to 1)

zNDC=2.zbuf- 1 	(To map in NDC, range [-1, 1])

## Depth buffer to real depth (in the camera coordinate system)

To use the de-projection matrix, the coordinates must be expressed in Normalized Device Coordinates: the values of x, y, and ZbufUnity must be defined in the range [-1,1], as described above. A 2D NDC pixel Pi(x, y), associated to its ZNDC data must be expressed in homogeneous coordinates, by the four coordinates vector x, y,ZNDC,wT.

In this system, if we consider only the coefficients relative to the depth mapping, the camera projection matrix is:

xyzNDCw=........00-ZF+ZNZF-ZN-2ZFZNZF-ZN00-10*XYZ1=> zNDC=-ZF+ZNZF-ZN.Z-2ZFZNZF-ZNw=-Z

Where ZN and ZF represent the depth of the near plane and the far plane of the Figure 2.

To represent a 3D cartesian point, the fourth coordinate w must be equal to 1: Both equations must be divided by -Z.

- zNDC=ZF+ZNZF-ZN+2ZFZNZF-ZN 1Zw=1

So, this contribution proposes to provide a depth map coded as:   zNDC=aZ+b

With (in our case of Unity): b=ZF+ZNZF-ZN and a=2ZFZNZF-ZN

ZN and ZF are camera parameters provided as auxiliary information, as described in paragraph 3. The coefficients a and b may also be provided by the projection matrix.

Conversely:

Z=-2ZFZNZF+ZN+zNDC.(ZN-ZF)

Note that the z axes of the camera and the 3D word are oriented in opposite directions, implying a negative Z relative to the camera coordinate system.

Since zNDC=2.zbuf- 1, Z can also be expressed as:

Z=-ZFZNZF+zbuf.(ZN-ZF)

# Auxiliary information format

In this section, the details on the format of auxiliary information are provided.

## Camera parameters in text file

A text file providing some camera parameters and/or matrices is associated with the gaming content generated by Unity. This information is provided per frame. An example is provided below:

Version 0.0 - projectionMatrix, worldToCameraMatrix, farClipPlane, nearClipPlane, focalLength

{"frame":0,"projectionMatrix":{"e00":1.0805524587631226,"e01":0.0,"e02":0.0,"e03":0.0,"e10":0.0,"e11":1.9209821224212647,"e12":0.0,"e13":0.0,"e20":0.0,"e21":0.0,"e22":-1.000400185585022,"e23":-0.6001200675964356,"e30":0.0,"e31":0.0,"e32":-1.0,"e33":0.0},"worldToCameraMatrix":{"e00":0.9910848140716553,"e01":0.08583169430494309,"e02":0.10190098732709885,"e03":11.727930068969727,"e10":-0.10190098732709885,"e11":0.981060266494751,"e12":0.16473309695720673,"e13":-42.39990997314453,"e20":0.08583169430494309,"e21":0.17364828288555146,"e22":-0.981060266494751,"e23":-10.99787712097168,"e30":0.0,"e31":0.0,"e32":0.0,"e33":1.0},"farClipPlane":1500.0,"nearClipPlane":0.30000001192092898,"focalLength":23.05178451538086}

{"frame":1,"projectionMatrix":{"e00":1.0805524587631226,"e01":0.0,"e02":0.0,"e03":0.0,"e10":0.0,"e11":1.9209821224212647,"e12":0.0,"e13":0.0,"e20":0.0,"e21":0.0,"e22":-1.000400185585022,"e23":-0.6001200675964356,"e30":0.0,"e31":0.0,"e32":-1.0,"e33":0.0},"worldToCameraMatrix":{"e00":0.9901020526885986,"e01":0.08996792137622833,"e02":0.10772034525871277,"e03":12.301990509033204,"e10":-0.10772049427032471,"e11":0.9791308045387268,"e12":0.1723344624042511,"e13":-42.77192306518555,"e20":0.08996772766113281,"e21":0.18223240971565247,"e22":-0.9791308045387268,"e23":-11.536568641662598,"e30":0.0,"e31":0.0,"e32":0.0,"e33":1.0},"farClipPlane":1500.0,"nearClipPlane":0.30000001192092898,"focalLength":23.05178451538086}

…

The first line is informative, providing the version of the auxiliary file, and the list of the elements available per frame. In the example, the Projection and the WorldToCamera matrices are provided, associated with the far (called farClipPlane in the Unity API) and the near planes (nearClipPlane), and the focal length.

Then each line provides auxiliary information synchronized to the gaming content frames. Each line is a JSON object, starting with the frame index, and followed by auxiliary elements. The projection matrix and the world to camera matrix are composed of 16 elements. Here, the inverse matrices are not provided. In this case, if required, they can be computed from the direct matrices.

## Depth and Motion vectors in EXR file

Unity can provide a depth and a motion vectors maps in floating point. For convenience, the depth and motion vectors information have been packed in EXR files.

The depth is stored in the red channel, the horizontal component of the motion vector in the green channel, and the vertical component in the blue channel.

# References

[1] The Perspective and Orthographic Projection Matrix - https://www.scratchapixel.com/lessons/3d-basic-rendering/perspective-and-orthographic-projection-matrix/projection-matrix-introduction.html

[2] Unity - Scripting API: Camera - https://docs.unity3d.com/ScriptReference/Camera.html

# Patent rights declaration(s)

InterDigital Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).