| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0182 |
| --- | --- |

| Title: | Non-EE2: On intra and inter LFNST/NSPT for low delay configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes an adaptation of intra and inter LFNST/NSPT for low-delay configuration. The intra and inter LFNST/NSPT are currently disabled in CTC for AHG12/EE2 due to increased encoder runtime over ECM. Therefore, this proposal suggests early termination of intra LFNST/NSPT based on the comparison of the best rate-distortion (RD) cost at the end of inter coding and the RD cost of intra coding without using LFSNT/NSPT for low-delay configuration.

On top of the ECM-12.0, simulation results {Y, U, V, EncT, DecT} of ECM-12.0 with enabling intra and inter LFNST/NSPT (Test 1) and Test 1 with the proposed early termination (Test 2) for overall test sequences in CTC are reported below:

Test 1: ECM-12 with enabling intra and inter LFNST/NSPT:

LDB: {-1.58%, -0.40%, -0.88%, 115.1%, 101.4%};

LDP: {-1.71%, -0.55%, -0.80%, 125.5%, 101.2%};

Test 2: Test 1 with the proposed early termination:

LDB: {-1.52%, -0.14%, -0.64, 106.5%, 101.6%};

LDP: {-1.67%, -0.27%, -0.24%, 109.8%, 101.6%}.

Introduction

In JVET-AG meeting, inter LFNST/NSPT, proposed by JVET-AG0061, was adopted in random access configuration of CTC for AHG12/EE2, but inter LFNST/NSPT in addition to intra LFNST/NSPT was decided to disable in low delay configuration due to significant increased encoder runtime over ECM. Furthermore, in JVET-AD meeting, JVET-AD0172 proposed an adaptation of only intra LFNST/NSPT for low delay configuration with introducing picture-header-level control flag of intra LFNST/NSPT to reduce encoder runtime and also save the signalling overhead of CU-level control flag of intra LFNST/NSPT. However, according to the meeting note of JVET-AD, since the experimental results for the additional gain introduced by the picture-level control flag compared to the non-normative approach were not presented and the benefit was unclear, the JVET-AD0172 was not adopted in the ECM.

Proposed method

This contribution proposes an early termination of intra LFNST/NSPT based on the comparison of the best rate-distortion (RD) cost at the end of inter coding and the RD cost of intra coding without using LFSNT/NSPT for low-delay configuration.

Experimental results

The proposed methods were implemented on top of ECM-12.0 [3] and tested according to the common test condition of low delay B (LB) and low delay P (LP) configurations as specified in the JVET-AF2017 [4] and JVET-AG2024 [5].

To evaluate the effect of the proposed early termination, the following two tests were conducted.

- Test 1: ECM-12 with enabling intra and inter LFNST/NSPT;

- Test 2: Test 1 with the proposed early termination.

Experimental results of Test 1 and Test 2 are summarized below. More detailed results can be found in the attached spreadsheets.

Table 1 Test 1 results over ECM-12.0.

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -1.31% | -0.44% | -0.61% | 116.0% | 101.2% | 100.3% | 95.7% |
| Class C | -2.06% | -0.67% | -0.39% | 117.3% | 101.9% | 100.3% | 98.6% |
| Class E | -1.40% | 0.04% | -1.97% | 110.7% | 101.1% | 99.5% | 100.0% |
| Overall | -1.58% | -0.40% | -0.88% | 115.1% | 101.4% | 100.1% | 97.7% |
| Class D | -1.30% | 0.55% | -0.93% | 114.4% | 101.5% | 100.6% | 99.6% |
| Class F | -1.20% | -1.16% | -1.59% | 110.3% | 100.9% | 101.9% | 98.3% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -1.41% | -0.41% | -0.40% | 127.4% | 100.8% | 100.2% | 99.5% |
| Class C | -2.13% | -0.72% | -0.80% | 130.1% | 100.7% | 100.3% | 100.1% |
| Class E | -1.67% | -0.57% | -1.49% | 116.6% | 102.4% | 101.3% | 100.0% |
| Overall | -1.71% | -0.55% | -0.80% | 125.5% | 101.2% | 100.5% | 99.8% |
| Class D | -1.47% | -0.84% | 0.01% | 124.6% | 101.1% | 100.2% | 100.0% |
| Class TGM | -1.22% | 0.40% | -0.41% | 115.6% | 100.8% | 100.0% | 99.8% |

Table 2 Test 2 results over ECM-12.0.

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -1.23% | -0.16% | -0.59% | 106.6% | 101.6% | 100.3% | 95.5% |
| Class C | -1.92% | -0.26% | -0.13% | 107.7% | 102.1% | 100.4% | 98.7% |
| Class E | -1.45% | 0.05% | -1.43% | 104.8% | 101.2% | 99.4% | 99.8% |
| Overall | -1.52% | -0.14% | -0.64% | 106.5% | 101.6% | 100.1% | 97.6% |
| Class D | -1.28% | 0.57% | -1.70% | 106.3% | 102.5% | 100.5% | 99.6% |
| Class F | -1.02% | -0.60% | -1.56% | 104.8% | 100.9% | 100.8% | 98.2% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -1.36% | -0.01% | 0.17% | 110.4% | 101.2% | 100.2% | 99.5% |
| Class C | -2.11% | -0.65% | -0.02% | 112.0% | 101.3% | 100.3% | 100.1% |
| Class E | -1.59% | -0.21% | -1.22% | 105.8% | 102.8% | 101.3% | 100.0% |
| Overall | -1.67% | -0.27% | -0.24% | 109.8% | 101.6% | 100.5% | 99.8% |
| Class D | -1.50% | -1.06% | -0.07% | 109.3% | 101.4% | 100.5% | 100.0% |
| Class F | -1.41% | 0.16% | -0.57% | 106.4% | 101.0% | 100.0% | 99.9% |

Conclusion

This contribution proposes an early termination of intra LFNST/NSPT based on the comparison of the best rate-distortion (RD) cost at the end of inter coding and the RD cost of intra coding without using LFSNT/NSPT for low-delay configuration. The proposed method brings some coding gains with reducing encoder runtime compared to ECM-12.0 with simply enabling intra and inter LFNST/NSPT. It is recommended to adopt the proposed method into the next EE.

Reference

- F. Wang, J. Gan, Y. Yu, H. Yu, D. Wang, “	EE2-3.3: Utilizing LFNST/NSPT for inter coding,” JVET-AG0172, January 2024.

- C. Bonnineau, F. Le Léannec, K. Naser, T. Poirier, S. Puri, “AHG12: Adaptation of LFNST/NSPT for low-delay configuration,” JVET-AD0172, Apr. 2023.

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October. 2023.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AG2024, January 2024.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).