| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0183-v1 |
| --- | --- |

| Title: | EE2-related: TMVP for chained motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin Zhi Zhang Jian-Liang Lin Vadim Seregin Marta Karczewicz | Tel: Email: | pohanlin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal presents the results of an enhanced chained motion vector prediction (CMVP) method on top of Test 3.1. In Test 3.1, CMVP candidates are derived by recursively tracing the MVs or BVs from a candidate in the inter merge candidate list. This contribution proposes to derive CMVP from TMVP which is always scaled to reference index 0 picture. On top of ECM-12.0, simulation results are reported as follows:

RA {-0.xx% Y, -0.xx% U, -0.xx% V, xxx% EncT, xxx% DecT }

LB {-0.xx% Y, -0.xx% U, -0.xx% V, xxx% EncT, xxx% DecT }

# Introduction

In Test 3.1, CMVP candidates are be derived as the accumulation of the recursively traced MVs and BVs based on the pre-derived MVs for the inter merge candidate list. For instance, a CMVP candidate, a set of motion vector MVk/m and reference picture RefPick/m can be derived by

MVk/m = MVk(0) + MVk(1) +MVk(2) + … + MVk(m),

RefPick/m = RefPick(m),

where k and m indicate the number of merge index and trace depths of the CMVP. Figure 1 shows the example of CMVP derivation.

Figure 1 Example of derivation of CMVP candidate

In Test 3.1, the trace depths m is set to 1.

# Proposed method

This contribution proposes to add TMVP with zero reference index if the existed TMVP reference index is not zero. When deriving CMVP from a TMVP merge candidate, the TMVP candidate will be scaled to the first reference picture with reference index is equal to 0. The scaled motion vector is stored and used to derive the CMVP candidates. Besides, CMVP is also extended to BMmerge mode.

# Experimental results

The proposed method has been implemented on top of EE2 Test 3.1 software. It is tested based on the common test condition as specified in the JVET document [1]. The test results are summarized below.

Table 1: Coding performance of the proposed method over ECM-12.0

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class C | -0.27% | -0.46% | -0.40% | #NUM! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.25% | -0.34% | -0.20% | #NUM! | #NUM! | #NUM! |
| Class F | -0.17% | -0.18% | -0.36% | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

# Conclusions

The simulation results show -0.xx%/-0.xx%/-0.xx% gain on Y/Cb/Cr with xxx%/xxx% encoding/decoding time in RA condition, -0.xx%/-0.xx%/-0.xx% gain on Y/Cb/Cr with xxx%/xxx% encoding/decoding time in LB condition. It is recommended to include the proposed methods in the next round of EE.

# References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).