| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0186-v2 |
| --- | --- |

| Title: | EE2-3.5: LIC model inheritance for merge modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Han Huang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | chunchic@qti.qualcomm.com hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the testing results of Test 3.5 (LIC model inheritance for merge modes) in the context of Exploration Experiments 2 (JVET-AG2024). Three tests were studied. Test 3.5a enables LIC (Local Illumination Compensation) model inheritance for GPM merge modes to infer LIC model parameters from merged candidate motion with LIC enabled. Then, the notion of LIC inheritance is further extended to regular merge modes for Test 3.5b and 3.5c, that perform mode decision in LIC flag swapping stage and ARMC stage, respectively. The experimental results on top of ECM-12.0 are reported, as follows:

Test 3.5a: LIC model inheritance for GPM

-- RA (Y/U/V): -0.03% / 0.02% / 0.04%, EncT 99.9%, DecT 100.0%.

-- LDB (Y/U/V): -0.08% / -0.20% / -0.10%, EncT 100.0%, 99.8%.

Test 3.5b: Test 3.5a + LIC model inheritance for merge modes at LIC flag swapping stage

-- RA (Y/U/V): -0.03% / -0.01% / 0.03%, EncT 99.4%, DecT 99.9%.

-- LDB (Y/U/V): -0.09% / -0.42% / -0.21%, EncT 99.9%, DecT 99.8%.

Test 3.5c: Test 3.5a + LIC model inheritance for merge modes at ARMC stage

-- RA (Y/U/V): -0.04% / 0.04% / 0.07%, EncT 100.1%, DecT 99.9%.

-- LDB (Y/U/V): -0.05% / -0.03% / -0.16%, EncT 100.1%, DecT 100.0%.

# Introduction

In the merge candidate list construction process of ECM, LIC flag may be inferred from causally neighboring blocks’ motion information stored in the motion field. Once inferred, the flag is set together with merge candidate motion and serves as the only indication telling the ECM codec whether LIC model parameters are going to be derived on-the-fly and applied to the motion-compensated predictors of template or prediction block of the current block. Both the LIC flags and model parameters are stored in motion field and/or CU data. However, OBMC is the only coding tool in ECM that refers to it. Thus, JVET-AG0195 proposed to make the most of the stored LIC information by inheriting model parameters in addition to LIC flag value when constructing merge candidate lists.

# Test description

## Test 3.5a: LIC model inheritance for GPM

LIC model parameters from causally neighboring blocks could be inherited conditionally, when they have LIC flags determined to be true, in addition to motion information during the construction process of GPM merge candidate list. Accordingly, LIC could be enabled for GPM without the model parameter derivation in motion compensation.

## Test 3.5b: Test 3.5a + LIC model inheritance for merge modes at LIC flag swapping stage

On top of Test 3.5a, this test additionally extends the notion of LIC model inheritance to regular merge modes. When merge list is constructed, JVET-AF0128 (adopted since ECM 11.0) further optimizes LIC flag value through template cost to determine whether the flag value could be swapped to the alternative value (e.g. true to false or false to true). Test 3.5b conditionally could compete a new merge candidate (that could inherit LIC model from causal neighborhood) with its corresponding others in the LIC swapping stage. Specifically, the LIC swapping stage could swap among three states, including LIC off, LIC on, LIC inheritance.

## Test 3.5c: Test 3.5a + LIC model inheritance for merge modes at ARMC stage

On top of Test 3.5a, this test also extends the notion of LIC model inheritance to regular merge modes. The new merge candidates (that could inherit LIC model from causal neighborhood) are competing jointly with all other merge candidates in the ARMC stage.

# Simulation result

The proposed method was implemented on top of the ECM-12.0 software [1] and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AG2024). The SW is submitted to the JVET git repository (https://vcgit.hhi.fraunhofer.de/ecm/jvet-ag-ee2/ECM/) at the branch TEST_3.5 for conducting the three EE tests, respectively. The testing results of Tests 3.5a, 3.5b, and 3.5c are summarized below.

- Performance results of Test-3.5a relative to ECM-12.0 anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Enc Vm | Dec Vm |
| Class A1 | -0.08% | -0.15% | -0.03% | 99.7% | 99.9% | 100.0% | 100.2% |
| Class A2 | 0.04% | 0.14% | 0.10% | 100.0% | 100.1% | 100.0% | 100.2% |
| Class B | -0.05% | 0.12% | 0.07% | 99.8% | 99.9% | 99.9% | 100.2% |
| Class C | 0.00% | -0.04% | 0.02% | 99.9% | 100.2% | 100.0% | 100.0% |
| Class E | x | x | x | x | x | x | x |
| Overall | -0.03% | 0.02% | 0.04% | 99.9% | 100.0% | 100.0% | 100.1% |
| Class D | 0.07% | 0.09% | 0.15% | 100.0% | 100.1% | 100.2% | 99.9% |
| Class F | -0.06% | -0.03% | -0.10% | 99.8% | 99.9% | 100.7% | 100.2% |
| Class TGM | -0.05% | -0.03% | -0.07% | 99.9% | 100.0% | 99.5% | 100.6% |

|  | Low Delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Enc Vm | Dec Vm |
| Class A1 | x | x | x | x | x | x | x |
| Class A2 | x | x | x | x | x | x | x |
| Class B | -0.10% | -0.18% | 0.05% | 99.9% | 99.7% | 99.5% | 101.0% |
| Class C | -0.05% | 0.00% | 0.01% | 100.1% | 100.0% | 100.5% | 100.0% |
| Class E | -0.08% | -0.49% | -0.48% | 99.9% | 99.7% | 100.0% | 99.7% |
| Overall | -0.08% | -0.20% | -0.10% | 100.0% | 99.8% | 99.9% | 100.3% |
| Class D | -0.09% | -0.20% | -0.72% | 100.1% | 99.8% | 100.3% | 100.1% |
| Class F | 0.03% | -0.72% | -0.78% | 100.2% | 100.1% | 101.2% | 100.5% |
| Class TGM | -0.19% | -0.11% | -0.08% | 100.3% | 100.2% | 99.1% | 100.8% |

- Performance results of Test-3.5b relative to ECM-12.0 anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Enc Vm | Dec Vm |
| Class A1 | -0.08% | -0.15% | -0.03% | 99.4% | 99.8% | 100.0% | 100.2% |
| Class A2 | 0.04% | 0.14% | 0.10% | 99.3% | 100.1% | 100.0% | 100.2% |
| Class B | -0.06% | -0.07% | 0.03% | 99.4% | 99.8% | 100.2% | 100.1% |
| Class C | 0.01% | 0.06% | 0.00% | 99.6% | 99.9% | 99.6% | 100.0% |
| Class E | x | x | x | x | x | x | x |
| Overall | -0.03% | -0.01% | 0.03% | 99.4% | 99.9% | 100.0% | 100.1% |
| Class D | 0.06% | -0.05% | -0.01% | 99.8% | 100.1% | 100.1% | 99.9% |
| Class F | -0.06% | -0.10% | 0.02% | 99.6% | 99.8% | 100.2% | 100.1% |
| Class TGM | -0.13% | -0.14% | -0.22% | 99.7% | 99.9% | 99.3% | 100.5% |

|  | Low Delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Enc Vm | Dec Vm |
| Class A1 | x | x | x | x | x | x | x |
| Class A2 | x | x | x | x | x | x | x |
| Class B | -0.16% | -0.30% | -0.13% | 99.9% | 99.8% | 99.2% | 101.0% |
| Class C | -0.07% | -0.30% | -0.01% | 100.0% | 99.5% | 101.0% | 100.0% |
| Class E | 0.00% | -0.78% | -0.63% | 99.8% | 100.1% | 101.0% | 99.8% |
| Overall | -0.09% | -0.42% | -0.21% | 99.9% | 99.8% | 100.3% | 100.3% |
| Class D | -0.17% | 0.05% | -0.43% | 100.4% | 100.2% | 100.2% | 100.0% |
| Class F | -0.12% | -0.63% | -0.60% | 100.3% | 100.7% | 102.4% | 100.5% |
| Class TGM | -0.15% | -0.26% | -0.22% | 100.1% | 100.3% | 98.5% | 100.9% |

- Performance results of Test-3.5c relative to ECM-12.0 anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Enc Vm | Dec Vm |
| Class A1 | -0.11% | -0.01% | 0.00% | 99.9% | 100.0% | 100.0% | 100.1% |
| Class A2 | 0.00% | 0.06% | 0.04% | 100.0% | 100.0% | 100.4% | 100.1% |
| Class B | -0.05% | 0.01% | 0.09% | 100.2% | 99.8% | 99.6% | 100.0% |
| Class C | 0.00% | 0.09% | 0.12% | 100.1% | 99.9% | 100.4% | 100.0% |
| Class E | x | x | x | x | x | x | x |
| Overall | -0.04% | 0.04% | 0.07% | 100.1% | 99.9% | 100.1% | 100.0% |
| Class D | 0.02% | 0.12% | 0.13% | 100.2% | 99.9% | 100.1% | 99.9% |
| Class F | -0.07% | -0.12% | -0.17% | 100.1% | 100.0% | 100.9% | 100.0% |
| Class TGM | -0.12% | -0.12% | -0.15% | 100.1% | 100.0% | 100.0% | 100.5% |

|  | Low Delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Enc Vm | Dec Vm |
| Class A1 | x | x | x | x | x | x | x |
| Class A2 | x | x | x | x | x | x | x |
| Class B | -0.13% | -0.28% | 0.00% | 100.2% | 100.0% | 99.5% | 100.8% |
| Class C | -0.01% | 0.08% | 0.00% | 100.3% | 100.0% | 100.0% | 99.8% |
| Class E | 0.01% | 0.22% | -0.66% | 99.9% | 99.8% | 101.1% | 99.8% |
| Overall | -0.05% | -0.03% | -0.16% | 100.1% | 100.0% | 100.0% | 100.2% |
| Class D | 0.00% | 0.22% | -0.82% | 98.8% | 100.0% | 100.2% | 100.0% |
| Class F | -0.15% | -0.44% | -0.65% | 100.2% | 100.1% | 99.4% | 100.5% |
| Class TGM | -0.18% | -0.14% | -0.12% | 100.2% | 100.2% | 97.8% | 101.4% |

# References

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).