| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0189-v1 |
| --- | --- |

| Title: | EE1-2.2: Wide activation HOP model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Junru Li, Chaoyi Lin, Kai Zhang, Li Zhang (Bytedance) | Email: | yue.li@bytedance.com |
| Source: | Bytedance |  |  |

_____________________________

- Abstract

This contribution presents the test result of EE1-2.2, where a filter architecture for HOP is developed by rebalancing the network depth and width of existing HOP model in NNVC-8.0. In particular, the number of feature maps in the backbone block is increased while the number of backbone blocks is reduced to lower latency.

BD-rate changes on top of NNNC-8.0 HOP anchor (NN intra + HOP.3) are reportedly summarized as below:

RA: -0.20%, -0.63%, -0.38%, LD: -0.40%, 0.20%, -0.34%, AI: -0.03%, -0.42%, 0.14%

Complexity savings are reportedly summarized as below:

kMAC/pixel: 1% reduction (from 466 to 461), number of convolutional layers: 21% reduction (from 196 to 154).

In addition, about 10% reduction in training time is observed due to lower latency.

Introduction

At JVET-AG meeting, a new filter architecture as shown in Fig. 1 was adopted as the HOP model in NNVC-8.0 [1]. This HOP model has a simplified residual block design compared with its predecessor but more residual blocks. The increased residual blocks lead to additional latency.

Fig. 1. HOP architecture in NNVC-8.0.

Proposed HOP Filter Architecture

In this EE test, a filter architecture for HOP is developed by rebalancing the network depth and width of existing HOP model in NNVC-8.0. As shown in Fig. 2, numbers of feature maps before the activation layer in a residual block, i.e. C1 and C22, increase to 192 and 80, enabling an effect of wider activation, while the number of backbone blocks, i.e. N, reduces to 24, giving lower latency during training and inference.

Note that with the proposed change, the total number of convolutional layers is significantly reduced by 21%, i.e., from 196 to 154, while the worst case kmac/pixel is also slightly reduced by 1%, i.e., from 466 to 461. Besides, a 10% speedup is also observed for training process.

Fig. 2. Proposed HOP architecture in EE1-2.2 test.

Inference

SADL is used for performing the inference of the proposed NN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.40M |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | Total Parameters Number × 2 |
|  | Multiply Accumulate (kMAC/pixel) | 461 |
| Optional | Total Conv. Layers | 154 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

Same training procedure as HOP3 in NNVC-8.0 is applied. The network information in the training stage is provided in Table 2.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | 180h |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | NNVC, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |

Experimental results

The proposed model is integrated into NNVC-8.0 and tested according to the common test conditions defined in [2].

Compared with NNVC-8.0 HOP anchor (nn intra + hop.3), the proposed method brings on average {-0.20%, -0.63%, -0.38% }, {-0.40%, 0.20%, -0.34%, }, and {-0.03%, -0.42%, 0.14%} BD-rate changes for {Y, Cb, Cr} under RA, LD, and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 HOP3 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.44% | -0.09% | 0.12% | 99% | 95% |
| Class A2 | -0.09% | -0.71% | 0.20% | 97% | 97% |
| Class B | -0.15% | -1.08% | -0.57% | 94% | 95% |
| Class C | -0.17% | -0.40% | -0.96% | 95% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.63% | -0.38% | 96% | 96% |
| Class D | -0.25% | -0.65% | -1.69% | 95% | 97% |
| Class F | -0.01% | -0.03% | 0.13% | 96% | 95% |
|  | Low Delay B Main10 |  |  |  |  |
|  | BD-rate Over NNVC-8.0 HOP3 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.31% | 0.31% | -0.85% | 93% | 103% |
| Class C | -0.30% | -1.57% | -1.23% | 100% | 99% |
| Class E | -0.68% | 2.38% | 1.70% | 91% | 99% |
| Overall | -0.40% | 0.20% | -0.34% | 95% | 100% |
| Class D | -0.23% | 0.67% | -1.94% | 96% | 103% |
| Class F | -0.15% | -0.47% | -0.07% | 100% | 102% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 HOP3 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.05% | -0.32% | 1.09% | 94% | 98% |
| Class A2 | -0.06% | -0.46% | -1.35% | 100% | 95% |
| Class B | 0.04% | -1.26% | 0.30% | 100% | 97% |
| Class C | -0.09% | 0.16% | 0.16% | 95% | 99% |
| Class E | -0.03% | 0.17% | 0.41% | 96% | 97% |
| Overall | -0.03% | -0.42% | 0.14% | 98% | 97% |
| Class D | -0.07% | 0.37% | 0.28% | 96% | 92% |
| Class F | 0.02% | 0.52% | 0.06% | 97% | 105% |

Conclusion

This contribution proposes a rebalanced HOP filter architecture on top of existing HOP model in NNVC-8.0 and achieves better performance with significantly reduced latency as well as lower complexity. It is suggested to adopt the proposed HOP architecture into next version of NNVC.

References

- Y. Li, C. Lin, J. Li, K. Zhang, L. Zhang, D. Rusanovskyy, Y. Li, M. Karczewicz, R. Chang, L. Wang, X. Xu, S. Liu, EE1-1.1: Report on training with HOP architecture change for EE1-0 (variant 1), JVET-AG0174, Teleconference, January 2024.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AF2016, Hannover, DE, October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).