| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0197-v1 |
| --- | --- |

| Title: | AHG9: Showcase on multiplane image SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Showcase |  |  |
| Author(s) or Contact(s): | Ashmik Harinkhede, Yash Pawar, Binu E. R., Vijayakumar G. R., Abhijeet Sarate, Jeeva Raj Arumugam, Jay Shingala (Ittiam) Walt Husak, Dae Yeol Lee, Sean McCarthy, Sejin Oh, Guan-Ming Su, Gary J. Sullivan, Peng Yin (Dolby) | Email: | vijayakumar.gr@ittiam.com sean.mccarthy@dolby.com |
| Source: | Ittiam Systems, Ltd., Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution describes a smartphone application to demonstrate the multiplane image information (MPII) SEI message. The application performs real time HEVC decoding, MPII SEI message parsing, MPI reconstruction, view synthesis based on viewpoint, and rendering for 16-layer MPI content.

# Introduction

Draft text for the MPII SEI message syntax and semantics is included in Technologies under consideration for future extensions of VSEI (version 3), JVET-AG2032 [1].

The AHG9 on SEI message studies was established at the 33rd JVET meeting. Mandates of this ad hoc group include the following:

- Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing.

- Coordinate with WG 4 and AG 5 to evaluate visual quality in the context of MPI, and prepare for conducting subjective viewing during the next meeting.

To progress on the AHG9 mandates, a joint AG 5, WG 4, and WG 5 (AHG9) meeting by teleconference was held on March 28, 2024, to discuss evaluation of video quality in the context of MPI during the JPEG and MPEG meetings in Rennes, Fr, April 2024. JVET-AH0045 [2] is the report of the AG 5, WG 4, and WG 5 (AHG9) meeting.

Based on the discussions at the joint meeting, it would be beneficial to be able to view limited-layer MPI video encoded in HEVC and signalled by the MPII SEI message. The demonstration application described in this document is provided for that purpose.

# Description

Two smartphone models are used for the demonstration: Samsung Galaxy S23 Ultra and Samsung Galaxy S24 Ultra. The operating system for both smartphones is Android 14.

The demo application uses the hardware HEVC decoder to decode the frames extracted from MP4 file through the Android Media Extractor. The MPII SEI messages are parsed parallelly in the player and this information is used in the MPI reconstruction to extract individual layers from the decoded frames. The MPI renderer uses OpenGL to project the layers.

Figure 1. Overview of MPII SEI message demonstration application

The demonstration application has been tested for support for 8-bit 25 fps YUV 4:2:0 16-layer MPI video packed in up to 2048x8192 resolution pictures encoded as HEVC Main Profile Level 6.1 at bitrates ranging from approximately 7.5 Mbps to 40.5 Mbps.

The viewpoint for MPI rendering is based on the orientation of the smartphone. The device gyroscope is used to determine the orientation. The gyroscope data is updated approximately every 60 msec.

- References

- S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, “Technologies under consideration for future extensions of VSEI (version 3)”, JVET-AG2032, teleconference, January, 2024.

- S. McCarthy, J.-R. Ohm, M. Wien, B. Kroon, “AHG9: Report on the joint AG 5, WG 4, and WG 5 (AHG9) conference call on preparations to evaluate video quality in the context of MPI”, JVET-AH0042, Rennes, Fr, April Jan, 2024

- W. Husak, S. McCarthy, T. Lu , “AHG9: Test material for the multi-plane image information SEI message”, JVET-AG0332, teleconference, January, 2024.

# Patent rights declaration(s)

Ittiam Systems, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).