| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0198-v1 |
| --- | --- |

| Title: | AHG9: No display and no display purpose SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Gary J. Sullivan, Peng Yin | Email: | sean.mccarthy@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

Two alternative proposals are made in this contribution. The first alternative (Alternative 1) is to include the no display SEI message in the next edition of VSEI or next editions of AVC and VVC. The no display SEI message is currently specified in HEVC but not in AVC, VVC, or VSEI. The no display SEI message indicates that the current picture should not be displayed. The no display SEI message could be used when a decoded output picture is intended to be available for use by a post-decoder process but not intended to be displayed. As examples, a decoded output picture not intended for display could be used as input to an NNPF or a machine analysis process or used as a fusion picture in the context of the generative face video SEI message included in the Technologies under Consideration for future editions of VSEI. The second and preferred alternative (Alternative 2) is to adopt a new no display purpose SEI message based on the no display SEI message. It is proposed to include a syntax element to indicate explicitly that the current picture should not be displayed but should be available for other purposes. It is proposed to adopt Alternative 2 in the next editions of AVC, HEVC, and VVC

# Problem statement

In some applications, a decoded output picture might not be intended for display but might be intended to be available for use by a post-decoder process other than display. As examples, a decoded output picture not intended for display could be used as input to an NNPF or a machine analysis process or used as a fusion picture in the context of the generative face video SEI message included in the Technologies under Consideration for future editions of VSEI.

The no display SEI message specified in HEVC could be used to indicate that a decoded output is not intended for display. However, the no display SEI message is not currently specified in AVC, VVC, or VSEI. Additionally, the no display SEI message does not explicitly indicate that the decoded output picture is intended be available for use by an application other than display. As a result, receivers could remove a decoded picture that should not be displayed even when the decoded picture should be used by an application other than display.

The syntax and semantics of the no display SEI message specified in HEVC are copied below for convenience.

D.2.26 No display SEI message syntax

| no_display( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

D.3.26 No display SEI message semantics

The no display SEI message indicates that the current picture should not be displayed.

The full-frame freeze SEI messages specified in AVC provide functionality similar in some aspects to the no display SEI message. The full-frame freeze SEI messages are summarized below.

The full-frame freeze SEI message indicates that the current picture and any subsequent pictures in output order that meet specified conditions should not affect the content of the display.

The full-frame freeze release SEI message cancels the effect of any full-frame freeze SEI message sent with pictures that precede the current picture in output order.

The full-frame snapshot SEI message indicates that the current frame is labelled for use as determined by the application as a still-image snapshot of the video content.

The full-frame freeze SEI message semantics specify the current picture should not affect the content of the displays. However, an undisplayed decoded output picture could be used by an application that does affect the content of the display. The full-frame snapshot SEI message could be used to indicate the current picture is labelled for use by the applications, but constrains the current picture be used as a still-image snapshot which is undefined.

The full-frame SEI messages are not specified in HEVC, VVC, or VSEI.

# Proposal

## Alternative 1

It is proposed to include the no display SEI message syntax and semantics specified in HEVC in the next edition of VSEI or the next editions of AVC and VVC.

## Alternative 2 (preferred)

It is proposed to include a new no display purpose SEI message in the next edition of VSEI or the next editions of AVC, HEVC, and VVC. The proposed no display purpose SEI message is based on the no display SEI message and includes a syntax element to explicitly indicate that the current decoded output picture is intended be available for use by an application other than display.

Table X.X Persistence scope of SEI messages (informative)

| No display purpose | The access unit containing the SEI message |
| --- | --- |

X.1	No display purpose SEI messages

X.1.1	No display purpose SEI message syntax

| no_display_purpose ( payloadSize ) { | Descriptor |
| --- | --- |
| ndp_purpose | u(16) |
| } |  |

X.1.1	No display purpose SEI message semantics

The no display purpose SEI message indicates that the current decoded output picture should not be displayed.

ndp_purpose greater than 0 indicates the current decoded output picture is intended to be used for the purpose associated with the bitMask value in Table X.X.

The value of ndp_purpose shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this version of this Specification. Values of 4 to 65 535, inclusive, for ndp_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore no display purpose information SEI messages with ndp_purpose in the range of 4 to 65 535, inclusive

Table X.X Definition of ndp_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Determined by the application |
| 0x02 | Generative video |

# Conclusion

It is proposed to adopt Alternative 2 in the next editions of AVC, HEVC, and VVC.

# Patent rights declaration

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).