| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0200-(v2) |
| --- | --- |

| Title: | EE2-2.10a and 2.10d: SATD based IntraTMP BV search without signalling and BV reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Xiaoyu Xiu, Changyue Ma, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | {chenwei06, xiaoyuxiu, machangyue, jhuhong-jheng, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In EE2-2.10a, a SATD based cost function is introduced to replace the current IntraTMP BV search on refinement search. Compared to the test 2.10b, 2.10a does not add additional signaling.

In EE2-2.10d, a reordering method is introduced to reduce the signalling overhead of IntraTMP BV.

On top of the ECM12.0 [1], simulation results of EE2-2.10a and 2.10d are reported as below:

EE2-2.10a

AI: Overall: {-0.03%,-0.04%,-0.03%,100.2%,101.0%,100.4%,100.0%};

RA: Overall: {-0.01%,-0.06%,-0.02%,99.8%,100.1%,100.0%,100.0%};

EE2-2.10d

AI: Overall: {-0.09%,-0.07%,-0.04%,100.3%,100.6%,100.2%,100.0%};

RA: Overall: {-0.05%,0.02%,-0.11%, xx.x%, xx.x %, xx.x %, xx.x %}; some data points copied from the anchor

# Introduction

Intra template matching prediction (Intra TMP) is anintra prediction mode that predicts the current CU from the reconstructed regions within the current frame. The best prediction block is determined by matching the L-shaped reconstructed neighboring samples of the CU (based on the SAD cost) in a predefined search range. In ECM-12.0 [1], in the initial search, the intraTMP search regions is subsampled by a factor of 3. After finding the best match, a refinement process is further applied by refining the BV around the best match within a reduced range.

In ECM-12.0, sub-pel precision is also enabled for IntraTMP, which includes three sub-pel positions (1/2-pel, 1/4-pel and 3/4-pel). For each sub-pel position, eight directions are supported. In the current design, the selected sub-pel BV is explicitly signaled in bitstream, which takes up to 6-bit signaling overhead.

# Proposed methods

In EE2-2.10a, it is proposed to use the SATD as the cost function for the integer BV search. To reduce the complexity, the SATD based cost function is only applied in the refinement process while the initial search still utilizes the SAD. Additionally, the SATD is always enabled at the refinement stage without any control signalling.

In EE2-2.10d, it is proposed to enable the reordering to reduce the signaling overhead of intraTMP BVs. Specifically, a candidate list, which includes the integer-pel BV (output from the integer BV search) and 16 sub-pel BVs (which includes the BVs at 1/2-pel and 1/4-pel sub-pel positions with eight directions) around the selected integer-pel BV, is generated. The list is sorted based on the SAD cost between the template samples of the current CU and their corresponding prediction samples. After the reordering, the first two candidates are allowed to be selected with one single flag being signaled from encoder to decoder.

# Experimental Results

The proposed methods have been implemented on top of the ECM-12.0 [1]. Testing results of EE2-2.10a and EE2-2.10d under AI and RA configurations are summarized in Table 1 and Table 2. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Test results of EE2-2.10a.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | 0.00% | 0.07% | 99.6% | 99.8% | 100.8% | 99.8% |
| Class A2 | -0.03% | 0.00% | -0.02% | 100.1% | 101.1% | 100.0% | 100.0% |
| Class B | -0.05% | 0.00% | 0.01% | 100.1% | 101.3% | 101.1% | 99.9% |
| Class C | 0.02% | -0.04% | -0.06% | 100.4% | 101.1% | 100.0% | 100.0% |
| Class E | -0.09% | -0.20% | -0.14% | 100.7% | 101.6% | 100.0% | 100.0% |
| Overall | -0.03% | -0.04% | -0.03% | 100.2% | 101.0% | 100.4% | 100.0% |
| Class D | -0.03% | -0.01% | -0.15% | 99.8% | 101.9% | 100.0% | 100.0% |
| Class F | 0.02% | 0.01% | -0.05% | 100.3% | 101.2% | 100.0% | 100.0% |
| Class TGM | -0.03% | 0.04% | -0.12% | 100.7% | 101.8% | 100.0% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.21% | -0.08% | 99.7% | 100.3% | 100.0% | 99.9% |
| Class A2 | 0.00% | 0.12% | 0.01% | 99.9% | 100.4% | 100.0% | 100.1% |
| Class B | -0.01% | -0.11% | -0.17% | 99.7% | 100.0% | 100.1% | 100.0% |
| Class C | 0.03% | -0.01% | 0.20% | 99.9% | 99.8% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.06% | -0.02% | 99.8% | 100.1% | 100.0% | 100.0% |
| Class D | 0.03% | 0.01% | 0.09% | 100.0% | 100.1% | 100.0% | 100.0% |
| Class F | 0.09% | -0.16% | 0.08% | 100.1% | 100.4% | 99.6% | 99.6% |
| Class TGM | 0.06% | 0.06% | 0.08% | 99.7% | 100.4% | 99.4% | 100.1% |

Table 1: Test results of EE2-2.10d.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.02% | 0.06% | 99.9% | 100.2% | 100.6% | 100.0% |
| Class A2 | -0.10% | -0.06% | -0.10% | 100.1% | 100.4% | 100.0% | 100.1% |
| Class B | -0.07% | 0.00% | -0.01% | 100.4% | 101.0% | 100.0% | 100.1% |
| Class C | -0.08% | -0.13% | 0.08% | 100.5% | 100.7% | 100.4% | 100.0% |
| Class E | -0.17% | -0.17% | -0.28% | 100.2% | 100.3% | 100.0% | 100.0% |
| Overall | -0.09% | -0.07% | -0.04% | 100.3% | 100.6% | 100.2% | 100.0% |
| Class D | -0.09% | 0.10% | -0.01% | 100.1% | 100.1% | 99.9% | 100.0% |
| Class F | -0.14% | 0.06% | 0.00% | 100.1% | 100.7% | 100.9% | 100.0% |
| Class TGM | -0.20% | -0.18% | -0.29% | 100.2% | 100.4% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.01% | -0.09% |  |  |  |  |
| Class A2 | -0.07% | 0.18% | -0.02% |  |  |  |  |
| Class B | -0.08% | 0.04% | -0.15% | 100.2% | 99.8% | 100.3% | 100.0% |
| Class C | 0.00% | -0.11% | -0.13% | 100.3% | 100.2% | 100.9% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | 0.02% | -0.11% |  |  |  |  |
| Class D | -0.01% | -0.22% | -0.03% | 100.5% | 100.3% | 100.0% | 100.0% |
| Class F | -0.03% | 0.06% | -0.12% | 100.3% | 100.5% | 100.2% | 100.0% |
| Class TGM | -0.12% | -0.12% | -0.13% | 100.4% | 100.1% | 100.0% | 100.4% |

# Conclusions

In EE2-2.10a, a SATD based cost function with no additional signaling is introduced to replace the current IntraTMP BV search at the refinement search. In EE2-2.10d, a reordering method is introduced to reduce the signaling overhead of IntraTMP BV. The proposed methods reportedly bring noticeable coding gains with negligible run-time impacts. It is recommended to adopt both 2.10a and 2.10d into the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).