| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0201-(v1) |
| --- | --- |

| Title: | EE2-2.10g: combination test of EE2-2.10d and EE2-2.11c |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Xiaoyu Xiu, Xianglin Wang (Kwai) Nengfu Qiu, Junyan Huo, Yanzhuo Ma, Fuzheng Yang (Xidian University) Lai Zhang, Yue Yu, Fan Wang, Haoping Yu, Dong Wang (OPPO) | Email: | chenwei06@kwai.com nfqiu@stu.xidian.edu.cn zhanglai@oppo.com |
| Source: | Kwai Inc., Xidian University, OPPO |  |  |

_____________________________

# Abstract

This contribution reports the results for EE2 Test 2.10g which is a combination of EE2 Test 2.10d and Test 2.11c. On top of ECM-12.0 [1], the reported performance of the proposed combination is summarized as follows:

- AI: -0.xx%, -0.xx%, -0.xx%  { xx.x %, xx.x %}

- RA: -0.xx%, -0.xx%, -0.xx%  { xx.x %, xx.x %}

# Introduction

In EE2-2.10d [2], it is proposed to enable the reordering to reduce the signaling overhead of intraTMP BVs. Specifically, a candidate list, including the integer-pel BV (from the integer BV search) and 16 sub-pel BVs (i.e., the 1/2-pel and 1/4-pel sub-pel BVs) around the selected integer-pel BV, is generated. The list is sorted based on the SAD cost between the template samples of the CU and their corresponding prediction samples. Then, the first two candidates are allowed to be selected with one single flag being signaled from encoder to decoder.

In EE2-2.11c [3], the auto-relocated BVP for IntraTMP merge candidates and the template matching based sorting process for IntraTMP merge candidates are tested.

Test EE2-2.10g is the direct combination of Test 2.10d and Test 2.11c.

# Experimental Results

The proposed combination has been implemented on top of the ECM-12.0. Testing results under AI and RA configurations are summarized in Table 1. The simulations were performed following the common test conditions [4]. More detailed results can be found in the attached spreadsheets.

Table 1: Test results of EE2-2.10g.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | 0.08% | -0.04% |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.17% | -0.16% | -0.07% |  |  |  |  |
| Class C | -0.18% | -0.22% | -0.09% |  |  |  |  |
| Class E | -0.25% | -0.33% | -0.45% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.09% | -0.09% | -0.18% |  |  |  |  |
| Class F | -0.44% | -0.30% | -0.27% |  |  |  |  |
| Class TGM | -0.90% | -0.79% | -0.94% |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Conclusions

In EE2-2.10g, a combination of the Test 2.10d and Test 2.11c is introduced. The proposed combination can bring interesting coding gains without significant enc/dec run-time impacts. It is recommended to adopt the EE2-2.10g into the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

[2] W. Chen, et. al., “EE2-2.10a and 2.10d: SATD based IntraTMP BV search without signalling and BV reordering”, JVET-AH0200, April. 2024.

[3] N. Qiu, et. al., “EE2-2.11: Auto-relocated BVP for IntraTMP merge candidates”, JVET-AH0055, April. 2024.

[4] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for the implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).