| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0205-v1 |
| --- | --- |

| Title: | [EE1-2.3: Integer implementation of HOP In-loop filter with Transformer blocks and Attention blocks] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposed] |  |  |
| Author(s) or Contact(s): | Yun Li Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | yli30@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A unified filter architecture for the High-performance Operation Point (HOP) was defined in JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041, and group convolution, rank reduction and flipping were adopted into HOP in JVET-AG0174. Complete inference results for HOP training are reported in JVET-AH0014.

Based on the architecture of the HOP network, Transformer blocks were introduced in JVET-AF0158, and the inference was performed by using the Pytorch inference engine. This model has improved the coding gain significantly compared to HOP. EE1-2.3 aimed to integrate HOP with transformer with SADL float/ int16 support. In addition, The Attention model presented in JVET-AG0162 with the SADL integer implementation was adjusted during training with a new luma-chroma weighting.

In this document, the results of integer-Transformer implementation and the adjusted Attention model with SADL support are reported. Both models are based on the HOP3 architecture presented in JVET-AG0174, and the filter parameters are modified and constrained to the complexity and memory requirements of the HOP3 architecture. The training was conducted by using the NNVC8.0 HOP training guidelines, and SADL-int16 quantized model was used for the inference.

- Attention model (int16):

Complexity of the model in kMAC/pixel: 465 (block-wise) with 1.39 M parameters.

The proposed filter (int16 model) with NNIntra ON provides: {-0.8%, -4.7%, -4.3%} for AI test configuration, and {-0.3%, -5.9%, -4.8%} for RA test configuration, versus the HOP anchor (NNIntra ON).

Compared to the VTM, the proposed filter provides: {-13.4%, -15.7%, -16.8%}, for the AI configuration, and {-14.0%, -19.0%, -18.8%} for the RA configuration.

- Transformer model (int16):

Complexity of the model in kMAC/pixel: 465 (block-wise) with 1.35 M parameters.

The proposed filter (int16 model) with NNIntra ON provides: {-1.0%, -9.2%, -8.7%} for AI test configuration, and {-0.9%, -11.3%, -10.5%} for RA test configuration, versus the HOP anchor (NNIntra ON).

Compared to the VTM, the proposed filter provides: {-13.6%, -19.3%, -20.4%} for the AI configuration, and {-14.6%, -23.5%, -23.8%} for the RA configuration.

It is proposed to adopt the Transformer model for HOP.

# Introduction

A unified filter architecture for the High-performance Operation Point (HOP) was defined in JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041, and group convolution, rank reduction and flipping were introduced in JVET-AG0174. Complete inference results for HOP training are reported in JVET-AH0014.

Based on the architecture of the HOP network, Transformer blocks were introduced in JVET-AF0158 [1] with the inference performed by using the Pytorch inference engine, and SADL float/int support is further needed to assess its performance. Additionally, the model with Attention blocks, presented in JVEG-AG0162 [2] and derived from the Transformer blocks, has a luma-chroma imbalance problem and a low RA luma gain.

In the contribution, HOP with transformer with SADL float/ int16 support was implemented, and the Attention model was adjusted during training with a new luma-chroma weight balance of 16:1:1.

- Proposed method

EE1-2.3 contains two subtests, i.e., Transformer model and Attention model. Block diagram of the subtest 2, Transformer model, is shown in Figure 1. The transformer block contains an Attention network and a Feed Forward Network (FFN), which are shown in Fig. 2 and Fig. 3, respectively, and the transformer block is integrated into the Residual Backbone Block (RBB) as illustrated in Fig. 4.

Figure 1 HOP with transformer blocks.

Figure 2 Attention block

Figure 3 Feed forward network

Figure 4 Backbone blocks with the transformer block

In this application, all the nonlinear layers, i.e. LayerNorm, Norm, and Softmax in Fig. 2 have been approximated with integer arithmetic operations, and they have been integrated into the SADL for inference. The approximation is an adaptation from existing techniques, for example, Newton’s method is utilized for computing the integer square root, and range reduction and polynomial approximation is applied to the calculation of exponential functions. In this proposal, these layers are quantized with 11 bits.

For the subtest 1, the Attention block is simplified from the transformer block to preserve the self-attention mechanism in the system, and this block is showed in Fig. 5, where Fig. 5(a) is the original self-attention Transformer module with FFN, and Fig. (b) is the Attention block derived from the Transformer module with all normalization layers and FFN removed (shown in shadow-grey color).

Figure 5 Simplified Attention block from the Transformer: (a) The complete Transformer; (b) Simplified Attention block implemented with int16 SADL.

The resulting Attention block was integrated into the end of the RBB as illustrated in Fig. 6.

Figure 3 An Attention block is integrated into the backbone block of HOP network as the HOP attention block.

In this proposal, only two HOP Attention Blocks are utilized to replace the 8th and the 15th HOP backbone block, respectively, whereas the rest of the HOP backbone blocks remain unchanged.  To adjust the complexity to that of HOP, the number of residual blocks is increased to 29. The Conv1 in Fig. 5 consists of a 1x1xCx3C convolution followed by a 3x3x3Cx3C group convolution. This is the same as the Transformer block except that the group number is set to 8. The Conv2 contains a 1x1xCxC convolution.

The information for the inference and the training stage is presented in Table 1 and 2 for the Transformer model, respectively, and is illustrated in Table 3 and Table 4 for the Attention model, respectively.

Table1. Network information for the inference (Transformer model)

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.35M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 465 (367 frame-wise) |
|  | Total Conv. Layers |  | 244 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 2.72/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training (Transformer model)

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | ~432h |
|  | Training data information: | DIV2K BVI TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Table3. Network information for the inference (Attention model)

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.39M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 465 (367 frame-wise) |
|  | Total Conv. Layers |  | 198 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 2.78/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 4. Network information for the training (Attention model)

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | ~336h |
|  | Training data information: | DIV2K BVI TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

- Simulation results

The method was integrated in the NNVC8.0 and tested following the EE1 CTC and evaluation procedure [3]. The proposed method has been trained following the EE1-0 training procedure (Stage 3 HOP training only) specified in JVET-AE0191 and updated for EE1 testing as specified in JVET-AG0041. The training is being conducted with a target of 20 epochs as specified by the guidance.

The proposed method was tested with NNVC8.0 reference software and the trained model is quantized to uniform 11-bit for the parameters with SADL, and the SADL modules are added and modified for running the proposed models. In addition, the attention model is run with a fixed block size of 128x128.

Summary of the inference results are reported below, complete results in CTC template are provided along with this document.

(1a.)  Attention model (int16):

Comparative results to the NNVC8.0 HOP3 filter are shown in Table 5. NN Intra was enabled for both the test and the anchor. Comparison against VTM is shown in Table 6.

As observed in Tables 5, the proposed filter outperforms the HOP anchor with NNIntra ON by {-0.8%, -4.7%, -4.3%} for the AI configuration and {-0.3%, -5.9%, -4.8%} for the RA configuration, respectively.

Compared to VTM, shown in Table 6, the proposed method provides {-13.4%, -15.7%, -16.8%} for the AI configuration, and {-14.0%, -19.0%, -18.8%} for the RA configuration, respectively.

Table 5 Proposed method (int16 model) vs NNVC8.0 HOP, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.81% | -2.65% | -1.91% | -0.42% | -3.54% | -3.04% |
| Class A2 | -0.81% | -7.34% | -12.66% | 0.00% | -5.95% | -14.92% |
| Class B | -0.92% | -7.00% | -3.18% | -0.40% | -12.03% | -3.79% |
| Class C | -0.69% | -1.56% | -1.55% | -0.29% | 0.06% | 0.38% |
| Class E | -0.83% | -4.67% | -4.05% |  |  |  |
| Overall | -0.82% | -4.73% | -4.33% | -0.29% | -5.89% | -4.76% |
| Class D | -0.28% | -2.09% | -1.47% | -0.07% | -1.85% | 0.02% |
| Class F | -0.91% | -1.11% | -2.85% | -0.16% | 1.30% | -0.17% |

Table 5 Proposed method (int16 model) vs VTM, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -13.57% | -11.29% | -17.58% | -15.60% | -13.21% | -21.33% |
| Class A2 | -13.22% | -17.34% | -20.24% | -14.77% | -19.00% | -23.93% |
| Class B | -11.96% | -16.04% | -13.39% | -12.82% | -23.71% | -16.11% |
| Class C | -12.89% | -14.40% | -16.57% | -13.66% | -17.34% | -16.45% |
| Class E | -16.52% | -19.34% | -18.51% |  |  |  |
| Overall | -13.41% | -15.65% | -16.79% | -13.99% | -18.97% | -18.81% |
| Class D | -12.08% | -13.85% | -16.90% | -14.60% | -18.52% | -17.17% |
| Class F | -9.70% | -12.66% | -12.78% | -8.74% | -11.87% | -11.15% |

(1b.)  Attention model (float):

Comparative results to the NNVC8.0 HOP3 filter are shown in Table 7.

As observed in Tables 7, the proposed filter outperforms the HOP anchor with NNIntra ON by {-1.0%, -4.5%, -4.2%} for the AI configuration and {-0.6%, -6.2%, -5.0%} for the RA configuration, respectively.

Table 7 Proposed method (float) vs NNVC8.0 HOP, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.96% | -2.14% | -1.86% | -1.04% | -4.09% | -3.13% |
| Class A2 | -0.95% | -6.72% | -11.28% | -0.23% | -6.02% | -15.19% |
| Class B | -1.05% | -6.46% | -3.01% | -0.67% | -12.42% | -4.09% |
| Class C | -0.82% | -1.88% | -1.94% | -0.62% | -0.27% | 0.28% |
| Class E | -0.99% | -4.84% | -4.36% |  |  |  |
| Overall | -0.96% | -4.49% | -4.18% | -0.64% | -6.23% | -4.95% |
| Class D | -0.40% | -2.60% | -2.02% | -0.26% | -2.12% | 0.30% |
| Class F | -1.16% | -1.54% | -3.29% | -0.45% | 0.84% | -0.38% |

(2a.)  Transformer model (int16):

Comparative results to the NNVC8.0 HOP3 filter are shown in Table 8. NN Intra was enabled for both the test and the anchor. Comparison against VTM is shown in Table 9.

As observed in Tables 8, the proposed filter outperforms the HOP anchor with NNIntra ON by {-1.0%, -9.2%, -8.7%} for the AI configuration and {-0.9%, -11.3%, -10.5%} for the RA configuration, respectively.

Compared to VTM, shown in Table 9, the proposed method provides {-13.6%, -19.3%, -20.4%} for the AI configuration, and {-14.6%, -23.5%, -23.8%} for the RA configuration, respectively.

Table 8 Proposed method (int16 model) vs NNVC8.0 HOP, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.15% | -7.73% | -6.56% | -1.30% | -9.43% | -8.81% |
| Class A2 | -1.16% | -13.04% | -18.18% | -0.66% | -12.59% | -22.58% |
| Class B | -1.03% | -13.25% | -8.62% | -0.83% | -18.37% | -9.90% |
| Class C | -0.84% | -3.55% | -4.04% | -1.03% | -2.85% | -3.58% |
| Class E | -0.90% | -7.30% | -7.50% |  |  |  |
| Overall | -1.01% | -9.15% | -8.67% | -0.94% | -11.29% | -10.53% |
| Class D | -0.39% | -3.66% | -4.21% | -0.59% | -4.47% | -4.13% |
| Class F | -1.11% | -3.18% | -4.26% | -0.44% | -1.08% | -2.47% |

Table 9 Proposed method (int16 model) vs VTM, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -13.85% | -15.43% | -21.22% | -16.34% | -18.17% | -25.78% |
| Class A2 | -13.51% | -22.07% | -25.21% | -15.34% | -24.50% | -31.00% |
| Class B | -12.05% | -21.37% | -18.06% | -13.19% | -29.06% | -21.43% |
| Class C | -13.02% | -15.91% | -18.44% | -14.31% | -19.65% | -19.71% |
| Class E | -16.57% | -21.44% | -21.25% |  |  |  |
| Overall | -13.56% | -19.30% | -20.40% | -14.55% | -23.46% | -23.76% |
| Class D | -12.18% | -15.05% | -18.72% | -15.06% | -20.68% | -20.41% |
| Class F | -9.87% | -14.31% | -14.00% | -8.98% | -13.75% | -13.05% |

(2b.)  Transformer model (float):

Comparative results to the NNVC8.0 HOP3 filter are shown in Table 10.

As observed in Tables 10, the proposed filter outperforms the HOP anchor with NNIntra ON by {-1.2%, -8.9%, -8.6%} for the AI configuration and {-1.1%, -11.4%, -10.6%} for the RA configuration, respectively.

Table 10 Proposed method (float) vs NNVC8.0 HOP, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.32% | -8.17% | -6.98% | -1.57% | -9.92% | -8.96% |
| Class A2 | -1.36% | -13.14% | -17.65% | -0.80% | -12.61% | -22.39% |
| Class B | -1.19% | -12.20% | -8.44% | -0.95% | -18.30% | -10.20% |
| Class C | -0.91% | -3.82% | -4.25% | -1.00% | -2.95% | -3.58% |
| Class E | -1.15% | -6.44% | -7.36% |  |  |  |
| Overall | -1.17% | -8.86% | -8.62% | -1.06% | -11.39% | -10.63% |
| Class D | -0.45% | -3.71% | -3.95% | -0.55% | -4.39% | -3.83% |
| Class F | -1.13% | -3.10% | -3.94% | -0.42% | -1.13% | -2.42% |

- Conclusion

In this contribution, the results of EE1-2.3 test, float/int16 SADL implementation of HOP+Transformer and HOP+Attention filter, are reported. The Transformer was implemented with int16 SADL and introduced into the HOP architecture, whereas the simplified Attention model is adjusted for the luma-chroma weight of the training loss function.

For the Attention model, compared to NNVC-8.0 HOP anchor (NNIntra ON), the filter (int16 model) provides: AI: {-0.8%, -4.7%, -4.3%} and RA: {-0.3%, -5.9%, -4.8%}. Compared to VTM, it provides: AI: {-13.4%, -15.7%, -16.8%}, and RA: {-14.0%, -19.0%, -18.8%}

For the Transformer model, compared to NNVC-8.0 HOP anchor (NNIntra ON), the filter (int16 model) provides: AI: {-1.0%, -9.2%, -8.7%} and RA: {-0.9%, -11.3%, -10.5%}. Compared to VTM, it provides: AI: {-13.6%, -19.3%, -20.4%} and RA: {-14.6%, -23.5%, -23.8%}.

It is proposed to adopt the Transformer model for HOP.

- References

- EE1-1.5 HOP In-loop filter with transformer blocks, JVET-AF0158, Y. Li, D. Rusanovskyy, T. Ryder, S. Eadie, M. Karczewicz.

- EE1-1.5 Report on implementation of HOP In-loop filter with Transformer blocks, JVET-AG0162, Y. Li, D. Rusanovskyy, M. Karczewicz.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).