| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0206-v1 |
| --- | --- |

| Title: | [EE1 related]: Additional inference test for EE1-2.3 to adjust luma-chroma balance |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposed] |  |  |
| Author(s) or Contact(s): | Yun Li Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | yli30@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A unified filter architecture for the High-performance Operation Point (HOP) was defined in JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041, and group convolution, rank reduction and flipping were adopted into HOP in JVET-AG0174. Complete inference results for HOP training are reported in JVET-AH0014.

Based on the architecture of the HOP network, Transformer blocks were introduced in JVET-AF0158, and integer implementation of the Transformer model was described in JVET-AH0205. This model has improved the coding gain significantly compared to HOP. This contribution aims to rebalance the luma-chroma coding gain during inference by adjusting chroma offset.

In this document, the results of luma-chroma rebalancing of the integer-quantized Transformer model are reported. The simulation was conducted by using the NNVC8.0 reference software, and SADL-int16 quantized model was used for the inference.

Complexity of the model in kMAC/pixel: 465 (block-wise) with 1.35 M parameters.

The proposed filter (int16 model) with NNIntra ON provides: {-2.3%, 3.3%, 3.5%} for AI test configuration, and {-2.3%, 0.7%, 0.6%} for RA test configuration, versus the HOP anchor (NNIntra ON).

Compared to the VTM, the proposed filter provides: {-14.6%, -9.1%, -10.5%}, for the AI configuration, and {-15.7%, -13.5%, -14.3%} for the RA configuration.

It is proposed to study the proposed method in the next round of EE1.

# Introduction

A unified filter architecture for the High-performance Operation Point (HOP) was defined in JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041, and group convolution, rank reduction and flipping were adopted into HOP in JVET-AG0174. Complete inference results for HOP training are reported in JVET-AH0014.

Based on the architecture of the HOP network, Transformer blocks were introduced in JVET-AF0158 [1] and JVET-AH0162 [2], and the integer implementation of this model was described in JVET-AH0205. This model has improved the coding gain significantly compared to HOP. This contribution aims to rebalance the luma-chroma coding gain during inference by adjusting chroma offset.

In this document, the results of bitrate reassignment between luma and chroma for the integer-quantized Transformer model are reported. The simulation was conducted by using the NNVC8.0 reference software, and SADL-int16 quantized model was used for the inference.

- Proposed method

It is observed that the coding gain for chroma is approximately -10 percent whereas the Luma coding gain is around -1 percent for the Transformer model versus the HOP3 anchor, presented in JVET-AH0205. Therefore, it is proposed to increase the chroma offset to 2 from 1.

The information for the inference and the training stage is presented in Table 1 and 2 for the Transformer model, respectively.

Table1. Network information for the inference (Transformer model)

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.35M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 465 (367 frame-wise) |
|  | Total Conv. Layers |  | 244 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 2.72/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training (Transformer model)

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | ~432h |
|  | Training data information: | DIV2K BVI TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

- Simulation results

The method was integrated in the NNVC8.0 and tested following the EE1 CTC and evaluation procedure [3]. The proposed method has been trained following the EE1-0 training procedure (Stage 3 HOP training only) specified in JVET-AE0191 and updated for EE1 testing as specified in JVET-AG0041. The training is being conducted with a target of 20 epochs as specified by the guidance.

The proposed method was tested with NNVC8.0 reference software and the trained model is quantized to uniform 11-bit for the parameters with SADL.

Summary of the inference results are reported below, complete results in CTC template are provided along with this document.

Comparative results to the NNVC8.0 HOP3 filter are shown in Table 3. NN Intra was enabled for both the test and the anchor. Comparison against VTM is shown in Table 4.

As observed in Tables 3, the proposed filter outperforms the HOP anchor with NNIntra ON by {-2.3%, 3.3%, 3.5%} for the AI configuration and {-2.3%, 0.7%, 0.6%} for the RA configuration, respectively.

Compared to VTM, shown in Table 4, the proposed method provides {-14.6%, -9.1%, -10.5%} for the AI configuration, and {-15.7%, -13.5%, -14.3%} for the RA configuration, respectively.

Table 3 Proposed method (int16 model) vs NNVC8.0 HOP, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -2.74% | 5.91% | 3.89% | -3.81% | 2.78% | -0.72% |
| Class A2 | -3.55% | -1.51% | -8.15% | -2.65% | -2.66% | -15.36% |
| Class B | -1.90% | 0.31% | 5.78% | -1.54% | -6.16% | 3.63% |
| Class C | -1.78% | 8.51% | 8.10% | -1.87% | 10.27% | 9.82% |
| Class E | -1.64% | 3.74% | 4.74% |  |  |  |
| Overall | -2.25% | 3.33% | 3.49% | -2.30% | 0.71% | 0.61% |
| Class D | -1.30% | 7.83% | 7.54% | -1.26% | 8.04% | 7.86% |
| Class F | -2.14% | 5.11% | 3.91% | -1.61% | 8.87% | 6.55% |

Table 4 Proposed method (int16 model) vs VTM, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -15.26% | -3.23% | -13.17% | -18.42% | -7.42% | -19.06% |
| Class A2 | -15.60% | -13.12% | -16.71% | -17.05% | -16.53% | -24.81% |
| Class B | -12.82% | -10.33% | -5.75% | -13.81% | -18.76% | -9.62% |
| Class C | -13.83% | -6.37% | -8.95% | -15.03% | -9.19% | -8.61% |
| Class E | -17.19% | -12.47% | -11.54% |  |  |  |
| Overall | -14.64% | -9.09% | -10.49% | -15.71% | -13.49% | -14.28% |
| Class D | -12.96% | -5.54% | -9.47% | -15.63% | -10.47% | -10.72% |
| Class F | -10.80% | -7.58% | -7.04% | -10.05% | -5.95% | -5.54% |

- Conclusion

In this contribution, only inference is conducted for the Transformer model with the adjusted chroma offset. The results of rebalancing the luma-chroma coding gain for the Transformer model of in-loop filtering is reported. The chroma offset was increased from 1 to 2 for the simulation.

Compared to NNVC-8.0 HOP anchor (NNIntra ON), the filter (int16 model) provides: AI: {-2.3%, 3.3%, 3.5%} and RA: {-2.3%, 0.7%, 0.6%}. Compared to VTM, it provides: AI: {-14.6%, -9.1%, -10.5%}, and RA: {-15.7%, -13.5%, -14.3%}

It is proposed to study the proposed method in the next round of EE1.

- References

- EE1-1.5 HOP In-loop filter with transformer blocks, JVET-AF0158, Y. Li, D. Rusanovskyy, T. Ryder, S. Eadie, M. Karczewicz.

- EE1-1.5 Report on implementation of HOP In-loop filter with Transformer blocks, JVET-AG0162, Y. Li, D. Rusanovskyy, M. Karczewicz.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).