| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0207-v2 |
| --- | --- |

| Title: | [EE1 related: LOP input adjustment with trainable input transform] |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposed] |  |  |
| Author(s) or Contact(s): | Yun Li Dmytro Rusanovskyy Marta Karczewicz | Tel: Email: | yli30@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A unified filter architecture for the Low-performance Operation Point (LOP) was defined in JVET-AF0043. Updated training strategy and configuration were presented in JVET-AH0042. Complete inference results for LOP training are reported in JVET-AH0014.

Based on the architecture of the LOP network, input transform with DCT were introduced in JVET-AG0069 and this model has improved the coding gain for RA luma over -0.4 percent compared to LOP with a new two-stage data extraction. This contribution aims at improving the coding gain with a trainable input transform and with the training on the existing LOP2 dataset.

In this document, the results of proposed method are reported. The simulation was conducted by using the NNVC8.0 reference software, and Pytorch float model was used for the inference.

Complexity of the model in kMAC/pixel: 16.9 (block-wise).

The proposed filter (float) with NNIntra ON provides: {-0.2%, -4.2%, -4.4%} for AI test configuration, and {-0.5%, -4.2%, -3.8%} for RA test configuration, versus the LOP2 float (NNIntra ON).

Compared to the VTM, the proposed filter provides: {-8.4%, -16.8%, -17.0%} for the AI configuration, and {-7.4%, -16.5%, -15.7%} for the RA configuration.

It is proposed to study the proposed method in the next round of EE1.

# Introduction

A unified filter architecture for the Low-performance Operation Point (LOP) was defined in JVET-AF0043[1]. Updated training strategy and configuration were presented in JVET-AH0042. Complete inference results for LOP training are reported in JVET-AH0014.

Based on the architecture of the LOP network, input transform with DCT were introduced in JVET-AG0069[2] and this model has improved the coding gain for RA luma over -0.4 percent compared to LOP with a new two-stage data extraction. This contribution aims at improving the coding gain with a trainable component for the input adjustment with training on the existing LOP2 dataset.

In this document, the results of proposed method are reported. The simulation was conducted by using the NNVC8.0 reference software, and Pytorch float model was used for the inference.

- Proposed method

The fixed transform of JVE-AG00069 is replaced by using a trainable component with some modifications shown in Fig. 1. The parameter for the proposed method is identical to that of JVET-AG0069 except the following:

- The DCT component with reshaping is replaced by a trainable transform, e.g., a PixelUnshuffle operation of a factor 2 followed by a 1x1x4x|4 convolution, where the 1x1 convolution performs an operation corresponding to a 2x2 spatial transform on the original input pixel domain before the pixel arrangement with the PixelUnshuffle.

- The IDCT component with reshaping is replaced by an inverse transform component, for example, by a 1x1xCxC convolution followed by a Pixelshuffle operation with a factor of 2. The 1x1 convolution performs an operation corresponding to a 2x2 spatial inverse transform on the output features domain after pixel arrangement with the Pixelshuffle. The C is equal to 16 for the luma branch, and equal to 8 for the chroma branch.

- The inverse trainable transform in the luma branch is placed before the Pixelshuffle of the LOP2 network, because this Pixelshuflle operation is conducted in the VTM code.

- The entire filtering architecture is implemented as a standalone model, and the VTM source code is untouched.

Figure 1. Replacement of the fixed transform with trainable components and with modifications

The example of the proposed replacement of Transform is shown in Fig. 2. The above-mentioned convolutions increase the KMAC/pixel by 0.053 and the number of parameters by 424 with auxiliary data.

Figure 2. An example of the trainable transform for the input adjustment

The information for the inference and the training stage is presented in Table 1 and 2 for the proposed model, respectively.

Table1. Network information for the inference (Transformer model)

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | Pytorch |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 208531 |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 16.9 (13.4 frame-wise) |
|  | Total Conv. Layers |  |  |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float | 1.27/model |
|  |  | Int16 |  |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training (Transformer model)

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | ~240h |
|  | Training data information: | DIV2K BVI TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

- Simulation results

The method was integrated in the NNVC8.0 and tested following the EE1 CTC and evaluation procedure [3]. The proposed method has been trained following the training procedure specified in JVET-AF0043. The training is being conducted with a target of 60 epochs as specified by the guidance with an exception that the luma-chroma weight is set to 6:1:1 to match the training configuration in JVET-AG0069.

The proposed method was tested with NNVC8.0 reference software with Pytorch inference engine.

Summary of the inference results are reported below, complete results in CTC template are provided along with this document.

Comparative results to the NNVC7.1 LOP2 filter (float) are shown in Table 3 (Experiments show that the performance of NNVC7.1 and NNVC8.0 is identical). NN Intra was enabled for both the test and the anchor. Comparison against VTM is shown in Table 4.

As observed in Tables 3, the proposed filter outperforms the LOP2 float with NNIntra ON by {-0.2%, -4.2%, -4.4%} for the AI configuration and {-0.5%, -4.2%, -3.8%} for the RA configuration, respectively.

Compared to VTM, shown in Table 4, the proposed method provides {-8.4%, -16.8%, -17.0%} for the AI configuration, and {-7.4%, -16.5%, -15.7%} for the RA configuration, respectively.

Table 3 Proposed method (float) vs NNVC7.1 LOP2 (float), NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.70% | -4.76% | -6.78% | -1.31% | -5.78% | -6.73% |
| Class A2 | -0.36% | -4.05% | -2.37% | -0.80% | -4.14% | -2.19% |
| Class B | -0.18% | -4.09% | -4.64% | -0.33% | -4.37% | -4.11% |
| Class C | 0.15% | -2.79% | -2.88% | 0.02% | -2.73% | -2.53% |
| Class E | -0.15% | -5.59% | -5.83% |  |  |  |
| Overall | -0.22% | -4.16% | -4.42% | -0.53% | -4.17% | -3.83% |
| Class D | 0.48% | -2.30% | -1.09% | 0.48% | -2.09% | -1.51% |
| Class F | -0.03% | -1.97% | -1.59% | -0.12% | -1.25% | -0.56% |

Table 4 Proposed method (float) vs VTM, NNIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -9.25% | -16.39% | -17.04% | -9.16% | -14.68% | -16.67% |
| Class A2 | -7.56% | -16.28% | -13.35% | -7.68% | -15.85% | -11.66% |
| Class B | -7.56% | -16.60% | -17.65% | -6.94% | -17.56% | -16.63% |
| Class C | -7.44% | -16.05% | -16.73% | -6.60% | -17.14% | -16.78% |
| Class E | -10.94% | -18.80% | -19.91% |  |  |  |
| Overall | -8.38% | -16.76% | -17.00% | -7.44% | -16.53% | -15.68% |
| Class D | -7.28% | -14.70% | -15.99% | -6.80% | -16.74% | -16.26% |
| Class F | -5.35% | -12.44% | -11.67% | -3.93% | -10.58% | -9.90% |

- Conclusion

In this contribution, the results of the proposed model for the trainable input transform with training on the LOP2 dataset are reported.

Compared to NNVC-7.1 LOP float (NNIntra ON), the filter (float) provides: AI: {-0.2%, -4.2%, -4.4%} and RA: {-0.5%, -4.2%, -3.8%}. Compared to VTM, it provides: AI: {-8.4%, -16.8%, -17.0%}, and RA: {-7.4%, -16.5%, -15.7%}.

It is proposed to study the proposed method in the next round of EE1.

- References

- D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya (Ittiam), J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), R. Chang, L. Wang, X. Xu, S. Liu (Tencent), AHG11: Status of the joint EE1-0 (LOP.2) unified filter training, JVET-AF0043, October 2023.

- D. Liu, J. Ström, M. Damghanian, P. Wennersten, AhG11: LOP with inputs transformed, JVET-AG0069, 2024

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).