| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 14–24 April 2024 | Document: JVET-AH0208-v1 |
| --- | --- |

| Title: | Non-EE2: affine candidates derived from temporal collocated picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Wei Chen, Xianglin Wang | Email: | machangyue@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add affine candidates derived from temporal collocated picture into affine merge candidate list. Specifically, in the proposed scheme, new affine candidates are derived from the control point motion vectors (CPMVs) from temporal collocated picture. The simulation results on top of ECM-12.0 are summarized as follows:

RA : {xxx%, xxx%, xxx%, xxx%, xxx%}, LDB : {xxx%, xxx %, xxx%, xxx%, xxx%};

# Introduction

In ECM-12.0 [1], the affine merge candidate list contains the following candidates:

- SbTMVP candidates,

- Inherited from adjacent neighbors,

- Regression based motion vector field (RMVF) derived candidates,

- Constructed from adjacent neighbors,

- Constructed from non-adjacent neighbors,

- History-parameter-based affine model inheritance (HAMI) candidates,

- Pairwise average candidates,

- Zero MVs.

As shown above, only for the constructed affine candidates, the motion vectors in the right-bottom locations in the temporal collocated picture is utilized. For all the other affine candidates, the CPMVs are derived from spatial adjacent and non-adjacent neighbors of the current CU in the same picture. To further utilize the temporal motion vector, this proposal proposes to derive affine candidates from temporal collocated picture.

# Proposal

In this contribution, the affine candidates derived from temporal collocated picture are added into current affine merge candidate list. As shown in Fig. 1, the blocks in temporal collocated picture are scanned to identify the first block that is coded with affine mode. In Fig. 1, the numbers ((“1” and “2”, “3” and “4”, “5” and “6”, etc.)) represents the scanning order of the candidate blocks. Then, one new affine candidate is derived by scaling the CPMVs of the determined block in the collocated picture to the current CU based on its position and block-size. The derived affine candidate is inserted into current affine candidate list right after the affine candidates inherited from adjacent neighbors.

Figure 1. Temporal blocks scanning positions when deriving the temporal affine candidate

# Experimental results

The proposed methods were implemented on top EMC-12.0 and tests were performed following ECM CTC [2]. The test results of the proposed methods are summarized in the following table.

Table 1: Test result of the temporal inherited affine candidates

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.07% | -0.03% | 0.12% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.71% | -0.19% | -0.63% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

# Conclusion

The contribution proposes to derive affine candidates from temporal collocated pictures. Simulation results show that the proposed methods can provide noticeable coding benefits with minor enc/dec run-time impact. It is recommended to study the proposed methods in the next round of EE.

# References

[1] ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation JVET procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).