| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0209-v1 |
| --- | --- |

| Title: | EE2-2.13: Matrix based intra prediction replacing conventional intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Hongtao Wang, Patrick Garus, Vadim Seregin, Marta Karczewicz, Samuel Eadie, Gleb Verba. 5775 Morehouse drive San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com hongtaow@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the results of EE2 test 2.13, where a matrix based intra prediction replacing some directional intra modes is proposed where the weights are intra mode and block shape dependent which are applied to L shaped causal template to generate final prediction.

On top of the ECM-12.0, simulation result of the proposed method is:

AI: -0.31%, -0.33%, -0.34%, EncT: 101.2%   , DecT:  100.2%.

RA: -0.xx%, -0.xx%, -0.xx%, EncT: 10x.x%   , DecT:  10x.x%.

# Proposed method

In this test, a matrix of weights, which are defined for a block shape and intra mode, is introduced, those weights are multiplied by the neighbour reference template to derive the prediction samples replacing conventional intra prediction. The weights are applied to the reference samples of the L shaped causal neighborhood template as shown in the Fig 1.

Fig 1: L shaped neighborhood for a given predicted block

The reference samples in the causal neighborhood are denoted as r, and F(x,y) is the matrix of weights. Then the prediction P(x,y) can be derived as

P(x,y) = ∑k F(x,y,k)*r(k),

where k denotes the index of the reference sample in the template

In the test, this prediction is used for block size with both width and height up to 32 (except for 4x32,32x4, 8x32 and 32x8). The template size is 2 for blocks with both width and height up to 16 and it is only used for mode 0, 1, and (2+2*k). For other blocks, template size is set to 1; is used for mode 0, 1, and (2+4*k); prediction is only performed for 16x16 positions, and the rest of the samples are generated by bilinear interpolation. For all block sizes, block shape and mode-based symmetry is used. Reference length is set to W and H for modes greater than 18 and less than 50 and set to 2*W and 2*H otherwise.

The filters are trained with BVI sequences composing of 800 sequences with diverse resolutions.

# Experimental results

The proposed method has been implemented on top of the ECM-12.0. The simulations were performed following the common test conditions [2] and summarized in the below table.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.31% | -0.31% | -0.26% | 100.5% | 100.7% | 100.3% | 101.2% |
| Class A2 | -0.30% | -0.32% | -0.43% | 100.3% | 98.1% | 100.3% | 101.0% |
| Class B | -0.35% | -0.29% | -0.43% | 101.5% | 100.2% | 100.0% | 101.8% |
| Class C | -0.21% | -0.32% | -0.20% | 102.1% | 101.7% | 101.1% | 102.3% |
| Class E | -0.40% | -0.42% | -0.37% | 101.2% | 99.7% | 100.9% | 102.2% |
| Overall | -0.31% | -0.33% | -0.34% | 101.2% | 100.2% | 100.5% | 101.8% |
| Class D | -0.26% | -0.06% | -0.33% | 100.9% | 99.7% | 101.3% | 102.5% |
| Class F | -0.12% | -0.13% | -0.12% | 100.6% | 98.9% | 100.9% | 102.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.22% | -0.23% | -0.29% | 100.1% | 99.5% | 100.1% | 100.2% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | -0.21% | -0.18% | -0.42% | 100.6% | 100.1% | 100.1% | 101.5% |
| Class C | -0.12% | -0.21% | 0.09% | 100.1% | 100.3% | 101.3% | 102.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -0.10% | 0.05% | 0.16% | 100.3% | 101.1% | 101.0% | 102.4% |
| Class F | -0.03% | 0.00% | -0.05% | 100.3% | 100.0% | 100.3% | 101.7% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# References

[1] ECM-12.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).