| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0211-v1 |
| --- | --- |

| Title: | Non-EE2: Intra merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Vadim Seregin, Marta Karczewicz | Tel: Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

It is proposed to construct an intra candidate list by the intra modes that are used in intra-coded neighbouring blocks to predict a current coding block. The experimental results over ECM-12.0 are summarized as follows:

AI:   -0.14% (Y-BDrate), 10x% (EncT), 100% (DecT)

RA:  xx% (Y-BDrate), xx% (EncT), xx% (DecT)

# Introduction

Current and neighboring blocks may have similar behaviour and the mode selections may be similar or the same. In ECM, many intra coding tools beyond VVC codec, such as DIMD, TIMD, IntraTMP, TMRL are employed etc. This contribution proposes to merge the intra mode used in a selected neighboring block to predict a current coding block.

# Proposed method

In the proposed method, a list of intra mode candidates is constructed using the intra modes from the neighbour blocks. It is similar to inter merge mode, but instead of motion vector the neighbour intra modes are added to the list. A flag is signalled to indicate whether to enable the proposed intra merge mode. If the flag is true, at least one intra merge candidate list is constructed. An index coding is further signalled to indicate which candidate is used for this CU. The checked neighbouring blocks are those around the current block as shown in Figure 1.

Figure 1. Neighbouring blocks for merging checks

# Simulation results

The proposed method is implemented on top of ECM-12.0. The simulations are run under Common Test Conditions for ECM [1]. The results are shown in the table below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.19% | 0.11% | 0.01% |  |  | 100.1% | 100.1% |
| Class A2 | -0.20% | 0.01% | -0.03% |  |  | 100.1% | 100.1% |
| Class B | -0.14% | 0.07% | 0.01% |  |  | 100.1% | 100.1% |
| Class C | -0.11% | 0.07% | 0.11% |  |  | 99.7% | 100.0% |
| Class E | -0.05% | -0.04% | 0.01% |  |  | 100.1% | 100.0% |
| Overall | -0.14% | 0.05% | 0.02% |  |  | 100.0% | 100.1% |

- References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).