| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0213-v3 |
| --- | --- |

| Title: | Non-EE2: MVP extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Jian-Liang Lin, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Email: | zhizhang@qti.qualcomm.com jilin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to add scaled MV candidates, TMVP candidates with alternative reference indices, and TMVP candidates based on motion trajectory crossing the current block. The proposed method was implemented on top of ECM-12.0 and the test results are as follows.

RA: -0.13% (Y), -0.12% (U), -0.09% (V), 100.5% (EncT), 102.0% (DecT), 101.1% (EncVmPeak) , 101.8% (DecVmPeak)

LDB: -0.09% (Y), -0.31% (U), -0.05% (V), 100.3% (EncT), 100.7% (DecT), 99.8% (EncVmPeak) , 99.5% (DecVmPeak)

# Introduction

In ECM-12.0, to derive the reference picture for TMVP and SbTMVP, it determines a reference picture where the POC distance comparing to the current picture is the same or closest compared to the POC distance between the collocated block’s reference picture and the collocated picture. The MV is then scaled accordingly. For SbTMVP, the position of the collocated block is derived from the position of the current block with a spatial MV offset, and for TMVP, it is derived from the position of the current block with a fixed offset.

# Proposal

This contribution proposes to add additional candidates using a selected reference picture with a scaled MV. For merge candidates, it adds a candidate with the reference index 1 when the existing candidates has reference index 0, otherwise it adds a candidate with the reference index 0, and a TMVP candidate with a spatial MV offset (as in SbTMVP). For TMVP and SbTMVP, it adds a candidate with a reference index corresponding to the collocated block’s reference picture, if the picture is not in the reference picture list of the current block, it selects a reference picture between the collocated picture and the collocatd reference picture with the largest POC distance to the current picture. Finally, it adds a bi-TMVP candidate (MV0, MV1 shown in the next figure) if a motion trajectory between a block in a reference picture and its reference block crosses the current block.

# Simulation results

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.18% | -0.19% | -0.20% | 100.1% | 101.6% | 100.8% | 101.6% |
| Class A2 | -0.07% | -0.07% | -0.05% | 99.7% | 101.6% | 100.6% | 101.2% |
| Class B | -0.10% | -0.05% | -0.04% | 100.9% | 102.5% | 100.6% | 102.1% |
| Class C | -0.17% | -0.20% | -0.09% | 100.9% | 102.0% | 102.2% | 102.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.13% | -0.12% | -0.09% | 100.5% | 102.0% | 101.1% | 101.8% |
| Class D | -0.16% | -0.07% | -0.23% | 98.6% | 102.6% | 102.8% | 102.0% |
| Class F | -0.12% | -0.12% | -0.17% | 99.7% | 100.1% | 101.5% | 101.8% |
| Class TGM | -0.01% | 0.00% | -0.06% | 100.1% | 100.6% | 100.9% | 101.6% |

1 QP22 point in ClassA1, 2 QP22 and 1 QP27 point in ClassA2 are copied from ECM12 anchor.

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.08% | -0.18% | 0.21% | 100.3% | 101.1% | 100.0% | 99.3% |
| Class C | -0.09% | -0.06% | -0.25% | 100.7% | 100.6% | 99.6% | 100.1% |
| Class E | -0.09% | -0.87% | -0.23% | 99.7% | 100.4% | 99.9% | 99.1% |
| Overall | -0.09% | -0.31% | -0.05% | 100.3% | 100.7% | 99.8% | 99.5% |
| Class D | -0.34% | 0.42% | 0.41% | 100.2% | 101.1% | 99.6% | 100.0% |
| Class F | -0.03% | 0.12% | -0.69% | 100.2% | 101.0% | 97.5% | 99.7% |
| Class TGM | -0.22% | -0.09% | -0.03% | 99.1% | 99.8% | 99.7% | 99.2% |

# References

[1] ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).