| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0215-v1 |
| --- | --- |

| Title: | Non-EE2: Adaptive cost function selection in merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Cui, Zhi Zhang, Yan Zhang, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | kaicui@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes merge index-based cost function selection in merge mode for motion vector refinement process. Alternating between two cost functions, which are selected based on the parity of the merge index, allows for the refinement of the merge candidates in a merge list with different cost functions. The proposed method is applied to regular merge, bilateral matching merge, template matching merge and affine merge modes. The proposed method reports the following BD-rate performance on top of ECM-12.0:

RA: Y: -0.xx%, U: -0.xx%, V: -0.xx%, EncT: 10x.x%, DecT: 10x.x%, EncVmPeak: 100.0%, DecVmPeak: 100.0%

# Introduction

In the current ECM, a variety of cost functions are defined and utilized in different tools. Some examples of these cost functions are SAD (Sum of Absolute Differences), SATD (Sum of Transformed Absolute Differences), SSE (Sum of Squared Error), MR-SAD (Mean-Removal Sum of Absolute Differences). Different cost functions are used for various coding tools or at different coding stages to quantify the cost of a coding tool. For instance, during the multi-pass decoder-side motion vector refinement search process, the PU-level search uses SAD as the cost function, while the sub-PU level search uses SATD.

# Proposed method

This contribution proposes to use the merge index to indicate the cost function that is used to refine the corresponding merge candidate. Two cost functions are alternately selected for every other candidate in a merge list. If the merge index is an even number, the first cost function is selected, otherwise the second cost function is selected. The proposed method applies to regular merge mode, template matching merge mode, bilateral matching merge mode, and affine merge mode.

For bilateral matching merge and affine merge, the two cost functions are set to SATD and SAD. If the merge index is an even number, then SATD is selected, otherwise SAD is selected.

For template matching merge, the two cost functions are set to weighted SAD and SAD. If the merge index is an even number, then weighted SAD is selected to refine the merge candidate, otherwise SAD is selected.

The proposed method is not applied to low-delay pictures.

# Simulation results

The proposed method was implemented on top of ECM-12.0. The performance was evaluated following the common test condition defined in [2]. The test results are reported in the following table using ECM-12.0 as the anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.00% | -0.24% |  |  | 100.1% | 99.9% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07% | -0.04% | 0.02% |  |  | 99.7% | 99.9% |
| Class C | -0.04% | -0.14% | -0.17% |  |  | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.03% | -0.09% | 0.03% |  |  | 100.0% | 100.0% |
| Class F | -0.03% | -0.29% | -0.19% |  |  | 100.1% | 99.8% |
| Class TGM | 0.02% | 0.03% | 0.04% |  |  | 99.9% | 100.1% |

# References

[1] ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).