| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 34rd Meeting, Rennes, France, 17–24 April 2024 | Document: JVET-AH0217-v1 |
| --- | --- |

| Title: | CICP enum for generic subsampling and sample location |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Leo Barnes Dimitri Podborski Alexis Tourapis | Email: | lbarnes@apple.com dpodborski@apple.com atourapis@apple.com |
| Source: | Apple Inc |  |  |

_____________________________

# Abstract

At the 144th MPEG meeting, Apple presented a proposal for an expanded PixelInformationProperty box in HEIF. It was suggested in the discussion that it would be beneficial to have codec-agnostic chroma sample location signaling and that ISO/IEC 23091-2 / Rec. ITU-T H.273 would be the appropriate place for it given that clause 8.7 already contains this information for 4:2:0 subsampling. This proposal attempts to add signaling for other commonly used subsamplings and chroma locations to Rec. ITU-T H.273.

# Introduction

The current version of the PixelInformationProperty box contains information on the number of components and their bit-depths. The proposed expansion at MPEG 144 would add information such as data type (unsigned/signed integers, floating point) and channel content (color/monochrome, depth, alpha). It was also proposed that subsampling and chroma location would be useful information to have.

It was noted in the discussion that this has some overlap with the existing 4:2:0 chroma location signaling in ISO/IEC 23091-2 / Rec. ITU-T H.273. Rather than defining new constants in HEIF, it was suggested that Rec. ITU-T H.273 could be expanded to support more subsampling types and chroma locations, with the HEIF text then referencing these tables.

It was also noted in the discussion that subsampling may be desirable not only for chroma, but also for alpha. Due to this, the text in Rec. ITU-T H.273 should ideally not be explicitly tied to luma/chroma, but rather be applicable to subsampled content in general.

# Proposal

8.8.1 Generic subsampling type

Type: Unsigned integer, enumeration
Range: 0-15

GenericSubsamplingType indicates the subsampling of a channel compared to a channel that has not been subsampled. It is specified explicitly or implicitly per channel in a frame. At least one channel in a frame shall have a GenericSubsamplingType value of 0.

Table 9 – Interpretation of generic subsampling type indicator (GenericSubsamplingType)

| Value | Interpretation of GenericSubsamplingType |
| --- | --- |
| 0 | No subsampling |
| 1 | 2x horizontal subsampling (4:2:2) |
| 2 | 2x horizontal and vertical subsampling (4:2:0) |
| 3 | 4x vertical subsampling (4:1:1) |
| 4 | 2x vertical subsampling (4:4:0) |
| 5-14 | Reserved |
| 15 | Undefined |

Table 9 describes the meaning of the GenericSubsamplingType values. As an example, 4:4:4 subsampled content would be described with GenericSubsamplingType as [0,0,0], 4:2:0 as [0,2,2] and 4:2:2 as [0,1,1].

8.8.2 Generic subsampling sample location

Type: Unsigned integer, enumeration
Range: 0-15

GenericSubsamplingSampleLocType indicates the sample alignment of a channel compared to a channel that has not been subsampled. It is specified per channel in a frame. It is assumed that at least one channel in a frame is not subsampled.

The meaning of GenericSubsamplingSampleLocType depends on the value of GenericSubsamplingType for the same channel. The relative spatial positioning of the sample compared to a channel that is not subsampled, can be expressed by defining two variables HorizontalOffset and VerticalOffset as a function of GenericSubsamplingSampleLocType as given by Table 10. HorizontalOffset is the horizontal (x) position of the centre of the top-left sample of the current channel relative to the centre of the top-left sample of a channel that is not subsampled in units of non-subsampled samples. VerticalOffset is the vertical (y) position of the centre of the top-left sample of the current channel relative to the centre of the top-left sample of a channel that is not subsampled in units of non-subsampled samples.

Table 10 – Interpretation of the value of GenericSubsamplingSampleLocType for 
different GenericSamplingType values in terms of [HorizontalOffset,VerticalOffset]

|  | GenericSubsamplingType |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | 0 | 1 | 2 | 3 | 4 |
| 0 | (0.0, 0.0) | (0.0, 0.0) | (0.0, 0.5) | (0.0, 0.0) | (0.0, 0.5) |
| 1 | (0.0, 0.0) | (0.5, 0.0) | (0.5, 0.5) | (0.25, 0.0) | (0.0, 0.5) |
| 2 | (0.0, 0.0) | (0.0, 0.0) | (0.0, 0.0) | (0.0, 0.0) | (0.0, 0.0) |
| 3 | (0.0, 0.0) | (0.5, 0.0) | (0.5, 0.0) | (0.25, 0.0) | (0.0, 0.0) |
| 4 | (0.0, 0.0) | (0.0, 0.0) | (0.0, 1.0) | (0.0, 0.0) | (0.0, 1.0) |
| 5 | (0.0, 0.0) | (0.5, 0.0) | (0.5, 1.0) | (0.25, 0.0) | (0.0, 1.0) |
| 6-14 | Reserved | Reserved | Reserved | Reserved | Reserved |
| 15 | Undefined | Undefined | Undefined | Undefined | Undefined |

Table 10 describes the meaning of the GenericSubsamplingSampleLocType values given the GenericSubsamplingType for the current channel. For the case of GenericSubsamplingType equal to

2 (4:2:0), the values of GenericSubsamplingSampleLocType have the same meaning as Chroma420SampleLocType. The meaning for the other cases of GenericSubsamplingSampleLocType have the corresponding HorizontalOffset and VerticalOffset as for GenericSubsamplingType 2, where applicable.

If a channel has a GenericSubsamplingType of 0, the value of GenericSubsamplingSampleLocType for that channel is ignored.

Examples:

- GenericSubsamplingType [0,2,2] and GenericSubsamplingSampleLocType [2,1,1] corresponds to 4:2:0 subsampling with centre-centre chroma location (i.e. Chroma420SampleLocType 1).

- GenericSubsamplingType [0,3,3] and GenericSubsamplingSampleLocType [2,1,1] corresponds to 4:1:1 subsampling with centre chroma location.

# Patent rights declaration(s)

Apple Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).