| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0218-v1 |
| --- | --- |

| Title: | EE2-4.1 related: Alternative intra mode derivation for LFNST/NSPT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This proposal proposes to improve EE2-Test 4.1 [1] by allowing the 2nd DIMD intra prediction mode (IPM) as an alternative LFNST/NSPT IPM candidate. It is reported that on top of ECM-12.0, simulation results of the proposed method are as below:

RA: -0.18 % (Y), 0.06 % (U), 0.11 % (V), xxx% (EncT), xxx% (DecT)

LB: xxx % (Y), xxx % (U), xxx % (V), xxx% (EncT), xxx% (DecT) .

On top of Test 4.1, the results are reported as:

RA: -xxx % (Y), xxx % (U), xxx% (V), xxx%% (EncT), xxx% (DecT)

LB: -xxx % (Y), -xxx % (U), -xxx % (V), xxx% (EncT), xxx% (DecT).

- Introduction

In EE2 Test-4.1, the LFNST/NSPT transform set for for certain intra modes, i.e., DIMD, TIMD, MIP, SGPM, EIP and ITMP, is selected out of two candidate sets, which corresponds to two different intra prediction modes (IPM). The derivation of the two intra modes is as follows:

Table 1: The LFNST/NSPT multiple intra modes derivation in TEST-4.1

| Intra mode | Derived two intra modes for LFNST/NSPT |
| --- | --- |
| SGPM | Split mode + sgpmMode0 (sgpmMode1) |
| TIMD | 1st TIMD IPM + 2nd TIMD IPM |
| DIMD, ITMP | 1st DIMD IPM+ DIMD IPM derived with neighboring pixels |
| MIP, EIP | 1st DIMD IPM + Planar |

- Proposed method

In this proposal, the candidate LFNST/NSPT IPMs for MIP, EIP and ITMP are modified, where the intermediate results of DIMD processing are reused. In particular, the first LFNST/NSPT IPM for MIP, EIP and ITMP remains the 1st DIMD IPM, while the second IPM is replaced by the 2nd DIMD IPM derived based on the prediction signal of the current block. The proposed multiple intra mode derivation for LFNST/NSPT is summarized as in Table 2.

Table 2: The proposed LFNST/NSPT multiple intra modes derivation

| Intra mode | Derived two intra modes for LFNST/NSPT |
| --- | --- |
| SGPM | Split mode + sgpmMode0 (sgpmMode1) |
| TIMD | 1st TIMD IPM + 2nd TIMD IPM |
| DIMD | 1st DIMD IPM+ DIMD IPM derived with neighboring pixels |
| MIP, EIP, ITMP | 1st DIMD IPM + 2nd DIMD IPM |

- Experimental results

The proposed method has been implemented on top of ECM-12.0 [1] and EE2 Test 4.1(test_a in JVET-AH0056). Results under AI/RA configurations are presented in Table 3 and Table 4. The simulations were performed in accordance with the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 3: Coding performance of the proposed method on ECM-12.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.21% | 0.03% | -0.01% |  |  |  |  |
| Class A2 | -0.13% | 0.09% | 0.05% |  |  |  |  |
| Class B | -0.18% | 0.11% | 0.13% |  |  |  |  |
| Class C | -0.17% | 0.08% | 0.21% |  |  |  |  |
| Class E | -0.21% | -0.05% | 0.11% |  |  |  |  |
| Overall | -0.18% | 0.06% | 0.11% |  |  |  |  |
| Class D | -0.19% | 0.25% | 0.08% |  |  |  |  |
| Class F | -0.12% | 0.24% | 0.08% |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.12% | 0.03% | -0.32% |  |  |  |  |
| Class C | -0.03% | -0.10% | -0.13% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | 0.05% | 0.12% |  |  |  |  |
| Class F | 0.01% | 0.14% | -0.05% |  |  |  |  |

Table 4: Coding performance of the proposed method on Test-4.1 test_a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.03% | -0.09% | -0.05% |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | 0.02% | -0.01% |  |  |  |  |
| Class C | -0.06% | 0.01% | -0.04% |  |  |  |  |
| Class E | -0.03% | -0.19% | 0.03% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.04% | 0.17% | 0.05% |  |  |  |  |
| Class F | -0.04% | 0.24% | 0.09% |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.17% | -0.16% |  |  |  |  |
| Class C | -0.02% | 0.01% | -0.13% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.02% | 0.16% | -0.43% |  |  |  |  |
| Class F | 0.07% | 0.26% | -0.03% |  |  |  |  |

- Conclusions

An improved LFNST/NSPT multiple IPM derivation method is presented in this contribution. Simulation results indicate xxx%/xxx% coding gain in RA/LDB configurations beyond EE2 Test-4.1, without     complexity change. It is recommended to include the proposed methods in the next round of EE.

- References

[1] F. Wang, Y. Yu, H. Yu, D. Wang, “Non-EE2: Multiple Transform Set Selection for LFNST/NSPT,” JVET-AG0062, Jan. 2024.

[2] ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).