| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0219-v1 |
| --- | --- |

| Title: | Non-EE2: Improvements on Inter LFNST/NSPT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to enhance inter LFNST/NSPT in two aspects. Firstly, a multiple LFNST/NSPT kernel set selection method is applied to GPM-coded blocks. Secondly, inter LFNST/NSPT is enabled for SBT-coded blocks. It is reported that on top of ECM-12.0, simulation results of the proposed method are as below:

RA: -xxx % (Y), -xxx% (U), -xxx % (V), xxx% (EncT), xxx% (DecT)

LB: -xxx % (Y), -xxx % (U), -xxx % (V), xxx% (EncT), xxx% (DecT) .

- Introduction

In ECM-12.0, LFNST/NSPT can be applied to inter-coded blocks, where the transform kernels for intra LFNST/NSPT are reused by inter blocks. Based on the prediction signal of the current block, a Histogram of Gradients (HoG) is built in a similar way as DIMD, and the 1st DIMD intra prediction mode (IPM) corresponds to the highest amplitude is used to determine the LFNST/NSPT kernel set. It is asserted that a single IPM may not sufficiently profile a GPM-coded block. Besides, in ECM-12.0, inter LFNST/NSPT cannot be applied to an SBT-coded block.

- Proposed method

In this proposal, two aspects are investigated to enhance inter LFNST/NSPT:

Aspect #1: Multiple LFNST/NSPT kernel set for GPM mode.

For a GPM-coded block, an alternative IPM is derived for inter LFNST/NSPT. Beside the 1st DIMD IPM, a second DIMD IPM which corresponds to the 2nd highest HOG amplitude is used as an additional IPM candidate. A CU-level flag indicating the IPM index is signaled.

Aspect #2: LFNST/NSPT for SBT-coded blocks.

It is proposed to enable LFNST/NSPT for SBT-coded blocks. A CU-level LFNST/NSPT index is signaled for an SBT-coded block to indicate the usage of LFNST/NSPT. When LFNST/NSPT is applied to an SBT-coded CU, the TU with non-zero residual will perform LFNST/NSPT, and the prediction signal within the TU region is used to derive the IPM for transform.

- Experimental results

The proposed method has been implemented on top of ECM-12.0 [1]. Results under RA configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method on ECM-12.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.07% | 0.07% | 0.14% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.10% | 0.14% | 0.28% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

- Conclusions

Two aspects to improve inter LFNST/NSPT are presented in this contribution. Simulation results indicate xxx% coding gain in RA configurations upon ECM-12.0 with reasonable runtime increase. It is recommended to include the proposed methods in the next round of EE2.

- References

[1] ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).