| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0222-v2 |
| --- | --- |

| Title: | Non-EE2: GPM-Affine with MMVD and TM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Li Zhang2 1Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents to harmonize GPM-Affine with GPM-MMVD (GPM-Affine-MMVD), and GPM-TM (GPM-Affine-TM). With GPM-Affine-MMVD, a signaled MV offset is added to all CPMVs of an affine candidate, and the modified CPMVs are used for predicting a GPM sub-partition. With GPM-Affine-TM, an affine candidate could be refined by template matching and used for predicting a GPM sub-partition. On top of ECM-12.0, simulation results of the proposed method are reported as follows:

RA: {-0.06%, -0.09%, -0.15%; 102.9%, 100.5%};

LDB: {-0.08%, -0.21%, -0.23%; 101.4%, 100.3%}.

Introduction

Geometry partitioning mode with affine prediction (GPM-Affine) is used for predicting a GPM sub-partition with affine motion compensation. However, in the existing design of ECM, GPM-Affine is disallowed for GPM-MMVD and GPM-TM. The coding performance could be improved by harmonizing GPM-Affine with GPM-MMVD and GPM-TM.

Proposed method

In this contribution, GPM-Affine is further combined with GPM-MMVD and GPM-TM.

With GPM-Affine-MMVD, an MV offset is added to all CPMVs of an affine candidate and the modified CPMVs are used for predicting a GPM sub-partition. The signaling of the affine candidate and the MV offset is the same as GPM-Affine and GPM-MMVD.

With GPM-Affine-TM, an affine candidate could be refined by template matching using the same method in affine mode. A GPM sub-partition can be predicted by GPM-Affine-TM or GPM-TM.

Simulation results

The proposed method is implemented on top of ECM-12.0 [1]. Simulations are performed following the common test conditions of EE2 [2]. The simulation results of the proposed method compared to ECM-12.0 are shown in Table 1.

Table 1: Coding performance compared to ECM-12.0

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.17% | -0.15% | 102.4% | 100.5% |  |  |  |  |  |
| Class A2 | -0.10% | -0.10% | -0.10% | 102.2% | 101.0% |  |  |  |  |  |
| Class B | -0.07% | 0.02% | -0.14% | 103.0% | 100.3% | -0.04% | -0.09% | -0.08% | 101.1% | 100.0% |
| Class C | -0.06% | -0.15% | -0.19% | 103.8% | 100.2% | -0.08% | -0.15% | 0.16% | 102.2% | 101.3% |
| Class E |  |  |  |  |  | -0.15% | -0.50% | -1.00% | 101.0% | 99.6% |
| Overall | -0.06% | -0.09% | -0.15% | 102.9% | 100.5% | -0.08% | -0.21% | -0.23% | 101.4% | 100.3% |
| Class D | -0.11% | -0.18% | -0.14% | 103.7% | 99.9% | -0.22% | 0.13% | -0.38% | 102.2% | 101.1% |
| Class F | -0.03% | -0.07% | -0.12% | 102.8% | 99.9% | 0.13% | -0.52% | -0.49% | 102.3% | 98.8% |

Conclusions

In this contribution, GPM-Affine with MMVD and GPM-Affine with TM are proposed. It is recommended to study the proposed method in EE2.

Reference

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, Hannover, DE, 13–20 October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).