| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0224 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0086 (EE2-2.9: EIP with bias and clipping) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Luhang Xu | Tel: Email: | - xuluhang@oppo.com |
| Source: |  |  |  |

_____________________________

- Abstract

This contribution provides the crosschecking results of EE2-2.9a and EE2-2.9b on EIP (extrapolation-based intra prediction) with bias and clipping. The simulation results are shown as follows

AI {-0.02%/0.00%/0.00% for Y/U/V} EE2-2.9a with a bias term
AI {-0.01%/0.02%/0.04% for Y/U/V} EE2-2.9b with clipping

Introduction

This document reports the crosscheck results of EE2-2.9a and EE2-2.9b. It is confirmed that the implementation and the simulation results are matched to those provided by the proponent.

EE2-2.9a (EIP with bias)

It is proposed to include a bias term to the EIP filter models to make the recursive filter operation more likely to represent textures in the prediction block’s neighborhood. This is done by substituting the last spatial input to the filter (the furthest away sample from the output sample location) by a bias term given as 2B-1, where B is the bitdepth of the channel. That is, the filters are modified to the following form:

predx,y=i=013(ci×tx-offsetXi, y-offsetYi)+c14×2B-1

EE2-2.9b (EIP with clipping)

It is proposed to clip the predicted sample values to the range of the reference samples instead of the full sample value range. Reference sample area used for determining the range is the same that is used when generating the filter coefficients.

Crosscheck results

EE2-2.9a (EIP with bias)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.02% | 0.01% | 99.9% | 100.7% | 100.4% | 100.0% |
| Class A2 | -0.05% | -0.03% | 0.01% | 100.2% | 100.9% | 100.0% | 100.1% |
| Class B | -0.04% | 0.03% | 0.09% | 100.3% | 100.4% | 99.3% | 100.0% |
| Class C | -0.02% | -0.01% | -0.04% | 101.1% | 101.8% | 100.0% | 100.0% |
| Class E | 0.02% | -0.01% | -0.08% | 100.8% | 101.8% | 100.0% | 100.0% |
| Overall | -0.02% | 0.00% | 0.00% | 100.5% | 101.1% | 99.9% | 100.0% |
| Class D | -0.03% | 0.20% | -0.09% | 100.3% | 104.0% | 100.0% | 100.0% |
| Class F | 0.02% | 0.13% | -0.04% | 99.7% | 101.1% | 100.0% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

EE2-2.9b (EIP with clipping)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.01% | 0.07% | 100.4% | 101.8% | 100.0% | 100.0% |
| Class A2 | -0.03% | 0.04% | 0.03% | 100.5% | 102.3% | 100.0% | 100.0% |
| Class B | -0.02% | 0.07% | 0.07% | 100.3% | 101.2% | 99.3% | 100.0% |
| Class C | -0.01% | -0.01% | 0.15% | 100.7% | 102.4% | 100.0% | 100.0% |
| Class E | -0.02% | -0.02% | -0.15% | 100.7% | 102.4% | 100.0% | 100.0% |
| Overall | -0.01% | 0.02% | 0.04% | 100.5% | 102.0% | 99.8% | 100.0% |
| Class D | -0.03% | 0.17% | -0.06% | 100.2% | 103.8% | 100.0% | 100.0% |
| Class F | 0.04% | 0.23% | 0.10% | 98.5% | 101.6% | 100.0% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

*The EncT and DecT are not accurate.