| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0234 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0136 (EE2-2.12: Chroma intra prediction mode reordering) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Luhang Xu | Tel: Email: | - xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution provides the crosscheck results of EE2 Test 2.12 which studies reordering the non-CCP chroma intra prediction modes. The partial coding performance for {Y, U, V } are:

- Test 2.12a: reordering based on the existing list in ECM-12.0:

- Class-C {-0.02%, -0.07%, 0.03%} for AI

- Class-D {-0.04%, 0.20%, -0.15%} for AI.

- Test 2.12b: reordering based on an extended list in ECM-12.0:

- Class-C {-0.05%, 0.03%, -0.03%, 100.0%, 101.0%} for AI

- Class-D {-0.06%, 0.13%, -0.05%, 100.0%, 101.0%} for AI.

Introduction

In ECM-12.0, there are 7 non-CCP chroma intra prediction modes: chroma DBV mode, DM mode, chroma DIMD mode, and four default modes (i.e. planar mode, vertical mode, horizontal mode and DC mode). A fixed order is used to parse the non-CCP modes.

In EE2 Test 2.12, a non-CCP chroma intra prediction mode list is constructed. In Test 2.12a, the list was constructed by adding the existing 7 non-CCP intra prediction modes in ECM-12.0. In Test 2.12b, this list is further extended by adding additional candidate modes from pre-defined positions.

It is confirmed that the implantation and results are matched to the original proposal and the data from the proponents.

Crosscheck results

The simulations were conducted and partial results are shown in this section. The simulation time was not accurate.

Table-1. The simulation results of TEST-b against ECM-12.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class C | -0.02% | -0.07% | 0.03% | 103.1% | 105.9% | 100.4% | 100.3% |
| Class D | -0.04% | 0.20% | -0.15% | 102.6% | 107.4% | 100.4% | 100.1% |

Table-2. The simulation results of TEST-b against ECM-12.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class C | -0.05% | -0.03% | 0.03% | 104.1% | 107.5% | 100.7% | 100.2% |
| Class D | -0.06% | 0.13% | -0.05% | 102.8% | 107.2% | 100.4% | 100.1% |