| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0236 |
| --- | --- |

| Title: | Crosscheck of EE2-Test 3.2a and 3.2b: Alignment of temporal buffer usage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of EE2-Test 3.2a and 3.2b, i.e., alignment of temporal buffer usage, proposed in JVET-AH0068 in the context of Exploration Experiments 2 (JVET-AF2024). The cross-checker confirms that the software provided by the proponents matches the algorithm description of JVET-AH0068. All the testing results in this cross-check report match those provided by the proponent.

Summary

JVET-AH0068 [1] proposes an alignment of temporal buffer usage and reports its testing results over ECM-12.0 [2]. ECM-12.0 has misalignment of temporal buffer usage in several motion tools. For instance, some tools (Tools A: e.g., TMVP candidate derivation in inter merge, BM merge, affine merge, etc.) use an only one-CTU-row buffer, while other tools (Tools B: e.g., intra and inter CCP merge modes) can use the entire collocated picture.

EE2-Test 3.2a investigates the effect where the one-CTU-row buffer restriction is removed from Tools A of ECM-12, whereas EE2-Test 3.2b investigates the effect where the one-CTU-row buffer restriction is imposed into Tools B of ECM-12.

The cross-checker carefully analyzed the software provided by the proponents [3] where the Test 3.2a and Test 3.2b are implemented on top of ECM-12.0 [2]. Then, we confirmed that the software implementation perfectly matches the algorithm description of JVET-AH0068.

The cross-checker conducted simulations using the provided software according to the EE2 description [4] and the common test condition for enhanced compression tool testing [5]. The cross-check results are provided in the accompanying Excel sheets. The cross-checker could exactly reproduce the results provided by the proponent and reported in JVET-AH0068. The cross-checker compiled the provided software on a 64-bit Ubuntu environment. The experiments were performed on the cluster PCs.

Conclusion

The cross-checker has verified the description and results reported in JVET-AH0068. It is confirmed that the software provided by the proponent matches the proposal description, and the testing results produced by the cross-checker match the reported results.

Reference

- Z. Deng, K. Zhang, L. Zhang, Y. Kidani, H. Kato, K. Kawamura, “EE2-3.2: Alignment of temporal buffer usage,” JVET-AH0068, April 2024.

- EE2-3.2, https://vcgit.hhi.fraunhofer.de/ecm/jvet-ag-ee2/ECM/-/tree/TEST_3.2

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AG2024, January 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023.