| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0248-v1 |
| --- | --- |

| Title: | AHG8: Reduced residual encoding in VVC for machine consumption |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou, Antti Hallapuro, Honglei Zhang Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This is follow-up contribution of JVET-AG0217 where the proposed method has also been tested in combined with Region of Interest (RoI) technique. This contribution proposes to scale down residual values of inter blocks at the VTM encoder for machine consumption. The value of the scaling function is set to unity at the block boundary, while its value decreases toward center of the block to 0.7. This scaling guarantees continuity of the reconstructed frame at boundaries of coding units, while encoding the residual of the coding unit potentially with a smaller number of bits. DC component of the residual block is deducted first, scaling is applied to the deducted residual, and then DC component is added back to the scaled residual. The proposed method is not applied to small coding units (i.e., width or height less than or equal to 4/2 for RA/LD). Distortion calculation and rate-distortion optimization is performed according to the existing process in VTM encoder. The contribution proposes to add this method to the technical report on optimization of encoders and receiving systems for machine analysis of coded video content. Average pareto BD-Rate results are as follows:

With original contents:

Object detection (SFU-HW): RA= -0.56%, LD = -2.84%

Object tracking (TVD): RA= -4.02%, LD = -2.41%

With RoI contents:

Object detection (SFU-HW): RA= -1. 68%, LD = -2.95%

Object tracking (TVD): RA= 2.93%, LD = -1.19%

# Introduction

This is follow-up contribution of JVET-AG0217 where reduced residual coding was proposed for machine analysis use case. In previous meeting, it was requested to test the performance of this method in combined with other related coding techniques for machine analysis. In this contribution, the performance of reduced residual techniques is analyzed when it is combined with RoI technique.

In recent JVET activities to optimize VVC/VTM encoder for encoding video content suitable for machine consumption, several methods have been studied and included in the technical report document ‎[1]. These methods include: 1. Pre-processing technologies (e.g., region of interest (RoI)-based coding, pre-processing of foreground and background regions, temporal subsampling, spatial subsampling, and de-noising filter), 2. encoding technologies (e.g., QP adaption for region of interest coding, QP change for temporal layers), 3. post-processing techniques (e.g., temporal resampling, spatial resampling), and 4. Metadata (e.g., annotated regions SEI, and NN post-filtering SEI). Some of techniques are task-aware, for example, RoI-based techniques. This contribution proposes a task-agnostic technique which improves the RD performance of video encoding for machines by reducing inter-prediction residual.

# Proposed reduced residual method

## Main idea

The proposed method optimizes inter-frame encoding by reducing inter-frame residual encoding for machine consumption. The main motivation is that there is no need to code each frame the same way as it is for human visual system. It is possible to code the content using less bits for residual data without sacrificing the performance of the machine tasks. The main idea, as shown in Figure 1, is to encode/reconstruct a current frame using the previously coded frames mainly by copying (i.e., inter predicting) the content of coding units from reference frames to the current frame with scaled down residual to reduce residual and consequently total bitrate. To keep the content’s continuity between coding units, residuals around coding unit boundaries shall be coded, meaning that the residual values of a coding unit is reduced (i.e., scaled down) in the center of the coding unit, while keeping the residual at the locations close to coding unit boundaries. Described in more details later, the residual scaling has not been applied to small blocks and the DC component of the residual. The proposed scaling of inter-residual is shown in Figure 2.

Figure 1. Reconstruction of the current frame by inter predicting coding units from the reference frame

Figure 2. Scaling of inter-residual in the proposed method

## Detail of the method

According to the proposed method, the residuals of an inter-block are scaled by scaling factors defined by a two-dimensional scaling function, shown in Figure 3, for each coding unit. The value of the scaling function is set to unity at the block boundary, while its value decreases toward center of the block to 0.7 for random access (RA) configuration. A slightly different scaling function is used for low delay (LD) configuration. The DC component of the residual block is coded easily and applying scaling increases the bitrate. So, in the proposed method, DC component of the residual block is deducted first, scaling is applied to the deducted residual, and then DC component is added back to the scaled residual.

DC = DC of Residual block R(x,y)

R_dc(x,y) = R(x,y) - DC

sR_dc(x,y) = R_dc(x,y)*S(x,y)

sR(x,y) = sR_dc(x,y) + DC

where x and y are the coordinates of a sample inside the residual block with respect to the top-left corner of the block, R(x,y), S(x,y), and sR(x,y) are the value of residual, scaling factor and scaled residual at location (x,y), respectively.

Figure 3. The scaling function of a coding unit (the example for 16x16 coding unit, center value = 0.7)

The scaled residual is encoded using transform, quantization and entropy coding process. At the decoder side, the scaled residual is decoded according to the decoding process of VVC, and is added to prediction signal to generate the reconstructed block. Distortion calculation and rate-distortion optimization is performed according to existing process in VTM encoder.

## Combination with RoI method

The performance of reduced residual technique is analyzed in combined with RoI method. In this test, the original input YUV files (Figure 4-(a)) have been processed by running VCM-RS v0.8 software and the RoI-processed YUV files (Figure 4-(b)) are collected from the output of RoI component. Then reduced residual coding technique has been used to code RoI-processed input data. The results as been compared to when RoI-processed input data coded with original VTM20.0.

| (a) | (b) |
| --- | --- |

Figure 4. Input contents for different tests, (a) unprocessed input, (b) RoI-processed input.

# Experimental setup

## Results on the unprocessed inputs

The proposed reduced residual methos is evaluated using common test conditions of JVET-AF2031‎[2]. Results are shown below. These results were cross-checked in JVET-AG0317.

Random access:

|  | Object Detection | End-to-End BD-Rate [%] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP | Luma | Chroma Cb | Chroma Cr |
| SFU-HW | Class A | NA | NA | -1.98% | 0.09% | -0.15% | -0.01% |
|  | Class B | NA | -1.46% | -2.99% | 0.22% | -0.58% | -0.60% |
|  | Class C | -0.11% | -1.31% | -0.11% | 0.48% | -0.49% | -0.17% |
|  | Class D | 1.78% | -0.79% | 1.78% | 0.77% | 0.19% | 0.25% |
|  | Average | NA | NA | -0.56% | 0.46% | -0.28% | -0.16% |

|  | Object Tracking | End-to-End BD-Rate [%] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | Luma | Chroma Cb | Chroma Cr |
| TVD | TVD-01-1 | NA | -7.46% | -5.19% | 0.04% | -0.67% | -0.16% |
|  | TVD-01-2 | NA | NA | -2.66% | 0.04% | -0.53% | -0.36% |
|  | TVD-01-3 | NA | -7.66% | -5.48% | 0.13% | -0.07% | 0.21% |
|  | TVD-02-1 | -8.64% | -13.37% | -8.64% | 0.24% | -0.12% | -0.15% |
|  | TVD-03-1 | -1.18% | -2.55% | -1.18% | 0.02% | -0.29% | -0.03% |
|  | TVD-03-2 | NA | -3.48% | -2.63% | 0.27% | -0.45% | -0.27% |
|  | TVD-03-3 | -2.40% | -3.56% | -2.40% | 0.59% | -0.59% | -0.52% |
|  | Average | NA | NA | -4.02% | 0.19% | -0.39% | -0.18% |

Low delay:

|  | Object Detection | End-to-End BD-Rate [%] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP | Luma | Chroma Cb | Chroma Cr |
| SFU-HW | Class A | -8.12% | -3.37% | -8.12% | -0.11% | -0.46% | -0.36% |
|  | Class B | NA | -1.93% | -2.53% | -0.01% | -1.33% | -0.99% |
|  | Class C | -0.37% | 0.39% | -0.37% | 0.61% | -0.70% | -0.75% |
|  | Class D | NA | NA | -4.31% | 1.23% | -0.63% | -1.17% |
|  | Average | NA | NA | -2.84% | 0.55% | -0.85% | -0.92% |

|  | Object Tracking | End-to-End BD-Rate [%] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | Luma | Chroma Cb | Chroma Cr |
| TVD | TVD-01-1 | 4.13% | -2.53% | 4.13% | -0.06% | -2.53% | -2.65% |
|  | TVD-01-2 | NA | 8.18% | 5.27% | 0.52% | -2.13% | -4.79% |
|  | TVD-01-3 | -5.79% | 2.09% | -5.79% | 0.13% | -2.15% | -1.24% |
|  | TVD-02-1 | NA | -2.56% | -3.38% | 0.84% | -0.07% | -0.61% |
|  | TVD-03-1 | -6.19% | -5.53% | -6.19% | -0.54% | -2.00% | -1.46% |
|  | TVD-03-2 | NA | NA | -10.56% | 0.26% | -1.67% | -2.62% |
|  | TVD-03-3 | -0.38% | -8.62% | -0.38% | 0.67% | -1.52% | -2.14% |
|  | Average | NA | NA | -2.41% | 0.26% | -1.72% | -2.22% |

## Results on the RoI-processed inputs

The results of the reduced residual on RoI-processed inputs are reported below. In this case, anchor is RoI-processed inputs encoded with original VTM20.0.

Random access

|  | Object Detection | End-to-End BD-Rate [%] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP | Luma | Chroma Cb | Chroma Cr |
| SFU-HW | Class A | NA | NA | 5.14% | 0.13% | -0.81% | -0.16% |
|  | Class B | NA | NA | -0.58% | 0.24% | -0.68% | -0.42% |
|  | Class C | -0.24% | 1.37% | -0.24% | 0.47% | -0.08% | 0.01% |
|  | Class D | -5.92% | -2.78% | -5.92% | 0.85% | -0.59% | 0.05% |
|  | Average | NA | NA | -1.68% | 0.49% | -0.48% | -0.12% |

|  | Object Tracking | End-to-End BD-Rate [%] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | Luma | Chroma Cb | Chroma Cr |
| TVD | TVD-01-1 | -3.93% | -9.75% | -3.93% | 0.37% | 0.30% | 0.28% |
|  | TVD-01-2 | NA | NA | 20.61% | 0.30% | -0.20% | -0.29% |
|  | TVD-01-3 | NA | -5.47% | -3.41% | 0.33% | 0.23% | -0.12% |
|  | TVD-02-1 | 4.57% | 2.02% | 4.57% | 0.47% | -0.01% | 0.16% |
|  | TVD-03-1 | -1.83% | -1.42% | -1.83% | 0.45% | 0.15% | 0.29% |
|  | TVD-03-2 | NA | 4.42% | 7.92% | 0.61% | 0.31% | 0.73% |
|  | TVD-03-3 | -3.40% | -5.22% | -3.40% | 0.67% | -0.11% | -0.17% |
|  | Average | NA | NA | 2.93% | 0.46% | 0.10% | 0.13% |

Low delay:

|  | Object Detection | End-to-End BD-Rate [%] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP | Luma | Chroma Cb | Chroma Cr |
| SFU-HW | Class A | -14.86% | -18.31% | -14.86% | 0.13% | -0.19% | 0.16% |
|  | Class B | -4.46% | -3.00% | -5.64% | 0.28% | -0.56% | -1.70% |
|  | Class C | 0.50% | -0.49% | 0.50% | 0.70% | -0.41% | -0.86% |
|  | Class D | -0.75% | -1.47% | -0.75% | 1.05% | -1.73% | -1.37% |
|  | Average | -2.59% | -2.93% | -2.95% | 0.64% | -0.84% | -1.20% |

|  | Object Tracking | End-to-End BD-Rate [%] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MOTA | MOTA (low4) | Pareto MOTA | Luma | Chroma Cb | Chroma Cr |
| TVD | TVD-01-1 | 0.50% | 7.45% | 0.50% | 2.01% | -1.46% | -2.86% |
|  | TVD-01-2 | NA | NA | -18.10% | 1.51% | 3.16% | 3.39% |
|  | TVD-01-3 | NA | 13.02% | 15.75% | 1.55% | NA | -0.94% |
|  | TVD-02-1 | 2.00% | 7.51% | 2.00% | 1.12% | 1.46% | 0.77% |
|  | TVD-03-1 | 3.62% | 2.84% | 3.62% | 0.92% | -0.70% | -1.98% |
|  | TVD-03-2 | -8.66% | -11.86% | -8.66% | 1.48% | -1.99% | 1.35% |
|  | TVD-03-3 | -3.44% | -4.81% | -3.44% | 1.51% | 0.45% | -0.15% |
|  | Average | NA | NA | -1.19% | 1.44% | NA | -0.06% |

# Proposal

The proposed text to be added to the TR as a new encoding sub-section 7.3 in the next version:

Residual block is calculated at the encoder side as the difference between the original block and the prediction block. However, this is a completely non-normative process, and encoders have choices to manipulate the residual values to optimize the performance of the encoder for the required purpose, without modifying the decoding process. For most of the machine tasks, there is no need to code each frame the same way as it is for human visual system. It is possible to code the content using less bits for residual data without sacrificing the performance of the machine tasks. As shown in Figure 5, a current frame can be coded using the previously coded frames mainly by copying (i.e., inter predicting) the content of coding units from reference frames to the current frame with scaled down residual to reduce residual and consequently total bitrate. To keep the content’s continuity between coding units, residuals around coding unit boundaries can be coded, meaning that the residual values of a coding unit is reduced (i.e., scaled down) in the center of the coding unit, while keeping the residual at the locations close to coding unit boundaries. The residual scaling can be skipped for small blocks and the DC component of the residual. Figure 6 shows the block diagram of an encoder with scaling of inter-residual.

Figure 5. Reconstruction of the current frame by inter predicting coding units from the reference frame

Figure 6. The block diagram of an encoder with scaling of inter-residual

In an example implementation, the residuals of an inter-block are scaled by scaling factors defined by a two-dimensional scaling function for each coding unit. As shown in Figure 7, the value of the scaling function is set to unity at the block boundary, while its value decreases toward center of the block to 0.7. The DC component of the residual block is coded easily and applying scaling increases the bitrate. In an example implementation, DC component of the residual block is deducted first, the scaling function, is applied to the deducted residual, and then DC component is added back to the scaled residual.

DC = DC of Residual block R(x,y)

R_dc(x,y) = R(x,y) - DC

sR_dc(x,y) = R_dc(x,y)*S(x,y)

sR(x,y) = sR_dc(x,y) + DC

where x and y are the coordinates of a sample inside the residual block with respect to the top-left corner of the block, R(x,y), S(x,y), and sR(x,y) are the value of residual, scaling factor and scaled residual at location (x,y), respectively.

Figure 7. The scaling function of a coding unit (the example for 16x16 coding unit, center value = 0.7)

The scaled residual is encoded using transform, quantization and entropy coding process. At the decoder side, the scaled residual is decoded according to the decoding process of VVC, and is added to prediction signal to generate the reconstructed block. Distortion calculation and rate-distortion optimization is performed according to existing process in VTM encoder.

# References

- ISO/IEC JTC 1/SC 29, Optimization of encoders and receiving systems for machine analysis of coded video content (draft 3), JVET-AE2030

- ISO/IEC JTC 1/SC 29, Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content, JVET-AF2031

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).