| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0250-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0081 (EE1-1.3: Training from scratch for the joint LOP model with inputs transformed) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Maria Santamaria Nokia Technologies, Tampere, Finland | Email: | maria.santamaria_gomez@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the training and inference crosscheck results for JVET-AH0081, which studies an LOP filtering with inputs in the frequency domain. The inputs to the filter are transformed to the frequency domain with the DCT and the outputs are transformed back to the spatial domain with the IDCT. JVET-AH0081 has 3 training stages. It is reported that for the inference cross-check of stage 1 matching results were obtained: 0.00% Y, 0.00% Cb, 0.00% Cr (AI), while for the training cross-check small differences were obtained: -0.13% Y, -0.09% Cb , -0.34% Cr (AI). It is also reported that for the inference cross-check of stage 2 matching results were obtained: 0.00% Y, 0.00% Cb, 0.00% Cr (AI and RA), while for the training cross-check small differences were obtained: 0.03% Y, -0.02% Cb , -0.66% Cr (AI) and -0.04% Y, -0.10% Cb, -1.12% Cr (RA). The stage 3 crosscheck is currently work in progress.

# Introduction

JVET-AH0081 [1] describes a LOP in-loop filter that process the data in the frequency domain. The software package that includes inference code and training scripts is available at [2]. The inference of stages 1 and 2 is over NNVC-7.1, while the inference of stage 3 is over NNVC-8.0.

## Stage 1

The stage 1 training dataset is the same as the one used to train LOP2 filter [3] (DIV2K). The inference is done using floating-point precision models and the anchor corresponds to the results reported in JVET-AH0081.

Table 1 shows the inference crosscheck, for which the model trained by the proponents was used.

Table 1. Stage 1 AI inference crosscheck

|  | BD-rate Over JVET-AH0081 |  |  |
| --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0,00 % | 0,00 % | 0,00 % |
| Class A2 | 0,00 % | -0,01 % | 0,00 % |
| Class B | 0,00 % | 0,00 % | 0,00 % |
| Class C | 0,00 % | -0,01 % | 0,00 % |
| Class E | 0,00 % | 0,00 % | 0,00 % |
| Overall | 0,00 % | 0,00 % | 0,00 % |
| Class D | 0,00 % | 0,02 % | 0,02 % |
| Class F | 0,00 % | 0,00 % | 0,00 % |

Table 2 shows the training crosscheck results. In this case, a new stage 1 model was trained.

Table 2. Stage 1 AI training crosscheck

|  | BD-rate Over JVET-AH0081 |  |  |
| --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0,30 % | -0,04 % | -0,01 % |
| Class A2 | -0,07 % | 0,12 % | -0,29 % |
| Class B | -0,11 % | -0,50 % | -0,31 % |
| Class C | -0,10 % | 0,04 % | -0,55 % |
| Class E | -0,07 % | 0,14 % | -0,47 % |
| Overall | -0,13 % | -0,09 % | -0,34 % |
| Class D | -0,07 % | -0,15 % | -0,27 % |
| Class F | -0,43 % | -0,19 % | -1,67 % |

## Stage 2

The float model trained by the proponents during stage 1 is used to generate the stage 2 datasets (BVI-DVC and TVD). Also, the stage 1 dataset is reused. The inference is done using int16 models the anchor corresponds to the results reported in JVET-AH0081.

Table 3 and Table 4 show the AI and RA inference crosscheck, respectively, for which the model trained by the proponents was used.

Table 3. Stage 2 AI inference crosscheck

|  | BD-rate Over JVET-AH0081 |  |  |
| --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0,00 % | 0,00 % | 0,00 % |
| Class A2 | 0,00 % | 0,00 % | 0,00 % |
| Class B | 0,00 % | 0,00 % | 0,00 % |
| Class C | 0,00 % | 0,00 % | 0,00 % |
| Class E | 0,00 % | 0,00 % | 0,00 % |
| Overall | 0,00 % | 0,00 % | 0,00 % |
| Class D | 0,00 % | 0,00 % | 0,00 % |
| Class F | 0,00 % | 0,00 % | 0,00 % |

Table 4. Stage 2 RA inference crosscheck

|  | BD-rate Over JVET-AH0081 |  |  |
| --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0,01 % | 0,00 % | 0,00 % |
| Class A2 | 0,00 % | 0,00 % | -0,01 % |
| Class B | 0,00 % | -0,01 % | 0,00 % |
| Class C | 0,00 % | 0,00 % | 0,00 % |
| Class E |  |  |  |
| Overall | 0,00 % | 0,00 % | 0,00 % |
| Class D | 0,00 % | 0,00 % | 0,00 % |
| Class F | -0,01 % | 0,00 % | 0,00 % |

Table 5 and Table 6 show the training crosscheck results for AI and RA, respectively. In this case, a new stage 2 model was trained, and its performance is compared against the stage 2 model provided by the proponents.

Table 5. Stage 2 AI training cross-check

|  | BD-rate Over JVET-AH0081 |  |  |
| --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0,03 % | 0,32 % | -1,13 % |
| Class A2 | 0,00 % | 0,07 % | -0,65 % |
| Class B | 0,02 % | 0,07 % | -0,66 % |
| Class C | 0,10 % | -0,06 % | -0,90 % |
| Class E | 0,03 % | -0,29 % | 0,09 % |
| Overall | 0,03 % | 0,02 % | -0,66 % |
| Class D | 0,09 % | -0,36 % | -0,94 % |
| Class F | 0,13 % | 0,20 % | -0,75 % |

Table 6. Stage 2 RA training crosscheck

|  | BD-rate Over JVET-AH0081 |  |  |
| --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0,13 % | -0,21 % | -1,26 % |
| Class A2 | -0,13 % | 0,02 % | -1,07 % |
| Class B | -0,06 % | 0,16 % | -1,18 % |
| Class C | 0,11 % | -0,42 % | -0,98 % |
| Class E |  |  |  |
| Overall | -0,04 % | -0,10 % | -1,12 % |
| Class D | 0,01 % | -0,90 % | -1,71 % |
| Class F | 0,05 % | -0,58 % | -0,05 % |

# Stage 3

Training and inference crosscheck are work in progress.

# References

- D. Liu, J. Ström, M. Damghanian, P. Wennersten, D. Rusanovskyy, Y. Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya. EE1-1.3: Training from scratch for the joint LOP model with inputs transformed. JVET-AH0081. April 2024.

- https://vcgit.hhi.fraunhofer.de/jvet-ag-ee1/VVCSoftware_VTM/-/tree/EE1-1.3

- D. Rusanovskyy, Y. Li, T. Shao, P. Yin, S. McCarthy, J. N. Shingla, A. Shyam, A. Suneja, S. P. Badya, J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang, R. Chang, L. Wang, X. Xu, S. Liu. AhG11/EE1: Status of the joint EE1-0 (LOP.2) training. JVET-AF0043. October 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).