| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0273-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0075 (EE2-2.1: TIMD merge mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ivan Zupancic | Email: | ivan.zupancic@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of EE2 test 2.1a and 2.1b reported in JVET-AH0075 (EE2-2.1: TIMD merge mode). The method inherits the TIMD modes from the previously TIMD coded blocks. For this, the adjacent and non-adjacent spatial neighboring blocks are scanned and if the scanned block is coded in TIMD or TIMD merge mode, then its TIMD information are added to a TIMD merge list. Up to five TIMD candidates are added into the list. In Test 2.1a, the best candidate from the list is selected to be potentially included in encoder RDO. In Test 2.1b, two best candidates from the list are selected to be potentially included in encoder RDO.

The crosscheck results for BD-rates match the results of tests EE2-2.1a and EE2-2.1b reported in JVET-AH0075. However, the encoding times obtained by the crosschecker is somewhat higher than that of the proponent for both tests EE2-2.1a and EE2-2.1b.

# Crosscheck results

The code of EE2 tests 2.1a and 2.1b was run according to the CTC [1] for AI and RA configuration. The obtained crosscheck results for tests EE2-2.1a and EE2-2.1b under AI and RA configurations are as follows:

EE2-2.1a:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.06% | 0.11% | 102.8% | 101.0% | 100.6% | 100.4% |
| Class A2 | -0.10% | -0.01% | -0.04% | 102.5% | 101.2% | 100.5% | 100.5% |
| Class B | -0.09% | 0.02% | -0.01% | 102.8% | 103.9% | 98.1% | 100.4% |
| Class C | -0.06% | -0.02% | 0.09% | 103.0% | 103.6% | 100.1% | 100.2% |
| Class E | -0.04% | 0.06% | 0.12% | 102.7% | 101.8% | 100.4% | 100.2% |
| Overall | -0.07% | 0.00% | 0.05% | 102.8% | 102.6% | 99.7% | 100.3% |
| Class D | -0.08% | 0.03% | 0.14% | 102.7% | 103.7% | 100.5% | 100.1% |
| Class F | -0.04% | 0.16% | -0.01% | 101.3% | 102.0% | 100.4% | 100.2% |
| Class TGM | 0.00% | 0.02% | -0.14% | 100.8% | 99.7% | 100.3% | 100.5% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.05% | 0.10% | -0.04% | 100.6% | #NUM! | 100.1% | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0.01% | -0.44% | 0.36% | 101.5% | #NUM! | 100.2% | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

EE2-2.1b:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% | -0.06% | 0.15% | 104.5% | 101.6% | 100.7% | 100.5% |
| Class A2 | -0.13% | 0.06% | 0.00% | 104.8% | 103.0% | 100.5% | 100.4% |
| Class B | -0.12% | 0.05% | -0.02% | 105.1% | 103.0% | 99.1% | 100.4% |
| Class C | -0.09% | 0.02% | 0.03% | 104.8% | 103.1% | 100.1% | 100.2% |
| Class E | -0.07% | 0.00% | -0.06% | 104.6% | 101.9% | 100.4% | 100.2% |
| Overall | -0.10% | 0.02% | 0.02% | 104.8% | 102.6% | 100.0% | 100.3% |
| Class D | -0.11% | 0.06% | 0.03% | 104.5% | 106.6% | 100.5% | 100.1% |
| Class F | -0.03% | 0.09% | 0.07% | 101.6% | 101.5% | 100.3% | 100.3% |
| Class TGM | 0.00% | 0.06% | -0.10% | 100.9% | 99.3% | 100.3% | 100.3% |

# Conclusion

Test EE2-2.1a and EE2-2.1b were cross-checked and the partial BD-rate results match with the results provided by the proponent. However, the encoding times obtained by the crosschecker are somewhat higher than that of the proponent. Inspection of the code shows consistency between the proposed and the implemented method.

# References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Jul. 2023.

# Patent rights declaration(s)

Noka does not have any current or pending patent rights relating to the technology described in this contribution.