| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0275-v1 |
| --- | --- |

| Title: | Non-EE2: CCALF with chroma ALF outputs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Marta Karczewicz, Vadim Seregin 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com martak@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-12.0, to filter a chroma sample, a cross-component adaptive loop filter (CCALF) is applied to spatial luma samples and luma residuals. In this proposal, spatial chroma results of adaptive loop filter are used as additional input to a CCALF. The proposed method was implemented on top of ECM-12.0, with common test condition, simulation results are reported as follows.

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra |  |  |  |  |  |  |  |
| Random access |  |  |  |  |  |  |  |
| Low delay B |  |  |  |  |  |  |  |

# Introduction

In ECM-12.0, a chroma sample is filtered by an adaptive loop filter (ALF) and a cross-component adaptive loop filter (CCALF). A chroma ALF is applied to spatial chroma samples. On the other hand, a CCALF is applied to spatial luma samples and spatial luma residuals.

# Proposed method

In this contribution, outputs of chroma ALF are used as additional input to a CCALF. To be specific, upper, left, right and lower chroma ALF results are used.

# Simulation results

The proposed method was implemented on top of ECM-12.0 and tested following common test conditions described in JVET-AF2017.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.01% | -0.23% | -0.17% | 100.3% | 99.7% | 100.0% | 100.0% |
| Class E | 0.05% | -0.57% | -1.85% | 100.3% | 100.1% | 100.0% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.05% | -0.15% | 100.2% | 99.8% | 99.8% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.02% | -0.13% | -0.17% | 100.4% | 99.7% | 100.8% | 99.7% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.03% | -0.37% | -0.17% | 100.0% | 100.2% | 99.9% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.09% | -0.92% | -1.18% | 99.9% | 100.1% | 100.4% | 99.1% |
| Class E | 0.06% | -2.03% | -3.69% | 99.9% | 99.7% | 98.2% | 98.8% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.07% | -0.12% | -0.78% | 99.6% | 100.4% | 100.2% | 99.9% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).