| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0280-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0107 (EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of JVET-AH0107 (EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement), which investigates the neural-network based inter prediction in this round of EE1. Training cross-check were conducted for LDRF and HDRF in this meeting cycle, and simulation results compared with proponents’ results are summrized as follows:

LDRF: 0.00%(Y), -0.10 %(U), -0.08%(V)

HDRF:

Introduction

EE1-3.1 proposes a deep reference frame generation (DRF) method to enhance VVC inter prediction. During the encoding and decoding process, the network receives two reconstructed frames from the decoded picture buffer as inputs and generates a new frame. The generated frame will be put into the reference picture lists as an additional reference for the current encoding picture, which enhances the inter prediction.

Building upon the low operation point deep reference frame generation (LDRF) network from JVET-AG0122, EE1-3.1 proposes a high operation point deep reference frame generation (HDRF) network, in which optical flows are calculated without scaling.

Crosscheck reports

Two training tasks were conducted in this contribution to verify the performance of EE1-3.1. One is training crosscheck of LDRF and another is HDRF. Vimeo dataset is used to train those two models and encoded by NNVC-7.1 with both NnIntra and NnLOP Filter enabled.

Follow the guidance and training scripts provided by proponents, the training process is successfully completed. Then the best checkpoint models in valid-tests were converted to float-sadl and doing inference tests.

Simulation results are shown as follows:

LDRF

Table. 1 retrained LDRF vs. NNVC-8.0

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.68% | -2.38% | -2.42% | -2.12% | -3.83% | -3.20% | 114% |  |
| Class A2 | -2.34% | -2.64% | -1.04% | -2.86% | -3.80% | -2.87% | 116% |  |
| Class B | -1.32% | -1.00% | -1.23% | -1.98% | -2.53% | -3.14% | 88% |  |
| Class C | -1.58% | -1.24% | -0.96% | -2.88% | -3.08% | -2.92% | 101% |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.66% | -1.67% | -1.36% | -2.42% | -3.19% | -3.04% | 102% |  |
| Class D | -1.16% | 0.16% | 0.71% | -2.28% | -2.07% | -1.74% | 90% |  |

*Please note that the running time is not reliable.

Table. 2 retrained LDRF vs. JVET-AH0107’s LDRF

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-AH0107 LDRF |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.04% | -0.39% | -0.10% | 0.02% | -1.49% | -0.05% | 91% |  |
| Class A2 | 0.01% | -0.10% | -0.04% | 0.02% | -0.50% | 0.01% | 95% |  |
| Class B | 0.00% | -0.01% | -0.13% | 0.05% | -0.21% | -0.30% | 78% |  |
| Class C | -0.04% | 0.01% | -0.04% | -0.03% | -0.13% | -0.08% | 96% |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.10% | -0.08% | 0.02% | -0.50% | -0.13% | 88% |  |
| Class D | -0.11% | 0.02% | -0.06% | -0.14% | -0.19% | -0.13% | 82% |  |

*Please note that the running time is not reliable.

HDRF

Simulation is still running, this would be updated later.

Reference

[1] W. Bao, et al. EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AH0107, Rennes, FR, April 2024