| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-0290-v1 |
| --- | --- |

| Title: | AHG9: real-time demonstration of MPI (V3C) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Remy Gendrot Franck Galpin Celine Guede Bertrand Chupeau Bertrand Leroy Gaelle Martin-Cocher | Email: | Remy.Gendrot@InterDigital.com Franck.Galpin@InterDigital.com Celine.Guede@InterDigital.com Bertrand.Chupeau@InterDigital.com Bertrand.Leroy@InterDigital.com Gaelle.Martin-Cocher@InterDigital.com |
| Source: | Interdigital Inc. |  |  |

_____________________________

# Abstract

This is an information document following the conference call on preparations to evaluate quality in the context of MPI (JVET-AH0045).

The Multi-Planar Image format (MPI) of the MIV format is showcased on several contents. Real-time demonstrations of the format for immersive experience using conventional video codec are proposed on several devices (smartphone, tablet, head-mounted displays, laptop PC). The demonstration shows that the format can be implemented on a wide range of devices, support a wide range of contents with challenging input contents in terms of depth or camera motion.

# Introduction

The document presents results and a real-time implementation of a platform jointly developed by InterDigital and Philips that showcases MPI [1] content leveraging the MPEG volumetric standard (MPEG-I V3C [2]).

MPEG’s Visual Volumetric Video-based Coding (V3C) standard is an extensive framework for the coding of volumetric video, from dynamic point clouds (V-PCC [3]) to multi-view plus depth and multi-plane image representations (MIV). It offers a single bitstream structure with a uniform bridge to systems-level standards. The common aspects including bitstream structure have been regrouped into the Visual volumetric video-based coding (MPEG-I V3C) standard [2]. To enable storage and delivery of such compressed volumetric content, MPEG has developed Carriage of V3C data [4].

# MPI Format

The multiplane image format (MPI) is a layered representation of a 3D scene viewed from a reference view (see Figure 1). Constructing an MPI from an MVD is done by un-projecting the pixels from the original cameras into 3D space and re-projecting them back onto the layers of a chosen reference camera. In the demonstrations, the location of the reference view has been chosen at the center of the camera rig and the internal resolution of the MPI similar to the one of the input sources in order to preserve the details of the scene.

Figure 1 - MPI layers and pixels

The MPI input format (mpi.pcs, see contribution [5]) is composed of pairs of raw video files and a JSON configuration file. The first video file contains the color information of the MPI and the second video file contains the transparency information (see Figure 2). Both video files contain a temporal concatenation of the stacked layers of the MPI. Each color layer is stored as a YUV 420 10 bits frame, and each transparency layer is stored as a YUV 420 8 bits frame. The Json file describes cameras parameters (extrinsics/intrinsics).

Figure 2 - InterdigiKart video files (color on left, transparency on right)

The TMIV encoder (TMIV v14.0 see [6]) generates 2 atlas (one for texture and one for transparency) and a file of metadata. Both video atlases are encoded in HEVC using FFMPEG v5.1.3 and then multiplexed with the metadata according to the V3C standard to generate a conform V3C bitstream.

The MPI MIV encoding is suitable for real-time rendering as it involves only two frames decoding and can be rendered directly using classical 3D engine. A demonstration on embedded devices like smartphones, tablets, head-mounted displays (e.g. Meta Quest3) is shown using the V3Cimmersive platform described in section 4.

# Information Data

Six input content are considered in this section (see Figure 3). Two of them are computer generated content (Mannequin and InterdigiKart) and the last fourth are natural content (BaskBall, MaskFull, Barn and Breakfast).

Figure 3 - Content screenshots (from left top to bottom right: Mannequin, BasketBall, InterdigiKart, MaskFull, Barn, Breakfast)

Some information related to the MPI generation process is related in Table 1:

- Number of source cameras

- Scaling factor: scaling factor applied on the final resolution, allowing to put more content inside a single atlas.

- MPI viewport resolution

- Number of depth layers generated: depends on the content and the quality of the input depth.

The MPI viewport resolution is related to the resolution of the source images, the complexity of the captured scene and the number of cameras in the RIG.

Table 1 - Demo Content Information

| Content Name | Source spatial resolution (HxW) | Source nb Views | scaling factor | Nb Layer Range | MPI Viewport Resolution | Atlas Resolution | QP [color; transparency] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mannequin | 1920*1080 | 15 | 1 | >100 | 2584x1496 | 4096x4096 | [20;18] |
| BasketBall | 2048x1088 | 3 | 1 | >200 | 2752x1288 | 4096x4096 | [32;40] |
| InterdigiKart | 1366x768 | 15 | 1 | >100 | 1768x1024 | 4096x4096 | [20;40] |
| MaskFull | 1920x1080 | 15 | 0.7 | >100 | 2224x1080 | 4096x4096 | [20;40] |
| Barn | 1920x1080 | 15 | 0.6 | >50 | 1504x840 | 4096x4096 | [32;44] |
| Breakfast | 1920x1080 | 15 | 0.6 | >100 | 1656x960 | 4096x4096 | [32;44] |

# Demo Description

The MPI volumetric video can be replayed on different devices (smartphone, tablet, head-mounted display) using the V3C immersive platform.

Figure 4 shows a global architecture diagram of this platform that is fully described in the MPEG-I contribution in the WG 07 MPEG 3D Graphics Coding and Haptics Coding subgroup of this meeting (see [7]).

Figure 4 - Immersive Video Decoder Platform

# Conclusion

This document describes a common Interdigital/Philips demonstration that showcase MPI capabilities of decoding in real time on multiple embedded devices in AR or VR. The demonstration shows that the format can support a wide range of contents with challenging input contents in terms of depth or camera motion.

The demo is available for viewing during the JVET and MPEG week in Rennes 2024.

# References

[1]	ISO/IEC 23090-12: MPEG immersive video

[2]	ISO/IEC 23090-5, Visual Volumetric Video-based Coding (V3C)

[3]	ISO/IEC 23090-5 Annex H: Video-based Point Cloud Compression (V-PCC)

[4]	ISO/IEC 23090-10 Carriage of V3C data

[5]	ISO/IEC JTC 1/SC 29/ WG 04 m55709 - Proposal for an optimized input format for MPI, January 2021, Online

[6] ISO/IEC JTC 1/SC 29/WG 4, N00242 – Test Model 14 for MPEG immersive video, July 2022, Online

[7]	ISO/IEC JTC 1/SC 29/WG 7 m67332 - [V3C][Demo] V3C and Haptic Immersive Video Demo, April 2024, Rennes

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).