| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0301-v1 |
| --- | --- |

| Title: | AHG4/AHG13: input for subjective quality testing of the FGC SEI message |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange M. Wien | Email: | philippe.delagrange@interdigital.com wien@lfb.rwth-aachen.de |
| Source: | InterDigital, RWTH |  |  |  |

_____________________________

# Abstract

This contribution provides input to refine the test plan for subjective quality testing of the FGC SEI message (currently JVET-AG2022). Candidate test sequences and test condition refinements are discussed.

# Introduction

Subjective quality testing of the FGC SEI message is currently delayed pending more test material, and the assessment of the effect of film grain synthesis at high quality has been separated from the first category into a new one, with test conditions to be refined.

This document is intended to report on the activity to refine the test plan, prior and during the 24th JVET meeting in Rennes.

# Candidate test sequences

## Scanned film

Two sequences of the 2004 SVT “FairyTale” test set (OldTownCross and CrowdRun), shot on 65mm film, are already part of the candidate sequences and have been used for expert viewing during the 30th JVET meeting in Antalya [1]. Since the SVT test set is a 6-minutes movie, more or different clips could be considered; this is under investigation. The outcome of the test in Antalya was positive, but it was questioned whether similar benefits would be observed using older film stock, that was anticipated to exhibit significantly different characteristics, likely coarser and stronger grain.

Older film material has then been actively sought after, and several sources have been found (see e.g. JVET-AG0228 about Prelinger Archives). This can match two use cases:

- Archive services, which collect professional or semi-professional material (e.g. promotional movies, or just old things) that can be of excellent quality, and home movies. Since this material is supposed to be mostly open for free viewing and have a tremendous interest for historical and cultural purposes, they face significant challenges giving access to their large database (tens of thousands of movies) with a decent quality: at least for previews, the grain is most often broken in a bad way by encoding which significantly alters the visual experience. And when a good quality is available, the files are too big to be downloaded and usable by the general public.	
Those organizations also offer material for commercial use in current productions (see next bullet).

- Current productions making use of small parts of old material, for illustration purpose (e.g. in reportings/stories), or as part of storytelling, such as the Meridian test movie starting with 30 seconds of old film, or Shawn Levy using 2 minutes of the 1945 “l’empreinte d’Attila” from Emile Gaudu in the ending part of the last episode of “all the light we cannot see” (released November 2, 2023 on NetFlix) [3].

Rick Prelinger, founder of Prelinger archives, has been contacted about the material he contributed to archive.org. He says the material is fine to use and even provided three selected clips.

Cinémathèque de Bretagne, an other archive service, has provided two sequences, and will likely provide more in the future.

The following material is currently under consideration, some of which may be reviewed during the meeting:

- 1906 (black & white) “A Trip Down Market Street just before the fire” from by Miles Brothers, scanned and made available by Prelinger Archives. Two 10s selections are available, including encoding at different bitrates for the first one.

- 1946 (color) “American Maker”, and outtakes from “American Choice”, both by Jam Handy (Prelinger Archives)

- “Brest 1917” (black and white, 1917 to 1919) and “Actualités Pont à Mousson” (color, 1967) provided by Cinémathèque de Bretagne

- First 30s of Meridian

The material from Prelinger Archive and Cinémathèque de Bretagne was scanned using a LaserGraphics ScanStation scanner. The 2004 SVT “FairyTale” film was scanned using a FilmLight NorthLight scanner. Both are high-end professional movie scanners.

Potential material under investigation are: Stem 1 (ASC, 2015), more of FairyTale (SVT, 2004), more archives

## Digital productions, with either camera noise or artistic grain

Sequences under consideration are:

- DinnerScene (but with a different starting point to avoid the scene cut)

- Meridian1 (policemen speaking, with one smoking)

- Meridian2 (old car approaching on a desert road between mountains)

- Original TearsOfSteel scenes (under investigation)

Some of those may be reviewed during the meeting, if material can be made ready for viewing.

The Meridian scenes apparently have artistic grain added on purpose, and would be of particular relevance for this activity.

## Experimental additive grain

See JVET-AH0299 about denoised and regrained sequences from “Tears of steel”. Some sequences can be reviewed during the meeting.

# Test conditions refinement

The assessment of effect of film grain synthesis at high quality is now a separate test category in JVET-AG2022 (it was previously included in the first category, now dedicated to the effect on a wider range of quality, that could be called “artefact hiding”). The idea is to compare native grain encoding with grain synthesis (both in terms of bitrate and grain aspect), on a high quality range.

Below are suggested refinements of test conditions for this category:

- The test sequences must contain meaningful grain (while the previous test category might consider adding grain to clean sequences to hide coding defects)

- The DCR test method is replaced with A/B comparison using 5 grades

- Test points and encoding considerations:

  - Two or three rates are currently considered in either 4.2.1 or 4.2.3 of JVET-AG2022. The additional one in the latter section considers an intermediate point where film grain is not fully removed by encoding, to assess effectiveness of grain synthesis to harmonize the grain or at least improve fidelity to original. This can probably be discussed while experimenting.

  - On some sequences with extreme characteristics (e.g. MarketStreet) it may not be possible to reach a QP with both grain removed and good video quality (third test point in 4.2.3 of JVET-AG2022); it that case, the intermediate point could be a good option.

# Liaison statement from 3GPP

A liaison statement has been received from 3GPP-SA4 (MPEG document m67968), reporting on the feasibility study on film grain synthesis in 3GPP SA4 and asking if we can consider testing the effect of film grain synthesis on top of HEVC. Currently we are only considering VVC.

# Conclusion

[Ed note: discussions on the refinement on test plan to be reported here in v2]

# References

- P. de Lagrange, W. Husak, M. Wien, “Draft plan for subjective quality testing of FGC SEI message”, JVET-AG2022, teleconference, January 2023

- M. Wien, “Results of expert viewing for testing of FGC SEI message”, JVET-AD0382, Antalya, TR, April 2023

- https://www.cinematheque-bretagne.bzh/toutes-les-actus-netflix-toute-la-lumi%C3%A8re-que-nous-ne-pouvons-voir-all-the-light-we-cannot-see-542-1831-0-1.html

- https://en.wikipedia.org/wiki/A_Trip_Down_Market_Street

- https://archive.org/details/MarketStreet19064KScan20181016

- https://archive.org/details/200723_American_Maker

- https://archive.org/details/200643_American_Choice_Outtakes_R3

- https://www.cinematheque-bretagne.bzh/base-documentaire-brest-1917-426-12940-0-1.html (item [12940])

- https://www.cinematheque-bretagne.bzh/base-documentaire-actualit%C3%A9s-pont-%C3%A0-mousson-426-1522-0-1.html (item [1522])