| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0307-v1 |
| --- | --- |

| Title: | AHG12: Multiple Transform Set Selection for Intra MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Charles Bonnineau Karam Naser Saurabh Puri Fabrice Le Léannec | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to apply Multiple Transform Set Selection (MTSS) to explicit intra MTS. Specifically, the planar transform set of the explicit intra MTS lookup-table is considered in addition to the initial transform set for several non-directional intra modes.

The following results are obtained compared to ECM-12.0:

AI: x.xx% x.xx% x.xx% 10x.x% 10x.x%

# Introduction

Multiple transform set selection (MTS) has been introduced in VVC, where DST7 and DCT8 transform pairs were tested in addition to the DCT2 primary transform. For intra directional mode, residual samples have larger absolute values if they are far away from the reference samples. Thus, the usage of a transform with an increasing basis function along the intra prediction direction might better catch the residual statistics.

To exploit this intra mode dependency in the ECM, it has been proposed to assign a transform set to each intra mode using a lookup table. Planar, DC and MIP intra modes have their own transform set in the lookup-table. Each transform set contains a total of six pairs of separable horizontal and vertical transforms. The number of candidates in each transform set is determined between 1, 4 or 6 by the absolute sum of coefficients in the block.

Some non-directional modes use a virtual intra prediction mode (VIPM) to obtain a transform set in this lookup-table. This VIPM is typically computed from the Histogram of Gradients (HoG), as in the directional intra mode derivation (DIMD) mode. However, some of these modes (e.g., intraTMP, EIP) may not have a clear residual directionality compared to angular intra modes.

Recently, Multiple Transform Set Selection (MTSS) [1] has been proposed in JVET. This mode offers the possibility to select an alternative transform set based on the HoG of the neighboring pixels of the current block.

It is asserted that for explicit intra MTS, the planar primary transform pairs can be adapted as alternative transform set to some intra modes.

# Proposed Solution

In this contribution, it is proposed to use an alternative transform set in explicit intra MTS for DIMD, TIMD, intraTMP and EIP modes. For these modes, the initial transform set remains unchanged, and the alternative transform set corresponds to the planar transform set in the lookup-table.

The usage of the alternative transform set is signaled by a CU-level flag to the decoder. If the alternative transform set is selected by the encoder, the flag is set to 1.

# Experimental results

The proposed method is implemented on top of ECM-12.0 [2]. The simulations were performed following the CTCs [3] for the all-intra (AI) configuration.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.03% | 0.18% | 0.19% |  |  |
| Class C | -0.02% | -0.01% | 0.21% |  |  |
| Class E | -0.02% | 0.15% | 0.02% |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.05% | 0.24% | 0.09% |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class TGM | #VALUE! | #VALUE! | #VALUE! |  |  |

# Conclusion

We recommend further study of the proposed method in EE2.

# References

- F. Wang, et. al., “Non-EE2: Multiple Transform Set Selection for LFNST/NSPT” JVET-AG0062, January 2024

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).