| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0314-v1 |
| --- | --- |

| Title: | EE2-3.5d: Combination of EE2-3.4 and EE2-3.5c |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) Lei Zhao2, Kai Zhang, Li Zhang (Bytedance) | Email: | chunchic@qti.qualcomm.com zhaolei.0324@bytedance.com |
| Source: | Qualcomm Incorporated and Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of the combination of EE2 Test 3.4 and Test 3.5c. On top of ECM-12.0, the reported performance of the proposed combination is summarized as follows:

-- RA (Y/U/V): -0.05% / -0.03% / -0.01%, EncT 100.1%, DecT 100.0%.

-- LDB (Y/U/V): -0.12% / 0.01% / 0.08%, EncT 100.2%, DecT 100.2%.

# Introduction

In EE2 Test-3.4 (JVET-AH0073), an adaptive GPM blending method with two alternative blending width candidate lists is investigated. In particular, list #0 ({τ/4, τ/2, τ, 2τ, 4τ}) comprises the same blending candidates as in ECM-12.0, and list #1 ({τ/2, τ, 2τ, 4τ, 8τ}) is designed for larger blocks, where at most 16 samples are blended on each side of the split boundary. The selection of the candidate list is adaptively determined based on the block size. If min(width, height) < 32, list #0 is used. Otherwise list #1 is used.

In EE2 Test-3.5c (JVET-AH0186), LIC model parameters from causally neighboring blocks could be inherited conditionally, when they have LIC flags determined to be true, in addition to motion information during the construction process of GPM merge candidate list. Accordingly, LIC could be enabled for GPM without the model parameter derivation in motion compensation. This test also extends the notion of LIC model inheritance to regular merge modes. The new merge candidates (that could inherit LIC model from causal neighborhood) are competing jointly with all other merge candidates in the ARMC stage.

This proposal provides results of the direct combination of Test 3.4 and Test 3.5c.

# Simulation result

The proposed method was implemented on top of the ECM-12.0 software [1] and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AG2024). The testing results are summarized below.

- Performance results relative to ECM-12.0 anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Enc Vm | Dec Vm |
| Class A1 | -0.12% | -0.20% | -0.04% | 100.0% | 100.4% | 99.9% | 100.0% |
| Class A2 | 0.05% | 0.11% | 0.14% | 100.4% | 100.2% | 100.0% | 100.1% |
| Class B | -0.08% | 0.05% | -0.07% | 99.9% | 99.9% | 100.5% | 100.1% |
| Class C | -0.02% | -0.09% | -0.01% | 100.2% | 99.8% | 100.0% | 100.0% |
| Class E | x | x | x | x | x | x | x |
| Overall | -0.05% | -0.03% | -0.01% | 100.1% | 100.0% | 100.2% | 100.0% |
| Class D | 0.03% | 0.19% | 0.02% | 100.1% | 100.3% | 100.1% | 100.0% |
| Class F | -0.05% | -0.02% | -0.10% | 100.2% | 100.1% | 100.7% | 99.9% |
| Class TGM | -0.07% | -0.11% | -0.15% | 99.9% | 99.8% | 99.4% | 100.7% |

|  | Low Delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Enc Vm | Dec Vm |
| Class A1 | x | x | x | x | x | x | x |
| Class A2 | x | x | x | x | x | x | x |
| Class B | -0.16% | 0.02% | 0.06% | 100.3% | 100.2% | 100.0% | 99.7% |
| Class C | -0.08% | 0.00% | 0.15% | 100.1% | 100.0% | 99.6% | 99.6% |
| Class E | -0.10% | 0.00% | 0.02% | 100.3% | 100.4% | 102.0% | 99.9% |
| Overall | -0.12% | 0.01% | 0.08% | 100.2% | 100.2% | 100.3% | 99.7% |
| Class D | -0.12% | 0.50% | -0.07% | 100.1% | 100.3% | 100.3% | 100.1% |
| Class F | 0.04% | -0.40% | -0.61% | 100.6% | 99.6% | 97.5% | 99.8% |
| Class TGM |  |  |  |  |  |  |  |

# References

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).