| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0315-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0145 (AHG12: Follow up on normative DMVR fix for ECM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jani Lainema | Tel: Email: | - first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports a cross-verification of techniques described in JVET-AH0145 (AHG12: Follow up on normative DMVR fix for ECM). The software provided by the proponents of JVET- AH0145 was studied, and it was found to match the description given in the contribution.

Simulations were performed according to the JVET Common Test Conditions using RA configuration. All the BD-rate results matched with the ones reported in JVET- AH0145. Also, the runtime and peak memory measurements were close to the ones reported in the studied contribution (within a typical margin considering differences in computing platforms and computing load variations). Full results are provided in the attached spreadsheets.

# Summary of the results

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03 % | 0.06 % | -0.03 % | 99.9% | 99.8% | 99.9% | 99.8% |
| Class A2 | -0.01 % | 0.05 % | 0.02 % | 99.6% | 99.9% | 100.0% | 100.2% |
| Class B | -0.01 % | 0.01 % | 0.03 % | 100.4% | 99.9% | 99.7% | 99.9% |
| Class C | 0.03 % | -0.02 % | 0.00 % | 100.3% | 100.3% | 100.2% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00 % | 0.02 % | 0.01 % | 100.1% | 100.0% | 99.9% | 100.0% |
| Class D | 0.01 % | 0.00 % | 0.08 % | 100.4% | 99.9% | 100.2% | 100.0% |
| Class F | -0.01 % | 0.02 % | 0.04 % | 100.5% | 101.1% | 99.3% | 100.1% |
| ClassTGM | 0.01 % | 0.01 % | 0.00 % | 101.1% | 101.6% | 99.8% | 99.6% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).