| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0318_v0 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0061 (Non-EE2: CCALF with Chroma SAO Input) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Wenbin Yin Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | yinwenbin.hit@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This contribution reports the crosscheck results of JVET-AH0061[1] (Non-EE2: CCALF with Chroma SAO Input). The proposed method in JVET-AH0061 uses Chroma SAO’s outputs as the input of several additional taps in CCALF. There are 4 new taps in a 3x3 diamond shape are introduced into the CCALF filters.

The available simulation results match with those provided in JVET-AH0061.

Simulation Results

The proposed method in JVET-AH0061 was implemented on top of the ECM-12.0 [2]. Simulations were performed following the common test conditions [3].

More detailed results could be found in the attached excel sheets. The results compared to ECM-12.0 are shown in Table 1.

Table 1 Performance of the Proposed Method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.01% | -0.24% | -0.17% | 99.6% | 99.8% | 98.6% | 100.3% |
| Class E | 0.04% | -0.43% | -1.56% | 99.9% | 101.3% | 100.2% | 100.4% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.18% | -0.19% | 100.0% | 99.9% | 100.0% | 100.1% |
| Class F |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.03% | -0.22% | -0.17% | 99.6% | 100.0% | 100.1% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.02% | -0.41% | -0.24% | 99.6% | 99.4% | 100.0% | 100.1% |
| Class F |  |  |  |  |  |  |  |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.08% | 0.36% | -0.62% | 99.5% | 100.8% | 100.1% | 100.1% |
| Class F |  |  |  |  |  |  |  |

Reference

- Z. Dai, N. Song, Y. Yu, H. Yu, D. Wang (OPPO), “Non-EE2: CCALF with Chroma SAO Input”, Document of Joint Video Experts Team, JVET-AH0061, Rennes, FR, April 2024.

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, DE, October 2023.