| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0324-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0114 (AHG 16: Updated Common Software Tools for Generative Face Video Compression) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peng Yin (Dolby) | Email: | pyin@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of the newly supported GFVC algorithms proposed in JVET-AH0114 (AHG16: Updated Common Software Tools for Generative Face Video Compression).

In v1, it shows that for Deep Animation Codec (DAC), the averaged BDRate of crosscheck result compared with the proposal is within 0.02% for all the metrics under GFVC test condition. It is therefore confirmed that the proposal passed the crosscheck.

In v2, Hybrid Deep Animation Codec (HDAC) and its extension (HDAC_Extension) is crosschecked. Compared to DAC, the deviation is a little bigger, especially for PSNR related metrics.  This is mainly due to the difference GPU server and test environment.

# Introduction

JVET-AH0114 proposed two updates for the AHG16 GFVC v2 software.

- New GFVC algorithms are incorporated into the AHG16 software: Deep Animation Codec (DAC), Hybrid Deep Animation Codec (HDAC) and HDAC extension.

- GFVC software code are well improved: including codebase readability and function format uniformity, removing CUDA dependency, and so on. The updated software package is available at https://github.com/xyzysz/GFVC_Software_v2/tree/master.

The crosscheck for DAC is performed following the instruction in the contribution and the software repository. The test is done uing A100 GPU server with cuda support.

# Crosscheck results

Table 1 shows the BDRate of the crosscheck result for DAC compared against the proposal in JVET-AH0114. More detailed results can refer to the accompanying excel sheet.

Table 2 and Table 3 shows the BDRate of the crosscheck result for HDAC and HDAC_Extension compared against the proposal in JVET-AH0114. The average BDRate difference seems bigger than expected, especially for RGB444_PSNR and YUV420_PSNR. In the accompanying excel sheet, we showed absolute difference between crosschecked results and JVET-AH0114 results. The maximum difference is 0.3dB.

Table 1. Test result over JVET-AH0114 for DAC

|  | DAC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over GFVC_AH0114 |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | 0.00% | 0.00% | 0.02% | -0.01% | 0.01% | 0.00% |
| Class B | 0.01% | 0.01% | 0.00% | 0.01% | 0.01% | 0.01% |
| Overall | 0.01% | 0.00% | 0.01% | 0.00% | 0.01% | 0.01% |

Table 2. Test result over JVET-AH0114 for HDAC

|  | HDAC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over GFVC_AH0114 |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -0.31% | 0.07% | 3.63% | 0.33% | 3.47% | 0.20% |
| Class B | 0.22% | 0.50% | 2.17% | 0.08% | 2.13% | 0.03% |
| Overall | -0.04% | 0.29% | 2.90% | 0.20% | 2.80% | 0.11% |

Table 3. Test result over JVET-AH0114 for HDAC_Extension

|  | HDAC_Extension |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over GFVC_AH0114 |  |  |  |  |  |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -0.89% | 0.06% | 1.75% | 0.78% | 5.66% | 0.41% |
| Class B | -0.29% | 0.55% | 4.26% | 0.85% | 1.84% | 0.82% |
| Overall | -0.59% | 0.30% | 3.00% | 0.82% | 3.75% | 0.62% |