| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0325-v2 |
| --- | --- |

| Title: | AHG9: On Text Description SEI (JVET-AH0052) and Text Comment SEI (JVET-AH0172) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a Text Descriptions SEI message, which combines features of the Text Description SEI proposed in JVET-AH0052 and the Text Comment SEI proposed in JVET-AH0172, with additional modifications. Three additional text description purposes are proposed to carry an encoder version, an encoder configuration, and a tag URI for unique identification of the bitstream. Two options are proposed for modified persistence syntax and semantics.

The v2 version of this document provides specification text to include the proposed persistence syntax in an annex of the TuC in section 4.

# Proposal

Two contributions at this meeting propose an SEI message to carry text, the Text Description SEI in JVET-AH0052 and the Text Comment SEI in JVET-AH0172.  This contribution proposes an alternate design for carrying text descriptions in an SEI that incorporates some of the features proposed in those contributions, with additional modifications.

Features of the proposed design are summarized as follows:

- Define additional purposes for the text description, as follows:

  - Encoder version:

    - Identifies the encoder name and version used to generate the bitstream, such as VTM10.0 or VVenC v1.11

  - Encoder configuration:

    - Identifies the encoder configuration, such as RA, LDP, LDB, AI

  - Tag URI:

    - Uniquely identifies the bitstream

- Signalling an ID to identify the Text descriptions SEI message

- Signalling of multiple text descriptions within the same SEI message, each with a signalled purpose indicator

- Optional language signalling for the SEI message

  - Additional languages are supported through use of an additional SEI message per language

- Two options to enable separate and overlapping persistence

- Option 1: Use typical persistence and cancel flags, with the text descriptions ID syntax element located before the cancel flag

- Option 2: Replace the persistence and cancel flags with a 2-bit persistence indicator syntax element with 4 possible values:

  - 0 indicates cancel (similar to cancel flag equal 1)

  - 1 indicates does not persist (similar to persistence flag equal 0)

  - 2 indicates persistence until cancelled (similar to persistence flag equal 1)

  - 3 indicates persistence for the entire CLVS. The SEI message contents may be repeated in the CVLS for that text descriptions ID value, but not be modified.

## Additional purposes

Identification within the bitstream of the encoder version and encoder configuration used to generate the bitstream is asserted to be useful for implementers of both encoder and decoder products to resolve issues with interoperability.

Strings representing the encoder version might identify, for example, “VTM20.0” or “VVenc v1.11”.

Strings representing the encoder configuration might identify, for example, “RA”, “AI”, “LDP”, “LDB”.

Such a feature is asserted to have been useful during development of the conformance test suite for VVC and earlier standards.

A tag URI provides a unique identifier that can be used to uniquely identify a bitstream.  As defined in IETF RFC 4151, a tag URI includes an authority name and a time/date stamp and optionally contains additional specification. Having such information associated with a bitstream could be useful for a variety of purposes.

These purposes are added to the table provided in JVET-AH0052.

Table xx – Definition of td_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General label information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Encoder version |
| 6 | Encoder configuration |
| 7 | Tag URI for identifying the encoded content |
| 8-255 | Reserved |

## Persistence options syntax

Separate and overlapping persistence of the text descriptions is supported with two syntax options. Option 2 is preferred.

The syntax options are based on JVET-AH0052 and JVET-AH0172, with modifications to support the features listed above.

### Option 1

Option 1 uses a typical persistence flag and cancel flag. The td_id syntax element is located above the cancel flag so that cancellation and persistence apply only to that value of td_id.

| text_descriptions( payloadSize ) { | Descriptor |
| --- | --- |
| td_id | u(6) |
| td_cancel_flag | u(1) |
| if( !td_cancel_flag ) { |  |
| td_persistence_flag | u(1) |
| td_num_descriptions_minus1 | u(7) |
| td_language_present_flag | u(1) |
| if ( td_language_present_flag ) |  |
| td_language | st(v) |
| for( i = 0; i <= td_num_descriptions_minus1; i++ ) { |  |
| td_purpose_idc[ i ] | u(8) |
| td_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |

### Option 2 (Preferred option)

Option 2 replaces persistence and cancel flags with a persistence indicator syntax element with 4 possible values.

- 0 indicates cancel (similar to cancel flag equal 1)

- 1 indicates does not persist (similar to persistence flag equal 0)

- 2 indicates persistence until cancelled or CLVS ends (similar to persistence flag equal 1)

- 3 indicates persistence for the entire CLVS. The SEI message contents may be repeated in the CVLS for that td_id value but are not modified.

The first 3 of the 4 values correspond to states that can be accomplished using typical persistence and cancellation flags. The 4th value represents a new state that provides additional information not available using only a persistence and a cancel flag.

Decoders may find it useful to be aware that the parameters signalled in the first Text Description SEI message with a particular td_id value in the CLVS applies to all pictures in the entire CLVS, without being required to check if any future Text Description SEI messages in the same CLVS cancel the persistence or contain different contents.

A semantic constraint on the allowable values of td_persistence_idc is proposed for the newly proposed values of td_purpose_id[ i ] - Encoder version, Encoder configuration, and Tag URI - as those purposes apply to the entire CLVS rather than to individual pictures.

The proposed syntax is below, with modifications from Option 1 highlighted.

| text_descriptions( payloadSize ) { | Descriptor |
| --- | --- |
| td_id | u(6) |
| td_cancel_flag | u(1) |
| if( !td_cancel_flag ) { |  |
| td_persistence_flag | u(1) |
| td_persistence_idc | u(2) |
| if( td_persistence_idc > 0 ) { |  |
| td_num_descriptions_minus1 | u(7) |
| td_language_present_flag | u(1) |
| if ( td_language_present_flag ) |  |
| td_language | st(v) |
| for( i = 0; i <= td_num_descriptions_minus1; i++ ) { |  |
| td_purpose_idc[ i ] | u(8) |
| td_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |

## Comparison with JVET-AH0052 and JVET-AH0172

JVET-AH0052 proposes signalling a purpose of the signaled text with the txt_descr_role syntax element. This contribution also proposes that functionality but renames the syntax element to td_purpose_idc.

JVET-AH0172 proposes optional signaling of a language once in the SEI message, which is also included in this contribution. JVET-AH0172 proposes the use of ISO 639-2 to indicate the language, while this contribution proposes the use of RFC 5646, as is used in the Annotated regions SEI.

JVET-AH0052 proposes signalling of multiple text strings for a given purpose, each with a signalled language. That design enables, for example, signalling of a particular type of information in more than one language within the same SEI message. This contribution instead proposes signalling multiple purposes, with a single string per purpose, and proposes optional signaling of a single language for the SEI message. In this contribution, multiple languages are still supported, but they will be carried in separate SEI messages.  This allows broadcasters to select the appropriate SEI message for their local language.

JVET-AH0172 proposes to enable separate and overlapping persistence of text comments. It is asserted that there is a problem with the persistence syntax in JVET-AH0172 so that its design does not adequately provide the desired functionality. The text_commend_id syntax element is not signaled when the cancel flag is equal to 1, so it is not possible to cancel the persistence of individual text comments.  This contribution also provides the functionality of separate and overlapping persistence of text comments, but with different persistence syntax and semantics from JVET-AH0172.

# Semantics

The semantics are based on JVET-AH0052 and JVET-AH0172, modified according to the proposed syntax in this contribution.

## Option 1

Text descriptions information SEI message semantics

The text descriptions information SEI message provides one or more text description about one or more pictures.

td_id indicates the identifier value of this text descriptions information SEI message.

td_cancel_flag equal to 1 indicates that the text descriptions information SEI message cancels the persistence of any previous text description information SEI message in output order that applies to the current layer and has the value of td_id equal to that of the current text descriptions information SEI message. td_cancel_flag equal to 0 indicates that text description information follows.

NOTE: A text descriptions information SEI message with td_cancel_flag equal to 1 can be used, for example, when the source content changes within a CLVS and hence none of the previous text descriptions in output order are applicable. A text description information SEI message with td_cancel_flag equal to 1 is used to cancel the persistence of the text descriptions with a particular value of td_id.

td_persistence_flag specifies the persistence of the text information descriptions SEI message for the current layer.

td_persistence_flag equal to 0 specifies that the text descriptions information applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text descriptions information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same value of td_id is output that follows the current picture in output order.

td_num_descriptions_minus1 plus 1 indicates the number of entries for td_purpose_id[ i ] and td_string[ i ] that follow.

td_language_present_flag equal to 1 indictes that the td_language syntax element is present. td_language_present_flag equal to 0 indictes that the td_language syntax element is not present.

td_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00 indicating the language of the td_string[ i ] syntax elements. The length of the td_language syntax element shall be less than or equal to 63 bytes, not including the null termination byte. When not present, the language of the td_string[ i ] syntax elements is unspecified.

td_purpose_idc[ i ] indicates the purpose of the td_string[ i ] text description string, as specified in Table xx.

Values of td_purpose_idc[ i ] that are identified as reserved for future use in Rec. ITU-T H.274 | ISO/IEC 23002-7 shall not be present in bitstreams conforming to this version of this Specification.

Table xx – Definition of td_purpose_idc

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General label information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Encoder version |
| 6 | Encoder configuration |
| 7 | Tag URI for identifying the encoded content |
| 8 - 127 | Reserved |
| 128 - 255 | Unspecified |

td_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the td_purpose_idc[ i ]. The length of td_string[ i ]  shall be in the range of 0 to 8191, inclusive.

When td_purpose_idc[ i ] is equal to 0, the interpretation of what information is conveyed in td_string[ i ] is application-defined.

When td_purpose_idc[ i ] is equal to 1, td_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope defined by td_cancel_flag and td_persistence_flag.

When td_purpose_idc[ i ] is equal to 2, td_string[ i ] specifies Artificial Intelligence (AI) marking information that pertains to the picture(s) in the persistence scope defined by td_cancel_flag and td_persistence_flag.

When td_purpose_idc[ i ] is equal to 3, td_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope defined by td_cancel_flag and td_persistence_flag.

When td_purpose_idc[ i ] is equal to 4, td_string[ i ] specifies content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) [3] that pertains to the picture(s) in the persistence scope defined by td_cancel_flag and td_persistence_flag.

When td_purpose_idc[ i ] is equal to 5, td_string[ i ] specifies a description of the version of the encoding system used to generate the CLVS.

When td_purpose_idc[ i ] is equal to 6, td_string[ i ] specifies a description of the configuration of the encoding system used to generate the CLVS.

When td_purpose_idc[ i ] is equal to 7, td_string[ i ] contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

When td_purpose_idc[ i ] is identified as unspecified in Table xx, td_string[ i ] should be interpreted as unknown or unspecified or specified by external means not specified in this Specification.

### Option 2

The semantic modifications from Option 1 are highlighted.

td_persistence_idc equal to 0 indicates that the text descriptions information SEI message cancels the persistence of any previous text description information SEI message in output order that applies to the current layer and has the value of td_id equal to that of the current text descriptions information SEI message.

td_persistence_idc equal to 1 specifies that the text descriptions information SEI message applies to the current decoded picture only.

txt_persistence_idc equal to 2 specifies that the text descriptions information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same value of td_id is output that follows the current picture in output order.

td_persistence_idc equal to 3 specifies that the text descriptions information applies to the entire CLVS.

It is a requirement of bitstream conformance that when td_persistence_idc is equal to 3 and this SEI message is not the first text descriptions SEI message in a CLVS, in decoding order, a text descriptions SEI message with the same value of td_id shall be present in the first PU of the CLVS in decoding order

All text description SEI messages in a CLVS that have a particular td_id value and td_persistence_idc equal to 3 shall have identical SEI payload content .

td_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the td_purpose_idc[ i ]. The length of td_string[ i ]  shall be in the range of 0 to 8191, inclusive.

When td_purpose_idc[ i ] is equal to 0, the interpretation of what information is conveyed in td_string[ i ] is application-defined.

When td_purpose_idc[ i ] is equal to 1, td_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope defined by td_cancel_flag and td_persistence_flag.

When td_purpose_idc[ i ] is equal to 2, td_string[ i ] specifies Artificial Intelligence (AI) marking information that pertains to the picture(s) in the persistence scope defined by td_cancel_flag and td_persistence_flag.

When td_purpose_idc[ i ] is equal to 3, td_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope defined by td_cancel_flag and td_persistence_flag.

When td_purpose_idc[ i ] is equal to 4, td_string[ i ] specifies content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) [3] that pertains to the picture(s) in the persistence scope defined by td_cancel_flag and td_persistence_flag.

When td_purpose_idc[ i ] is equal to 5, td_string[ i ] specifies a description of the version of the encoding system used to generate the CLVS.

When td_purpose_idc[ i ] is equal to 6, td_string[ i ] specifies a description of the configuration of the encoding system used to generate the CLVS.

When td_purpose_idc[ i ] is equal to 7, td_string[ i ] contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

When td_purpose_idc[ i ] is equal to 5, 6, or 7, it is a requirement of bitstream conformance that the value of td_persistence_idc shall be equal to 3.

# Recommendation

It is recommended that the proposed Text Descriptions SEI message be added to the TuC.

It is further recommended that the VTM and ECM reference software encoders be updated to provide a configuration option to signal a Text Descriptions SEI message containing the Encoder Version and Encoder Configuration purposes.

# Specification text for TuC persistence syntax

# Specification text is provided for inclusion of the proposed persistence syntax in the TuC. Two options are provided, depending on whether or not the SEI message includes an ID.

## SEI messages without an ID

| example( payloadSize ) { | Descriptor |
| --- | --- |
| ex_persistence_idc | u(2) |
| if( ex_persistence_idc > 0 ) { |  |
| /* additional syntax elements */ |  |
| } |  |
| } |  |

ex_persistence_idc specifies the persistence of the example SEI message for the current layer.

ex_persistence_idc equal to 0 indicates that the example SEI message cancels the persistence of any previous example SEI message in output order that applies to the current layer of the current example SEI message.

ex_persistence_idc equal to 1 specifies that the example SEI message applies to the current decoded picture only.

ex_persistence_idc equal to 2 specifies that the example SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an example SEI message that follows the current picture in output order.

ex_persistence_idc equal to 3 specifies that the example SEI message applies for the entire CLVS.

It is a requirement of bitstream conformance that when ex_persistence_idc is equal to 3 and this SEI message is not the first example SEI message in a CLVS, in decoding order, an example SEI message shall be present in the first PU of the CLVS in decoding order.

When td_persistence_idc is equal to 3 in the first PU in the CLVS in decoding order, all text description SEI messages in the CLVS shall have identical SEI payload content.

## SEI messages with an ID

| example( payloadSize ) { | Descriptor |
| --- | --- |
| ex_id | u(6) |
| ex_persistence_idc | u(2) |
| if( ex_persistence_idc > 0 ) { |  |
| /* additional syntax elements */ |  |
| } |  |
| } |  |

ex_persistence_idc specifies the persistence of the example SEI message for the current layer.

ex_persistence_idc equal to 0 indicates that the example SEI message cancels the persistence of any previous text description information SEI message in output order that applies to the current layer and has the value of ex_id equal to that of the current example SEI message.

ex_persistence_idc equal to 1 specifies that the example SEI message applies to the current decoded picture only.

txt_persistence_idc equal to 2 specifies that the example SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an example SEI message with the same value of ex_id is output that follows the current picture in output order.

ex_persistence_idc equal to 3 specifies that the example SEI message applies for the entire CLVS.

It is a requirement of bitstream conformance that when ex_persistence_idc is equal to 3 and this SEI message is not the first text descriptions SEI message in a CLVS, in decoding order, an example SEI message with the same value of ex_id shall be present in the first PU of the CLVS in decoding order.

All text description SEI messages in a CLVS that have a particular ex_id value and ex_persistence_idc equal to 3 shall have identical SEI payload content .

When ex_persistence_idc is equal to 3 in the first PU in the CLVS in decoding order, all text description SEI messages in the CLVS that have a particular ex_id value shall have identical SEI payload content.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).