| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0334-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0195 (EE1-related: complexity reduction of NN in-loop filters through early cropping) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | T. Dumas | Email: | thierry.dumas@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution summarizes the crosscheck results of JVET-AH0195 (EE1-related: complexity reduction of NN in-loop filters through early cropping). It is reported that by encoding the JVET CTC sequences via NNVC-8.0 and decoding the resulting bitstreams via the modified version of NNVC-8.0 in JVET-AH0195, no decoding mismatch is observed (AI, RA, and LDB).

# Introduction

In JVET-AH0195, in the last convolutional layers of a Neural Network (NN)-based in loop filter, the patch size is reduced gradually to avoid unnecessary computation. This way, the output patch returned by this NN-based in-loop filter can have the desired size, and subsequent cropping is not needed.

For instance, in LOP-2 [1], see Figure 1, from point (f) to point (i) included, the paddings in each of the three convolutions are ignored. Thus, the input to the convolutional layer at point (f) is 36x36 and the output of the convolutional layer at point (i) is 32x32. Then, following the pixel shuffling, no subsequent cropping is needed.

Figure 1: solution in JVET-AH0195 for reducing the patch size by using convolutions without padding, applied to LOP-2.

# Cross-check

The JVET CTC sequences [2] have been encoded via NNVC-8.0, then the resulting bitstreams have been decoded via the modified version of NNVC-8.0 in JVET-AH0195. No decoding mismatch has been observed, neither in AI nor in RA nor in LDB.

# Reference

- JVET-AG2019, “Description of algorithms and software in neural network-based video coding (NNVC) version 6”, F. Galpin, Y. Li, D. Rasanovskyy, J. Ström, L. Wang. 33rd Meeting, by teleconference, 17–26 January 2024.

- JVET-AF2016, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, E. Alshina, R. L. Liao, S. Liu, A. Segall, 32nd Meeting, Hannover, DE, 13–20 October 2023.