| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, by Rennes, FR, 17–24 April 2024 | Document: JVET-AH0343-v2 |
| --- | --- |

| Title: | AHG9: Proposed Specification Text for Text Description Information SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jill Boyce Miska M. Hannuksela P. de Lagrange D. Doyen E. François F. Urban C. Salmon-Legagneur Stephan Wenger Arianne Hinds Gilles Teniou | Tel: Email: | sdeshpande@sharplabs.com {jill.boyce, miska.hannuksela}@nokia.com philippe.delagrange@interdigital.com {swenger,ahinds,teniou}@global.tencent.com |
| Source: | Sharp Labs of America, Inc., Nokia, InterDigital, Tencent |  |  |

_____________________________

# Abstract

This document proposes specification text for a Text Description Information SEI message based on agreements in HLS session. In the updated version all the changes agreed during presentation are included.

# Introduction

The Technologies under Consideration for future extension of VSEI [1] document includes a copyright SEI and an AI marking SEI. Both these SEIs currently include a text string as the main syntax element which provides the corresponding copyright or AI marking information. Additionally, JVET-AG0184 proposed to embed text comments in a video bitstream via a new SEI.

It is asserted that each of these SEI message relates to signaling a text string for different roles or purposes. It is asserted that a common SEI message which provides text information for various purposes avoids the need to define multiple SEI messages which provide specific type of text information, while providing future extensibility.

In this proposal a text description information SEI message is proposed for this.

The main aspects of the proposed SEI message include the following:

- A purpose syntax element provides information about how to interpret the text information string

- An identifier for the text description SEI message

- A syntax element which specifies how many text strings are included in the message. Multiple strings can be included in the same message. For example, for a given purpose, different text strings may be signalled for different languages.

- A for loop which includes:

  - A text string syntax element which provides the text which is interpreted based on the purpose. For example, when the purpose indicates copyright information, the text string is the copyright text.

  - A syntax element which provides the RFC 5646 language tag specifying the language of the text string.

- Cancel and Persistence flag (similar in spirit to other SEIs)

# Proposed Specification Text

The proposed syntax and semantics for the text description information SEI message are as follows.

### Text description information SEI message syntax

| text_description( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_id | u(14) |
| txt_cancel_flag | u(1) |
| if( !txt_cancel_flag ) { |  |
| txt_persistence_flag | u(1) |
| txt_descr_purpose | u(8) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |

### Text description information SEI message semantics

The text description information SEI message provides text description about one or more pictures.

txt_descr_id indicates the identifier value of this text description information SEI message. The value of txt_descr_id shall be in the range of 1 to 16383, inclusive. Value 0 is reserved.

txt_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_id in output order that applies to the current layer. txt_cancel_flag equal to 0 indicates that text description information follows.

txt_persistence_flag specifies the persistence of the text information description SEI message for the current layer.

txt_persistence_flag equal to 0 specifies that the text description information applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text description information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same txt_descr_id is output that follows the current picture in output order.

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table xx.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6-255 | Reserved |

txt_num_strings_minus1 plus 1 indicates the number of entries for txt_descr_string_lang[ i ] and txt_descr_string[ i ] that follow.

txt_descr_string_lang[ i ] specifies the language of the txt_descr_string[ i ]. The language of the txt_descr_string[ i ]  shall be given by a language tag as defined by IETF RFC 5646 [2]. The length of txt_descr_string_lang[ i ]  shall be in the range of 0 to 49, inclusive.

txt_descr_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the txt_descr_purpose.

When txt_descr_purpose is equal to 0, the interpretation of what information is conveyed in the txt_descr_string is application-defined.

When txt_descr_purpose is equal to 1, the txt_descr_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 2, the txt_descr_string[ i ] specifies. when not a null string, AI marking information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 2 the string can contain information about the machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

When txt_descr_purpose is equal to 3, the txt_descr_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 4, the txt_descr_string[ i ] specifies content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) [3] that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 5, txt_descr_string[ i ] contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

# Conclusion

This document proposes a text description information SEI. It is proposed to adopt the SEI message into the VSEI version 4.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AG2032, M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy, Technologies under consideration for future extensions of VSEI, March 2023

- IETF: RFC 5646, “Tags for Identifying Languages,” Internet Engineering Task Force, Reston, VA, September 2009. https://datatracker.ietf.org/doc/html/rfc5646

- CTA, U.S. and Canadian Region Rating Tables (RRT) and Content Advisory Descriptors for Transport of Content Advisory Information Using ATSC Program and System Information Protocol (PSIP), Doc. CTA-766-D (ANSI), Consumer Technology Association, Arlington, VA, December 11, 2013