| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0345-v1 |
| --- | --- |

| Title: | AHG9: Updated text for AI marking SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger | Email: | swenger@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

During the discussion of JVET-AH0177, certain changes were requested to the text of the AI marking SEI message as currently present in the TuC.  The author collaborated with experts who had specific comments.  The result is shown below.  No non-editorial syntax changes are proposed, and the changes in the semantics are primarily in style.  As per the meeting agreement as of the time of writing, it is expected that this text will either be included in the VSEI working draft in unchanged form, or the text be merged with the proposed Text information SEI message in JVET-AH0343.

# Proposed Text

Below the proposed text.  The markup is against the AI marking SEI as present in the TuC document (JVET-AG2032).

8.36 Artificial intelligence marking SEI message

8.36.1 Artificial intelligence marking SEI message syntax

| ai_marking( payloadSize ) { | Descriptor |
| --- | --- |
| ai_marking_cancel_flag | u(1) |
| if ( !ai_marking_cancel_flag ) { |  |
| ai_marking_persistence_flag | u(1) |
| ai_marking_string | st(v) |
| } |  |
| } |  |

8.36.1 Artificial intelligence marking SEI message semantics

The artificial intellingence marking (AIM) SEI message indicates that some machine-learning-based processing was involved in the generation of the source content of the pictures associated with the SEI message. Further information about the machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures can be indicated in the content of an associated text string.

ai_marking_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous artificial intelligence (AI) marking SEI message in output order that applies to the current layer. ai_marking_cancel_flag equal to 0 indicates that AI marking information follows.

ai_marking_persistence_flag specifies the persistence of the AI marking SEI message for the current layer.

ai_marking_persistence_flag equal to 0 specifies that the AI marking information applies to the current decoded picture only.

ai_marking_persistence_flag equal to 1 specifies that the AI marking information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an AI marking SEI message is output that follows the current picture in output order.

ai_marking_string, when not a null string, provides AI marking information that pertains to the picture(s) within the persistence scope of the AIM SEI message.

NOTE: The ai_marking_string can contain information about the machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).