| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0347-v1 |
| --- | --- |

| Title: | EE plan for Multiple Transform Set Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang, Yue Yu, Haoping Yu, Dong Wang (OPPO) Lei Zhao, Kai Zhang, Li Zhang (Bytedance) Muhammed Coban, Marta Karczewicz (Qualcomm) Charles Bonnineau, Saurabh Puri, Karam Naser, Fabrice Le Léannec (InterDigital) | Tel: Email: | wangfan6@oppo.com zhaolei.0324@bytedance.com mcoban@qti.qualcomm.com charles.bonnineau@interdigital.com |
| Source: | OPPO, Bytedance Inc., Qualcomm Inc, InterDigital |  |  |

_____________________________

- Abstract

This contribution describes the EE plan for Multiple Transform Set Selection.

Introduction

The multiple transform set selection (MTSS) method for intra blocks coded with LFNST/NSPT was proposed in JVET-AG0062 [1]. The proposed method was further investigated in EE2-4.1. EE2-4.1 is reported in JVET-AH0056. The proposed MTSS method allows CUs coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP to select one LFNST/NSPT transform set out of 2 candidate sets. When the current block is coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP in combination with LFNST/NSPT, one more bin is employed to indicate whether the 1st or the 2nd candidate transform set is selected.

The 1st candidate transform set remains the same as the current ECM. The derivation of the 2nd candidate is as follows: for TIMD, if fusion is applied, the 2nd TIMD IPM is first considered; for SGPM, the 2 IPMs that SGPM uses are first considered; for MIP and EIP, PLANAR is first considered. The 1st and 2nd candidate should be different transform sets. If the 2nd candidate is not yet decided by the above process, it is derived by the HoG of neighboring reconstructed pixels.

In EE2-4.1, to reduce the encoding time, a CU size restriction is used for MTSS, where the CU size is measured with number of pixels within the CU. With the restriction, MTSS can be applied only to CUs with size equal or larger than 256.

In JVET-AH0218 and JVET-AH0237, it is proposed to use the 2nd DIMD IPM derived based on the prediction signal of the current block to derive the 2nd candidate for MIP, EIP and ITMP.

In JVET-AH0184, it is proposed to decrease the encoding time of this tool by using a reduced set of LFNST/NSPT kernels for the alternative transform set candidates of MTSS. Specifically, a single kernel is retained for each alternative transform set and its index is computed based on the value of the Virtual Intra Prediction Mode (VIPM) of the current block, using VIPM % 3.

The derivation methods (D0, D1, and D2) for the alternative(s) transform set(s) are summarized below:

|  | D0 (EE2-4.1) | D1 (AH0218, AH0237) | D2 (AH0184) |
| --- | --- | --- | --- |
| SGPM | sgpmMode0/sgpmMode1, IPM derived by HoG of prediction (AH0237) |  |  |
| TIMD | 2nd TIMD IPM if fusion |  |  |
| DIMD | IPM derived by HoG of neighboring pixels |  |  |
| MIP | PLANAR | IPM derived by HoG of prediction | PLANAR & IPM derived by HoG* |
| EIP | PLANAR | IPM derived by HoG of prediction | PLANAR & IPM derived by HoG* |
| ITMP | IPM derived by HoG of neighboring pixels | IPM derived by HoG of prediction | PLANAR & IPM derived by HoG* |

*Two alternative transform sets are selected for MIP, EIP and ITMP.

The methods (R0, R1) to reduce encoding running time are summarized below:

| R0 (EE2-4.1) | restrict MTSS by CU size |
| --- | --- |
| R1 (AH0184) | reduce MTSS transform kernels |

The methods (K0, K1) to select the kernel in the alternative(s) transform set(s) is as follows:

| K0 (EE2-4.1, AH0218, AH0237) | Same as current ECM |
| --- | --- |
| K1 (AH0184) | Derive using VIPM % 3 |

Description of planed EE tests

The planed tests are summarized below:

| test | description | Transform set derivation methods | Complexity reduction method | Kernel selection in the transform set | Number of additional kernels |
| --- | --- | --- | --- | --- | --- |
| Test_a | Illustrate the highest gain, no complexity reduction | D0, D1, combination | / | K0 | 3 |
| Test_b | Reduce complexity with only R0 | D0, D1, combination | R0 | K0 | 3 |
| Test_c | Reduce complexity with only R1 with performance improvement from D1 | D1, D2 | R1 | K1 | 2 (MIP, EIP, ITMP) or 1 |
| Test_d | Reduce complexity only with R1 | D1 | R1 | K1 | 1 |
| Test_e | Reduce complexity with R0 and R1 | D1 | R0, R1 | K1 | 1 |

Combination results of D1, D2, K1, R1:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ANCHOR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.06% | 0.07% | 102.9% | 99.6% |
| Class A2 | -0.06% | 0.02% | 0.02% | 103.2% | 99.0% |
| Class B | -0.07% | 0.00% | 0.16% | 102.6% | 100.3% |
| Class C | -0.04% | -0.01% | 0.04% | 101.2% | 98.4% |
| Class E | -0.07% | 0.01% | -0.05% | 102.3% | 97.5% |
| Overall | -0.07% | -0.01% | 0.06% | 102.4% | 99.1% |
| Class D | -0.06% | 0.09% | -0.08% | 100.9% | 96.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 100.9% | 98.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |