| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0349-v1 |
| --- | --- |

| Title: | AHG9/AHG16: On semantics fix for GFV SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Yan Ye Alibaba Kaifa Yang1, Ye-Kui Wang2 Yiling Xu1, Yue Li1 1Shanghai Jiao Tong University, China 2Bytedance Inc., US | Email: | jiechen.cj@alibaba-inc.com yan.ye@aliabba-inc.com sekiroyyy@sjtu.edu.cn yekui.wang@bytedance.com yl.xu@sjtu.edu.cn yueli983@sjtu.edu.cn |
| Source: | Alibaba, Shanghai Jiao Tong University, Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposed to fix the semantics of GFV SEI message as following:

gfv_coordinate_x_abs[ i ] indicates the normalized absolute value of is used to derive the x-axis coordinate of the i-th keypoint.

gfv_coordinate_y_abs[ i ] specifies the normalized absolute value of is used to derive the y-axis coordinate of i-th keypoint.

gfv_coordinate_z_abs[ i ] specifies the normalized absolute value of is used to derive the z-axis coordinate of the i-th keypoint.

gfv_coordinate_dx_abs[ i ] indicates the absolute difference value of the normalized value of is used to derive the x-axis coordinate difference of the i-th keypoint.

gfv_coordinate_dy_abs[ i ] specifies the absolute difference value of the normalized is used to derive the  y-axis coordinate difference of the i-th keypoint.

gfv_coordinate_dz_abs[ i ] specifies the absolute difference value of the normalized is used to derive the z-axis coordinate difference of the i-th keypoint.

Introduction

During the first round of contribution revewing, issues reported in JVET-AH0148[1] were not clearly understood by the experts, and meanwhile some semantics issues in the current GFV SEI message was pointed out. Thus, a side activity was conducted after the meeting dicussion to improve the GFV semantics and check if there are any non-editorial problems to be addressed.

Proposal

After the offline discussion, it is clarified that the issues originally reported in JVET-AH0148 are not relevant, and the following fixes of the semantics are suggested to avoid ambiguity of "normalized"

gfv_coordinate_x_abs[ i ] indicates the normalized absolute value of is used to derive the x-axis coordinate of the i-th keypoint.

gfv_coordinate_y_abs[ i ] specifies the normalized absolute value of is used to derive the y-axis coordinate of i-th keypoint.

gfv_coordinate_z_abs[ i ] specifies the normalized absolute value of is used to derive the z-axis coordinate of the i-th keypoint.

gfv_coordinate_dx_abs[ i ] indicates the absolute difference value of the normalized value of is used to derive the x-axis coordinate difference of the i-th keypoint.

gfv_coordinate_dy_abs[ i ] specifies the absolute difference value of the normalized is used to derive the  y-axis coordinate difference of the i-th keypoint.

gfv_coordinate_dz_abs[ i ] specifies the absolute difference value of the normalized is used to derive the z-axis coordinate difference of the i-th keypoint.

References

- K. Yang, Y.-K. Wang, Y. Xu, Y. Li, “AHG9/AHG16: On key point coordinates calculation for the generative face video SEI message” JVET-AH0148, Rennes, Jan.2024

Patent rights declaration(s)

Alibaba may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Shanghai Jiao Tong University and Bytedance Inc. do not have any current or pending patent rights relating to the proposed technology described in this contribution.