| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AF0353-v0 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0180 (AHG12: Modified CTC for encoding time reduction under low-delay configuration) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Waqas Ahmad | Email: | waqas.ahmad@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

The contribution reports the results of cross-check of the JVET-AH0180 proposed by InterDigital. This contribution proposed to update the configuration settings for LDB, ClassB, QP22 as follows:

Two tests are performed in the contribution:

- First test sets the maximum depth from 3 to 2 for temporal depth higher or equal to 1 (Test 1)

- The second is to, on top of the first change, also set the maximum depth from 2 to 1 for temporal depth higher or equal to 3.

Each test offers different trade-off between coding loss and runtime reduction. This crosscheck reports the results for Test1 and Test2.

# Verification

The changes in configuration file for LDB is provided by InterDigital and simulations were performed on top of ECM-12.0.

The request for crosscheck was made during the JVET meeting and the LDB, ClassB QP22 sequences takes around 12-15 days to complete. Hence, based on partial results the conclusion is drawn.

The RD performance is matched for both Test 1 and Test2 with the provided logfile.

It is observed that the trade-off between coding loss and EncT reduction for the Test 1 is more attractive compared to the Test2.

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.