| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH2006-v1 |
| --- | --- |

| Title: | Additional SEI messages for VSEI version 4 (Draft 2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Jill Boyce Jie Chen Sachin Deshpande Miska M. Hannuksela Hendry Sean McCarthy Gary J. Sullivan Ye-Kui Wang | Email: | jill.boyce@nokia.com jiechen.cj@alibaba-inc.com sdeshpande@sharplabs.com miska.hannuksela@nokia.com dr.hendry@lge.com sean.mccarthy@dolby.com gary.sullivan@dolby.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify additional SEI messages, including encoder optimization information, source picture timing information , object mask information, modality information, and text description information SEI messages and updates to the neural-network post-filter characteristics SEI message.

# Changes yet to be integrated:

# Changes that have been integrated:

In this document, relative to JVET-AG2034:

Changes incorporate JVET-AH0108, JVET-AH0111, JVET-AH0174, JVET-AH0175, JVET-AH0343, JVET-AH0346

- Encoder optimization information SEI modifications (JVET-AH0108, JVET-AH0111)

- Text description SEI (JVET-AH0343)

- Object mask information SEI modifications (JVET-AH0346)

- NNPFC SEI message modifications (JVET-AH0174)

- Film grain characteristics SEI message constraints (JVET-AH0175 item 1)

In JVET-AG2034:

- Changes incorporate: JVET-AG0070, JVET-AG0081, JVET-AG-0082v2, JVET-AG0086, JVET-AG0089, JVET-AG0148, JVET-AG0188, JVET-AG0322NNPFC SEI message: Temporal extrapolation purpose (JVET-AG0089).

- Source picture timing information SEI message: Include with the following modifications

  - various aspects from JVET-AG0070

    - (Proposal 1)_Editorial clarification of NOTE 1

    - (Proposal 2) Forbid the value 0 for spti_num_units_in_elemental_interval (option 2)

    - (Proposal 3)Modify semantic to fix bug in handling the case spti_source_type_present_flag equal 0

  - option 1 of JVET-AG0188

  - Modification from JVET-AG0082v2

- Encoder optimization information SEI message:

  - 2-bit indicators eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc (JVET-AG0086, item 1)

  - new optimization type from JVET-AG0081

- Object mask information SEI message: Include with modifications from JVET-AG0148

- Modality information SEI message:

- NNPFC SEI message application information signaling

- payloadType values for the added SEI message (VVC text). [Ed. (MH): The use of prefix SEI NAL unit was enabled for all new SEI messages, whereas the use of suffix SEI NAL unit was enabled for encoder optimization information and object mask information SEI messages, since their content may be determined while the encoder encodes the associated picture.]

# Changes to the specification text:

# Modify subclause 2.3 as follows:

Recommendation ITU-T T.35 (in force), Procedure for the allocation of ITU-T defined codes for non

standard facilities.

– IETF RFC 1321 (in force), The MD5 Message-Digest Algorithm.

– IETF RFC 4151 (in force), The 'tag' URI Scheme.

– IETF RFC 5646 (in force), Tags for Identifying Languages.

– IETF Standard 66 (in force), Uniform Resource Identifiers (URI): Generic Syntax.

– ISO/CIE 11664-1 (in force), Colorimetry – Part 1: CIE standard colorimetric observers.

– ISO/IEC 10646 (in force), Information technology – Universal coded character set (UCS).

– ISO/IEC 11578:1996, Information technology – Open Systems Interconnection – Remote Procedure Call

(RPC).

– ISO/IEC 15938-17 (in force), Information technology – Multimedia content description interface – Part 17:

Compression of neural networks for multimedia content description and analysis

– Doc. CTA-766-D (ANSI), CTA, U.S. and Canadian Region Rating Tables (RRT) and Content Advisory Descriptors for Transport of Content Advisory Information Using ATSC Program and System Information Protocol (PSIP), Consumer Technology Association, Arlington, VA, December 11, 2013

– ISO 20473:2007, Optics and photonics – Spectral bands.

# In clause 8.1, replace Table 4 with the following:

| Table 4 – Persistence scope of SEI messages (informative) |  |
| --- | --- |
| SEI message | Persistence scope |
| Filler payload | The PU containing the SEI message |
| User data registered by Rec. ITU-T T.35 | Unspecified |
| User data unregistered | Unspecified |
| Film grain characteristics | Specified by the syntax of the SEI message |
| Frame packing arrangement | Specified by the syntax of the SEI message |
| Parameter sets inclusion indication | The CLVS containing the SEI message |
| Decoded picture hash | The PU containing the SEI message |
| Mastering display colour volume | The CLVS containing the SEI message |
| Content light level information | The CLVS containing the SEI message |
| DRAP indication | The picture associated with the SEI message |
| Alternative transfer characteristics | The CLVS containing the SEI message |
| Ambient viewing environment | The CLVS containing the SEI message |
| Content colour volume | Specified by the syntax of the SEI message |
| Equirectangular projection | Specified by the syntax of the SEI message |
| Generalized cubemap projection | Specified by the syntax of the SEI message |
| Sphere rotation | Specified by the syntax of the SEI message |
| Region-wise packing | Specified by the syntax of the SEI message |
| Omnidirectional viewport | Specified by the syntax of the SEI message |
| Frame-field information | The PU containing the SEI message |
| Sample aspect ratio information | Specified by the syntax of the SEI message |
| Annotated regions | Specified by the syntax of the SEI message |
| Scalability dimension information | The CVS containing the SEI message |
| Multiview acquisition information | The CVS containing the SEI message |
| Multiview view position | The CVS containing the SEI message |
| Depth representation information | Specified by the semantics of the SEI message |
| Alpha channel information | Specified by the syntax of the SEI message |
| Extended DRAP indication | The picture associated with the SEI message |
| Display orientation | Specified by the syntax of the SEI message |
| Colour transform information | Specified by the syntax of the SEI message |
| Shutter interval information | The CLVS containing the SEI message |
| Neural-network post-filter characteristics | The CLVS containing the SEI message |
| Neural-network post-filter activation | Specified by the syntax of the SEI message |
| Phase indication | Specified by the semantics of the SEI message |
| Encoder optimization information | Specified by the syntax of the SEI message |
| Source picture timing information | Specified by the syntax of the SEI message |
| Object mask information | Specified by the syntax of the SEI message |
| Modality information | Specified by the syntax of the SEI message |
| Text descriptions | Specified by the syntax of the SEI message |

# Modify subclause 8.5.2 as follows:

fg_matrix_coeffs has the same semantics as specified in clause 7.3 for the vui_matrix_coeffs syntax element, except as follows:

–	fg_matrix_coeffs specifies the matrix coefficients of the film grain characteristics specified in the SEI message, rather than the matrix coefficients used for the CLVS.

–	When fg_matrix_coeffs is not present in the film grain characteristics SEI message, the value of fg_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

–	The values allowed for fg_matrix_coeffs are not constrained by the chroma format of the decoded video pictures that is indicated by the value of ChromaFormatIdc for the semantics of the VUI parameters.

fg_matrix_coeffs shall not be equal to 0 unless fg_bit_depth_luma_minus8 is equal to fg_bit_depth_chroma_minus8.

fg_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	fg_bit_depth_luma_minus8 is equal to fg_bit_depth_chroma_minus8.

–	fg_bit_depth_chroma_minus8 is equal to fg_bit_depth_luma_minus8 + 1.

# Modify subclause 8.19.2 as follows:

sdi_aux_id[ i ] equal to 0 indicates that the i-th layer in the current CVS does not contain auxiliary pictures. sdi_aux_id[ i ] greater than 0 indicates the type of auxiliary pictures in the i-th layer in the current CVS as specified in Table 15. When sdi_auxiliary_info_flag is equal to 0, the value of sdi_aux_id[ i ] is inferred to be equal to 0.

Table 15 – Mapping of sdi_aux_id[ i ] to the type of auxiliary pictures

| sdi_aux_id[ i ] | Name | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3 | AUX_OBJECT_MASK | Object mask picture |
| 4..127 |  | Reserved |
| 128..159 |  | Unspecified |
| 160..255 |  | Reserved |

NOTE 1 – The interpretation of auxiliary pictures associated with sdi_aux_id[ i ] in the range of 128 to 159, inclusive, is specified through means other than the sdi_aux_id[ i ] value.

sdi_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, for bitstreams conforming to this version of this Specification. Although the value of sdi_aux_id[ i ] shall be in the range of 0 to 3, inclusive, or 128 to 159, inclusive, in this version of this Specification, decoders shall also allow other values of sdi_aux_id[ i ] in the range of 0 to 255, inclusive.

If sdi_aux_id[ i ] is equal to 0, the i-th layer is referred to as a primary layer. Otherwise, the i-th layer is referred to as an auxiliary layer. When sdi_aux_id[ i ] is equal to 1, the i-th layer is also referred to as an alpha auxiliary layer. When sdi_aux_id[ i ] is equal to 2, the i-th layer is also referred to as a depth auxiliary layer. When sdi_aux_id[ i ] is equal to 3, the i-th layer is also referred to as an object mask auxiliary layer.

# Modify subclause 8.28.1 as follows:

### 8.28.1	General post-processing filtering process using NNPFs

#### 8.28.1.1	General

Input to this process is a bitstream BitstreamToFilter. Output of this process is a list of NNPF output pictures ListNnpfOutputPics.

First, BitstreamToFilter is decoded, and the list CroppedDecodedPictures is set to be the list of the cropped decoded pictures in output order resulted from decoding BitstreamToFilter.

Second, the filtering process for one picture, as specified in clause 8.28.1.2, is repeatedly invoked, in output order, for each cropped decoded picture that is in CroppedDecodedPictures and for which one or more NNPFs are activated.

The order of the pictures in ListNnpfOutputPics is in output order.

Within ListNnpfOutputPics there shall be no more than one picture pertaining to any particular output time instance. When for any particular picture in CroppedDecodedPictures there are multiple NNPFs activated and only one of the NNPFs is allowed to be chosen to be applied although any of the NNPFs may be chosen, the above constraint shall apply regardless of which NNPF is chosen to be applied to the particular picture.

For any particular pair of pictures inputPicA and inputPicB consecutive in output order in CroppedDecodedPictures, when there are one or more pictures intermediatePicSetA in ListNnpfOutputPics between inputPicA and inputPicB in output order, one and only one of the following shall apply:

–	The pictures in intermediatePicSetA shall be among the pictures that were output by applying a particular NNPF nnpfA with PictureRateUpsamplingFlag equal to 1 when a particular picture currPicA in CroppedDecodedPictures was the current picture.

–	The pictures in intermediatePicSetA shall be among the pictures that were output by applying a particular NNPF nnpfA with TemporalExtrapolationFlag equal to 1 when a particular picture currPicA in CroppedDecodedPictures was the current picture.

The application of any other NNPF that was used in the filtering process for one picture when currPicA was the current picture or the application of any NNPF (including nnpfA) that was used in the filtering process for one picture when any other picture currPicB in CroppedDecodedPictures was the current picture shall not output any picture between the inputPicA and inputPicB in output order.

NOTE – The intent of the constraints expressed in the above paragraph is to disallow generating NNPF output pictures between any particular pair of consecutive input pictures more than once.

#### 8.28.1.2	Filtering process for one picture using an NNPF

The filtering process specified in this clause applies to each cropped decoded picture, referred to as the current picture, that is in CroppedDecodedPictures and for which one or more NNPFs are activated.

When applying an NNPF to the current picture, the following applies:

–	The filtered and/or interpolated pictures are generated by the NNPF by applying the NNPF process specified in the semantics of the NNPFC SEI message, in a patch-wise manner, to the current picture.

–	The order of the pictures generated by the NNPF by applying the NNPF process being stored into the output tensor of the NNPF is in output order.

When the applied NNPF is the last NNPF that is applied to the current picture, the pictures generated by the NNPF and output by the NNPF process are included into ListNnpfOutputPics, in the same order as when the pictures are stored into the output tensor of the NNPF.

# Modify clause 8.28.2 as follows:

    - Neural-network post-filter characteristics SEI message

      - Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) { |  |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_filtering_flag[ i ] | u(1) |
| nnpfc_absent_input_pic_zero_flag | u(1) |
| } |  |
| if( ChromaUpsamplingFlag ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ColourizationFlag ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ResolutionResamplingFlag ) { |  |
| nnpfc_pic_width_num_minus1 | ue(v) |
| nnpfc_pic_width_denom_minus1 | ue(v) |
| nnpfc_pic_height_num_minus1 | ue(v) |
| nnpfc_pic_height_denom_minus1 | ue(v) |
| } |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | ue(v) |
| nnpfc_spatial_extrapolation_right_offset | ue(v) |
| nnpfc_spatial_extrapolation_top_offset | ue(v) |
| nnpfc_spatial_extrapolation_bottom_offset | ue(v) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_inp_order_idc > 0 ) |  |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| if( nnpfc_out_order_idc != 1 ) |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_out_order_idc != 0 ) |  |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_matrix_coeffs | u(8) |
| nnpfc_full_range_flag | u(1) |
| } |  |
| } |  |
| if( nnpfc_out_order_idc > 0 ) |  |
| nnpfc_chroma_loc_info_present_flag | u(1) |
| if( nnpfc_chroma_loc_info_present_flag ) |  |
| nnpfc_chroma_sample_loc_type_frame | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_luma_padding_val | ue(v) |
| if( nnpfc_inp_order_idc != 0 ) { |  |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) { |  |
| if( nnpfc_purpose = = 0 ) { |  |
| nnpfc_application_purpose_tag_uri_present_flag | u(1) |
| if( nnpfc_application_purpose_tag_uri_present_flag ) |  |
| nnpfc_application_purpose_tag_uri | st(v) |
| } |  |
| if( SpatialExtrapolationFlag ) |  |
| nnpfc_scan_type_idc | u(2) |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

      - Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified neural-network post-processing filters (NNPFs) for specific pictures is indicated with neural-network post-filter activation (NNPFA) SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the NNPF.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value array StrengthControlVal[ idx ] that shall contain real numbers in the range of 0 to 1, inclusive, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive.

Input picture with index 0 corresponds to the picture for which the NNPF defined by this NNPFC SEI message is activated by an NNPFA SEI message. Input picture with index i in the range of 1 to numInputPics − 1, inclusive, precedes the input picture with index i − 1 in output order.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

NOTE 1 – More than one NNPFC SEI message can be present for the same picture. When more than one NNPFC SEI message with different values of nnpfc_id is present or activated for the same picture, they can have the same value or different values of nnpfc_purpose and the same value or different values of nnpfc_mode_idc.

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20, where ( nnpfc_purpose & bitMask ) not equal to 0 indicates that the NNPF has the purpose associated with the bitMask value in Table 20. When nnpfc_purpose is greater than 0 and ( nnpfc_purpose & bitMask ) is equal to 0, the purpose associated with the bitMask value is not applicable to the NNPF. When nnpfc_pupose is equal to 0, the NNPF may be used as determined by the application and as specified by the nnpfc_application_purpose_tag_uri.

All NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose.

The value of nnpfc_purpose shall be in the range of 0 to 255, inclusive, in bitstreams conforming to this version of this Specification. Values of 256 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 256 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Temporal extrapolation (i.e., generating one or more future pictures) |
| 0x80 | Spatial extrapolation (i.e., generating content outside of the spatial area of the input pictures) |

The variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, ColourizationFlag, and TemporalExtrapolationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, colourization, and temporal extrapolation, respectively, are derived as follows:

ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0
SpatialExtrapolationFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : 0

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value or any one of a set of values including that value.

When ChromaFormatIdc is equal to 3, ChromaUpsamplingFlag shall be equal to 0.

When ChromaUpsamplingFlag is equal to 1, ColourizationFlag shall be equal to 0.

When PictureRateUpsamplingFlag or TemporalExtrapolationFlag is equal to 1 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

When TemporalExtrapolationFlag is equal to 1, the extrapolated pictures generated by the NNPF follow all input pictures of the NNPF in output order. When TemporalExtrapolationFlag is equal to 1 and there is a decoded output picture that follows, in output order, the current picture for which the NNPF is activated, the extrapolated pictures generated by the NNPF precede that decoded output picture in output order.

nnpfc_id contains an identifying number that may be used to identify an NNPF. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification encountering an NNPFC SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the following applies:

–	This SEI message specifies a base NNPF.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS.

nnpfc_base_flag equal to 1 specifies that the SEI message specifies the base NNPF. nnpfc_base_flag equal to 0 specifies that the SEI message specifies an update relative to the base NNPF.

The following constraints apply to the value of nnpfc_base_flag:

–	When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the value of nnpfc_base_flag shall be equal to 1.

–	All NNPFC SEI messages in a CLVS that have a particular nnpfc_id value and nnpfc_base_flag equal to 1 shall have identical SEI payload content.

When nnpfc_base_flag is equal to 0, the following applies:

–	This SEI message defines an update relative to the preceding base NNPF in decoding order with the same nnpfc_id value. Updates are not cumulative but rather each update is applied on the base NNPF, which is the NNPF specified by the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS. The NNPF defined by this SEI message is obtained by applying the update defined by this SEI message relative to the base NNPF with the same nnpfc_id value.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent NNPFC SEI message, in decoding order, having nnpfc_base_flag equal to 0 and that particular nnpfc_id value within the current CLVS, whichever is earlier.

nnpfc_mode_idc, when equal to 0, indicates that the neural network information is contained in the NNPFC SEI message, and the neural network information is in the format of an ISO/IEC 15938-17 bitstream. nnpfc_mode_idc equal to 1 indicates that the neural network information is identified by the URI indicated by nnpfc_uri with the format identified by the tag URI nnpfc_tag_uri.

The value of nnpfc_mode_idc shall be in the range of 0 to 255, inclusive. Values of 2 to 255, inclusive, for nnpfc_mode_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_mode_idc in the range of 2 to 255, inclusive.

nnpfc_alignment_zero_bit_a shall be equal to 0.

nnpfc_tag_uri contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the format and associated information about the neural network used as a base NNPF or an update relative to the base NNPF with the same nnpfc_id value specified by nnpfc_uri.

NOTE 3 – nnpfc_tag_uri enables uniquely identifying the format of neural network data specified by nnpfc_uri without needing a central registration authority.

nnpfc_tag_uri equal to "tag:iso.org,2023:15938-17" indicates that the neural network data identified by nnpfc_uri conforms to ISO/IEC 15938-17.

nnpfc_uri contains a URI with syntax and semantics as specified in IETF Internet Standard 66 identifying the neural network used as a base NNPF or an update relative to the base NNPF with the same nnpfc_id value.

nnpfc_property_present_flag equal to 1 specifies that syntax elements related to the filter properties including purpose, input formatting, output formatting, and complexity are present. nnpfc_property_present_flag equal to 0 specifies that no syntax elements related to the filter properties are present.

When nnpfc_base_flag is equal to 1, nnpfc_property_present_flag shall be equal to 1.

When nnpfc_property_present_flag is equal to 0, the values of all syntax elements that may be present only when nnpfc_property_present_flag is equal to 1 are inferred to be equal to their corresponding syntax elements, respectively, in the NNPFC SEI message that contains the base NNPF for which this SEI message provides an update.

When an NNPFC SEI message nnpfcCurr is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, is not a repetition of the first NNPFC SEI message with that particular nnpfc_id value (in this case the value of nnpfc_base_flag is equal to 0), and the value of nnpfc_property_present_flag is equal to 1, the following constraints apply:

–	The values of syntax elements following nnpfc_property_present_flag and preceding nnpfc_complexity_info_present_flag, in decoding order, in the NNPFC SEI message shall be the same as the values of corresponding syntax elements in the first NNPFC SEI message, in decoding order, that has that particular nnpfc_id value within the current CLVS.

–	Either nnpfc_complexity_info_present_flag shall be equal to 0 or both nnpfc_complexity_info_present_flag shall be equal to 1 in the first NNPFC SEI message, in decoding order, that has that particular nnpfc_id value within the current CLVS (denoted as nnpfcBase below) and all the following constraints apply:

–	nnpfc_parameter_type_idc in nnpfcCurr shall be equal to nnpfc_parameter_type_idc in nnpfcBase.

–	nnpfc_log2_parameter_bit_length_minus3 in nnpfcCurr, when present, shall be less than or equal to nnpfc_log2_parameter_bit_length_minus3 in nnpfcBase.

–	If nnpfc_num_parameters_idc in nnpfcBase is equal to 0, nnpfc_num_parameters_idc in nnpfcCurr shall be equal to 0.

–	Otherwise (nnpfc_num_parameters_idc in nnpfcBase is greater than 0), nnpfc_num_parameters_idc in nnpfcCurr shall be greater than 0 and less than or equal to nnpfc_num_parameters_idc in nnpfcBase.

–	If nnpfc_num_kmac_operations_idc in nnpfcBase is equal to 0, nnpfc_num_kmac_operations_idc in nnpfcCurr shall be equal to 0.

–	Otherwise (nnpfc_num_kmac_operations_idc in nnpfcBase is greater than 0), nnpfc_num_kmac_operations_idc in nnpfcCurr shall be greater than 0 and less than or equal to nnpfc_num_kmac_operations_idc in nnpfcBase.

–	If nnpfc_total_kilobyte_size in nnpfcBase is equal to 0, nnpfc_total_kilobyte_size in nnpfcCurr shall be equal to 0.

–	Otherwise (nnpfc_total_kilobyte_size in nnpfcBase is greater than 0), nnpfc_total_kilobyte_size in nnpfcCurr shall be greater than 0 and less than or equal to nnpfc_total_kilobyte_size in nnpfcBase.

nnpfc_num_input_pics_minus1 plus 1 specifies the number of pictures used as input for the NNPF. The value of nnpfc_num_input_pics_minus1 shall be in the range of 0 to 63, inclusive. When PictureRateUpsamplingFlag is equal to 1, the value of nnpfc_num_input_pics_minus1 shall be greater than 0.

The variable numInputPics, specifying the number of pictures used as input for the NNPF, is derived as follows:

numInputPics = nnpfc_num_input_pics_minus1 + 1	(76)

nnpfc_input_pic_filtering_flag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfc_input_pic_filtering_flag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture. Each NNPF-generated picture is stored in the output tensor of the NNPF. When nnpfc_num_input_pics_minus1 is equal to 0, nnpfc_input_pic_filtering_flag[ 0 ] is inferred to be equal to 1. When PictureRateUpsamplingFlag is equal to 0 and nnpfc_num_input_pics_minus1 is greater than 0, nnpfc_input_pic_filtering_flag[ i ] shall be equal to 1 for at least one value of i in the range of 0 to nnpfc_num_input_pics_minus1, inclusive.

nnpfc_absent_input_pic_zero_flag equal to 1 indicates that the NNPF expects an input picture that is not present in the bitstream to be represented by sample arrays with sample values equal to 0. nnpfc_absent_input_pic_zero_flag equal to 0 indicates that the NNPF expects an input picture inputPicA that is not present in the bitstream to be represented by the input picture inputPicB that is the closest to inputPicA in output order and is present in the bitstream.

nnpfc_out_sub_c_flag specifies the values of the variables outSubWidthC and outSubHeightC when ChromaUpsamplingFlag is equal to 1. nnpfc_out_sub_c_flag equal to 1 specifies that outSubWidthC is equal to 1 and outSubHeightC is equal to 1. nnpfc_out_sub_c_flag equal to 0 specifies that outSubWidthC is equal to 2 and outSubHeightC is equal to 1. When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1.

nnpfc_out_colour_format_idc, when ColourizationFlag is equal to 1, specifies the colour format of the NNPF-generated pictures and consequently the values of the variables outSubWidthC and outSubHeightC. nnpfc_out_colour_format_idc equal to 1 specifies that the colour format of the NNPF-generated pictures is the 4:2:0 format and outSubWidthC and outSubHeightC are both equal to 2. nnpfc_out_colour_format_idc equal to 2 specifies that the colour format of the NNPF-generated pictures is the 4:2:2 format and outSubWidthC is equal to 2 and outSubHeightC is equal to 1. nnpfc_out_colour_format_idc equal to 3 specifies that the colour format of the NNPF-generated pictures is the 4:4:4 format and outSubWidthC and outSubHeightC are both equal to 1. The value of nnpfc_out_colour_format_idc shall not be equal to 0.

When ChromaUpsamplingFlag and ColourizationFlag are both equal to 0, outSubWidthC and outSubHeightC are inferred to be equal to SubWidthC and SubHeightC, respectively.

nnpfc_pic_width_num_minus1 plus 1 and nnpfc_pic_width_denom_minus1 plus 1 specify the numerator and denominator, respectively, for the resampling ratio of the width of the NNPF-generated pictures relative to CroppedWidth. Both nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 shall be in the range of 0 to 65 535, inclusive.

The value of ( nnpfc_pic_width_num_minus1 + 1 ) ÷ ( nnpfc_pic_width_denom_minus1 + 1 ) shall be in the range of 1 ÷ 16 to 16, inclusive. When nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are not present, the values of nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are both inferred to be equal to 0.

The variable nnpfcOutputPicWidth, representing the width of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicWidth = Ceil( CroppedWidth *		(77)
		( nnpfc_pic_width_num_minus1 + 1 ) ÷ ( nnpfc_pic_width_denom_minus1 + 1 ) )

When SpatialExtrapolation is equal to 1, nnpfcOutputPicWidth is updated as follows:

nnpfcOutputPicWidth += outSubWidthC * ( nnpfc_spatial_extrapolation_left_offset +	(77)
		nnpfc_spatial_extrapolation_right_offset ) [Ed. (YK): For now don't worry about the equation index values; all fields, including these, will be updated after the changes are integrated into the basis text.]

It is a requirement of bitstream conformance that the value of nnpfcOutputPicWidth % outSubWidthC shall be equal to 0.

nnpfc_pic_height_num_minus1 plus 1 and nnpfc_pic_height_denom_minus1 plus 1 specify the numerator and denominator, respectively, for the resampling ratio of the height of the NNPF-generated pictures relative to CroppedHeight. Both nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 shall be in the range of 0 to 65 535, inclusive.

The value of ( nnpfc_pic_height_num_minus1 + 1 ) ÷ ( nnpfc_pic_height_denom_minus1 + 1 ) shall be in the range of 1 ÷ 16 to 16, inclusive. When nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are not present, the values of nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are both inferred to be equal to 0.

The variable nnpfcOutputPicHeight, representing the height of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicHeight = Ceil( CroppedHeight *		(78)
		( nnpfc_pic_height_num_minus1 + 1 ) ÷ ( nnpfc_pic_height_denom_minus1 + 1 ) )

When SpatialExtrapolation is equal to 1, nnpfcOutputPicHeight is updated as follows:

nnpfcOutputPicHeight += outSubWidthC * ( nnpfc_spatial_extrapolation_top_offset +	(78)
		nnpfc_spatial_extrapolation_bottom_offset )

It is a requirement of bitstream conformance that the value of nnpfcOutputPicHeight % outSubHeightC shall be equal to 0.

When ResolutionResamplingFlag is equal to 1, at least one the following conditions shall be true:

–	The value of nnpfcOutputPicWidth is not equal to CroppedWidth.

–	The value of nnpfcOutputPicHeight is not equal to CroppedHeight.

–	SpatialExtrapolationFlag is equal to 1.

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the NNPF between the i-th and the ( i + 1 )-th input picture for the NNPF. The value of nnpfc_interpolated_pics[ i ] shall be in the range of 0 to 63, inclusive. When the nnpfc_interpolated_pics[ i ] syntax elements are present, the value of nnpfc_interpolated_pics[ i ] shall be greater than 0 for at least one value of i in the range of 0 to nnpfc_num_input_pics_minus1 − 1, inclusive.

NOTE 4 – When PictureRateUpsamplingFlag is equal to 1 for an NNPF and the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, only for a single value of i in the range of 0 to numInputPics − 1, inclusive, the value of nnpfc_interpolated_pics[ i ] is greater than 0.

nnpfc_extrapolated_pics_minus1 plus 1 specifies the number of extrapolated pictures generated by the NNPF subsequent to all input pictures for the NNPF in output order. The value of nnpfc_extrapolated_pics_minus1 shall be in the range of 0 to 62, inclusive.

The variables NumInpPicsInOutputTensor, specifying the number of pictures that have a corresponding input picture and are present in the output tensor of the NNPF, InpIdx[ idx ], specifying the input picture index, to the list of input pictures in reverse output order, of the idx-th picture that is present in the output tensor of the NNPF and has a corresponding input picture, and numPicsInOutputTensor, specifying the total number of pictures present in the output tensor of the NNPF, are derived as follows:

for( i = 0, numPicsInOutputTensor = 0; i < numInputPics; i++ )
	if( nnpfc_input_pic_filtering_flag[ i ] ) {
		InpIdx[ numPicsInOutputTensor ] = i
		numPicsInOutputTensor++
	}						(79)
NumInpPicsInOutputTensor = numPicsInOutputTensor
if( PictureRateUpsamplingFlag )
	for( i = 0; i  <=  numInputPics − 2; i++ )
		numPicsInOutputTensor  +=  nnpfc_interpolated_pics[ i ]
if( TemporalExtrapolationFlag )
	numPicsInOutputTensor  +=  nnpfc_extrapolated_pics + 1

nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_‌extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset specify the spatial extrapolation area. The luma samples with horizontal picture coordinates from outSubWidthC * nnpfc_spatial_extrapolation_left_offset to nnpfcOutputPicWidth − ( outSubWidthC * nnpfc_spatial_extrapolation_right_offset ) and vertical picture coordinates from outSubWidthC * nnpfc_spatial_extrapolation_top_offset to nnpfcOutputPicWidth − ( outSubWidthC * nnpfc_spatial_extrapolation_bottom_offset ) correspond to the spatial area of the input picture. The value of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_‌extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be in the range of 0 to 65 536, inclusive. At least one of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be greater than 0.

nnpfc_component_last_flag equal to 1 indicates that the last dimension in the input tensor inputTensor to the NNPF and the output tensor outputTensor of the NNPF is used for a current channel. nnpfc_component_last_flag equal to 0 indicates that the third dimension in the input tensor inputTensor to the NNPF and the output tensor outputTensor of the NNPF is used for a current channel.

NOTE 5 – The first dimension in the input tensor and in the output tensor is used for the batch index, which is a common practice in some neural network frameworks. While the equations in the semantics of this SEI message use the batch size corresponding to the batch index equal to 0, it is up to the post-processing implementation to determine the batch size used as the input to the neural network inference process.

NOTE 6 – For example, when nnpfc_inp_order_idc is equal to 3 and nnpfc_auxiliary_inp_idc is equal to 1, there are 7 channels in the input tensor, including four luma matrices, two chroma matrices, and one auxiliary input matrix. In this case, the process DeriveInputTensors( ) would derive each of these 7 channels of the input tensor one by one, and when a particular channel of these channels is processed, that channel is referred to as the current channel during the process.

nnpfc_inp_format_idc indicates the method of converting a sample value of the input picture to an input value to the NNPF. The value of nnpfc_inp_format_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_inp_format_idc in the range of 2 to 255, inclusive, are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_inp_format_idc in the range of 2 to 255, inclusive.

When nnpfc_inp_format_idc is equal to 0, the input values to the NNPF are real numbers and the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) − 1 )			(80)

InpC( x )= x ÷ ( ( 1  <<  BitDepthC ) − 1 )			(81)

When nnpfc_inp_format_idc is equal to 1, the input values to the NNPF are unsigned integer numbers and the functions InpY( ) and InpC( ) are specified as follows:

shiftY = BitDepthY − inpTensorBitDepthY
if( inpTensorBitDepthY  >=  BitDepthY)
	InpY( x ) = x  <<  ( inpTensorBitDepthY − BitDepthY )	(82)
else
	InpY( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthY ) − 1, ( x + ( 1  <<  ( shiftY − 1 ) ) )  >>  shiftY )

shiftC = BitDepthC − inpTensorBitDepthC
if( inpTensorBitDepthC  >=  BitDepthC )
	InpC( x ) = x  <<  ( inpTensorBitDepthC − BitDepthC ) 	(83)
else
	InpC( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthC ) − 1, ( x + ( 1  <<  ( shiftC − 1 ) ) )  >>  shiftC )

The variable inpTensorBitDepthY is derived from the syntax element nnpfc_inp_tensor_luma_bitdepth_minus8 as specified below. The variable inpTensorBitDepthC is derived from the syntax element nnpfc_inp_tensor_chroma_bitdepth_minus8 as specified below.

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor. nnpfc_auxiliary_inp_idc equal to 1 specifies that auxiliary input data is derived as specified in Equation 95.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 255, inclusive. Values of 2 to 255, inclusive, for nnpfc_auxiliary_inp_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_auxiliary_inp_idc in the range of 2 to 255, inclusive.

nnpfc_inp_order_idc indicates the method of ordering the sample arrays of an input picture to form an input tensor to the NNPF.

The value of nnpfc_inp_order_idc shall be in the range of 0 to 255, inclusive. Values of 4 to 255, inclusive, for nnpfc_inp_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_inp_order_idc in the range of 4 to 255, inclusive.

When ChromaFormatIdc is not equal to 1, nnpfc_inp_order_idc shall not be equal to 3.

When ChromaFormatIdc is equal to 0, nnpfc_inp_order_idc shall be equal to 0.

When ChromaUpsamplingFlag is equal to 1, nnpfc_inp_order_idc shall not be equal to 0.

Table 21 contains an informative description of nnpfc_inp_order_idc values.

Table 21 – Description of nnpfc_inp_order_idc values

| nnpfc_inp_ order_idc | Description |
| --- | --- |
| 0 | If nnpfc_auxiliary_inp_idc is equal to 0, one luma matrix is present in the input tensor for each input picture, and the number of channels is 1. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix and one auxiliary input matrix are present, and the number of channels is 2. |
| 1 | If nnpfc_auxiliary_inp_idc is equal to 0, two chroma matrices are present in the input tensor, and the number of channels is 2. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 1, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 3. |
| 2 | If nnpfc_auxiliary_inp_idc is equal to 0, one luma and two chroma matrices are present in the input tensor, and the number of channels is 3. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 4. |
| 3 | If nnpfc_auxiliary_inp_idc is equal to 0, four luma matrices and two chroma matrices are present in the input tensor, and the number of channels is 6. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 1, four luma matrices, two chroma matrices, and one auxiliary input matrix are present in the input tensor, and the number of channels is 7. The luma channels are derived in an interleaved manner as illustrated in Figure 12. This nnpfc_inp_order_idc can only be used when the input chroma format is 4:2:0. |
| 4..255 | Reserved |

Figure 12 – Illustration of deriving the four luma channels (right) from the luma component (left) when nnpfc_inp_order_idc is equal to 3

nnpfc_inp_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the input integer tensor. The value of inpTensorBitDepthY is derived as follows:

inpTensorBitDepthY = nnpfc_inp_tensor_luma_bitdepth_minus8 + 8	(84)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_inp_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the input integer tensor. The value of inpTensorBitDepthC is derived as follows:

inpTensorBitDepthC = nnpfc_inp_tensor_chroma_bitdepth_minus8 + 8	(85)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the NNPF are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) − 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  outTensorBitDepthY ) − 1, inclusive, and the chroma sample values output by the NNPF are unsigned integer numbers in the range of 0 to ( 1  <<  outTensorBitDepthC ) − 1, inclusive.

The value of nnpfc_out_format_idc shall be in the range of 0 to 255, inclusive. Values of 2 to 255, inclusive, for nnpfc_out_format_idc are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_out_format_idc in the range of 2 to 255, inclusive.

nnpfc_out_order_idc indicates the output order of samples resulting from the NNPF.

The value of nnpfc_out_order_idc shall be in the range of 0 to 255, inclusive. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive.

When ChromaUpsamplingFlag is equal to 1, nnpfc_out_order_idc shall not be equal to 0 or 3.

When ColourizationFlag is equal to 1, nnpfc_out_order_idc shall not be equal to 0.

Table 22 contains an informative description of nnpfc_out_order_idc values.

Table 22 – Description of nnpfc_out_order_idc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the output tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the output tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the output tensor, thus the number of channels is 3. |
| 3 | Four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. This nnpfc_out_order_idc can only be used when the output chroma format is 4:2:0. |
| 4..255 | Reserved |

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive. The value of outTensorBitDepthY is derived as follows:

outTensorBitDepthY = nnpfc_out_tensor_luma_bitdepth_minus8 + 8	(86)

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive. The value of outTensorBitDepthC is derived as follows:

outTensorBitDepthC = nnpfc_out_tensor_chroma_bitdepth_minus8 + 8	(87)

When BitDepthUpsamplingFlag is equal to 1, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 is present and outTensorBitDepthY is greater than BitDepthY.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is present and outTensorBitDepthC is greater than BitDepthC.

When nnpfc_inp_tensor_luma_bitdepth_minus8, nnpfc_inp_tensor_chroma_bitdepth_minus8, nnpfc_out_tensor_luma_bitdepth_minus8, and nnpfc_out_tensor_chroma_bitdepth_minus8 are present and outTensorBitDepthY is greater than inpTensorBitDepthY, outTensorBitDepthC shall not be less than inpTensorBitDepthC. When nnpfc_inp_tensor_luma_bitdepth_minus8, nnpfc_inp_tensor_chroma_bitdepth_minus8, nnpfc_out_tensor_luma_bitdepth_minus8, and nnpfc_out_tensor_chroma_bitdepth_minus8 are present and outTensorBitDepthC is greater than inpTensorBitDepthC, outTensorBitDepthY shall not be less than inpTensorBitDepthY.

nnpfc_separate_colour_description_present_flag equal to 1 indicates that a distinct combination of colour primaries, transfer characteristics, matrix coefficients, and scaling and offset values applied in association with the matrix coefficients for the picture resulting from the NNPF is specified in the SEI message syntax structure. nnpfc_separate_colour_description_present_flag equal to 0 indicates that the combination of colour primaries, transfer characteristics, matrix coefficients, and scaling and offset values applied in association with the matrix coefficients for the picture resulting from the NNPF is the same as implied by the VUI parameters vui_colour_primaries, vui_tranfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag that are indicated or inferred for the CLVS.

nnpfc_colour_primaries has the same semantics as specified in clause 7.3 for the vui_colour_primaries syntax element, except as follows:

–	nnpfc_colour_primaries specifies the colour primaries of the picture resulting from applying the NNPF specified in the SEI message, rather than the colour primaries used for the CLVS.

–	When nnpfc_colour_primaries is not present in the NNPFC SEI message, the value of nnpfc_colour_primaries is inferred to be equal to vui_colour_primaries.

nnpfc_transfer_characteristics has the same semantics as specified in clause 7.3 for the vui_transfer_characteristics syntax element, except as follows:

–	nnpfc_transfer_characteristics specifies the transfer characteristics of the picture resulting from applying the NNPF specified in the SEI message, rather than the transfer characteristics used for the CLVS.

–	When nnpfc_transfer_characteristics is not present in the NNPFC SEI message, the value of nnpfc_transfer_characteristics is inferred to be equal to vui_transfer_characteristics.

nnpfc_matrix_coeffs describes the equations used in deriving luma and chroma signals from the green, blue, and red, or Y, Z, and X primaries. Its semantics apply to the pictures resulting from applying the NNPF specified in this SEI message and are as specified for MatrixCoefficients in Rec. ITU-T H.273 | ISO/IEC 23091-2 with BitDepthY and BitDepthC being equal to outTensorBitDepthY and outTensorBitDepthC, respectively.

When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

nnpfc_matrix_coeffs shall not be equal to 0 unless both of the following conditions are true:

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

nnpfc_matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 is equal to nnpfc_out_tensor_luma_bitdepth_minus8 + 1, nnpfc_out_order_idc is equal to 2, outSubHeightC is equal to 1, and outSubWidthC is equal to 1.

nnpfc_full_range_flag indicates the scaling and offset values applied in association with the matrix coefficients as specified by nnpfc_matrix_coeffs. Its semantics are as specified for the VideoFullRangeFlag parameter in Rec. ITU-T H.273 | ISO/IEC 23091-2. When not present, the value of nnpfc_full_range_flag is inferred to be equal to 0.

nnpfc_chroma_loc_info_present_flag equal to 1 indicates the presence of the nnpfc_chroma_sample_loc_type_frame syntax element in the NNPFC SEI message. nnpfc_chroma_loc_info_present_flag equal to 0 indicates the absence of the nnpfc_chroma_sample_loc_type_frame syntax element in the NNPFC SEI message. When nnpfc_chroma_loc_info_present_flag is not present, its value is inferred to be equal to 0. When ColourizationFlag is equal to 0 or nnpfc_out_colour_format_idc is not equal to 1, the value of nnpfc_chroma_loc_info_present_flag shall be equal to 0.

nnpfc_chroma_sample_loc_type_frame, when not equal to 6 and nnpfc_out_colour_format_idc is equal to 1, specifies the location of chroma samples of the output pictures, as shown in Figure 1. nnpfc_chroma_sample_loc_type_frame equal to 6 and nnpfc_out_colour_format_idc equal to 1 indicates that the location of the chroma samples is unknown or unspecified or specified by other means not specified in this Specification. The value of nnpfc_chroma_sample_loc_type_frame shall be in the range of 0 to 6, inclusive.

nnpfc_overlap indicates the overlapping horizontal and vertical sample counts of adjacent input tensors of the NNPF. The value of nnpfc_overlap shall be in the range of 0 to 16 383, inclusive.

nnpfc_constant_patch_size_flag equal to 1 indicates that the NNPF accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. nnpfc_constant_patch_size_flag equal to 0 indicates that the NNPF accepts as input any patch size with width inpPatchWidth and height inpPatchHeight such that the width of an extended patch (i.e., a patch plus the overlapping area), which is equal to inpPatchWidth + 2 * nnpfc_overlap, is a positive integer multiple of nnpfc_extended_patch_width_cd_delta_minus1 + 1 + 2 * nnpfc_overlap, and the height of the extended patch, which is equal to inpPatchHeight + 2 * nnpfc_overlap, is a positive integer multiple of nnpfc_extended_patch_height_cd_delta_minus1 + 1 + 2 * nnpfc_overlap.

nnpfc_patch_width_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the horizontal sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to Min( 32 766, CroppedWidth − 1 ), inclusive.

nnpfc_patch_height_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the vertical sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to Min( 32 766, CroppedHeight − 1 ), inclusive.

nnpfc_extended_patch_width_cd_delta_minus1 plus 1 plus 2 * nnpfc_overlap, when nnpfc_constant_patch_size_flag equal to 0, indicates a common divisor of all allowed values of the width of an extended patch required for the input to the NNPF. The value of nnpfc_extended_patch_width_cd_delta_minus1 shall be in the range of 0 to Min( 32 766, CroppedWidth − 1 ), inclusive.

nnpfc_extended_patch_height_cd_delta_minus1 plus 1 plus 2 * nnpfc_overlap, when nnpfc_constant_patch_size_flag equal to 0, indicates a common divisor of all allowed values of the height of an extended patch required for the input to the NNPF. The value of nnpfc_extended_patch_height_cd_delta_minus1 shall be in the range of 0 to Min( 32 766, CroppedHeight − 1 ), inclusive.

Let the variables inpPatchWidth and inpPatchHeight be the patch size width and the patch size height, respectively.

If nnpfc_constant_patch_size_flag is equal to 0, the following applies:

–	The values of inpPatchWidth and inpPatchHeight are either provided by external means not specified in this Specification or set by the post-processor itself.

–	The value of inpPatchWidth + 2 * nnpfc_overlap shall be a positive integer multiple of nnpfc_extended_patch_width_cd_delta_minus1 + 1 + 2 * nnpfc_overlap and inpPatchWidth shall be less than or equal to CroppedWidth. The value of inpPatchHeight + 2 * nnpfc_overlap shall be a positive integer multiple of nnpfc_extended_patch_height_cd_delta_minus1 + 1 + 2 * nnpfc_overlap and inpPatchHeight shall be less than or equal to CroppedHeight.

Otherwise (nnpfc_constant_patch_size_flag is equal to 1), the value of inpPatchWidth is set equal to nnpfc_patch_width_minus1 + 1 and the value of inpPatchHeight is set equal to nnpfc_patch_height_minus1 + 1.

The variables outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, and outPatchCHeight are derived as follows:

outPatchWidth = ( nnpfcOutputPicWidth * inpPatchWidth ) / CroppedWidth	(88)

outPatchHeight = ( nnpfcOutputPicHeight * inpPatchHeight ) / CroppedHeight	(89)

horCScaling = SubWidthC / outSubWidthC			(90)

verCScaling = SubHeightC / outSubHeightC		(91)

outPatchCWidth = outPatchWidth * horCScaling		(92)

outPatchCHeight = outPatchHeight * verCScaling		(93)

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to nnpfcOutputPicWidth * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfcOutputPicHeight * inpPatchHeight.

nnpfc_padding_type indicates the process of padding when referencing sample locations outside the boundaries of the input picture as described in Table 23. The value of nnpfc_padding_type shall be in the range of 0 to 15, inclusive. Values of 5 to 15, inclusive, for nnpfc_padding_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_padding_type in the range of 5 to 15, inclusive.

Table 23 – Informative description of nnpfc_padding_type values

| nnpfc_padding_type | Description |
| --- | --- |
| 0 | Zero padding |
| 1 | Replication padding |
| 2 | Reflection padding |
| 3 | Wrap-around padding |
| 4 | Fixed padding |
| 5..15 | reserved |

nnpfc_luma_padding_val indicates the luma value to be used for padding when nnpfc_padding_type is equal to 4. The value of nnpfc_luma_padding_val shall be in the range of 0 to ( 1  <<  BitDepthY ) − 1, inclusive.

nnpfc_cb_padding_val indicates the Cb value to be used for padding when nnpfc_padding_type is equal to 4. The value of nnpfc_cb_padding_val shall be in the range of 0 to ( 1  <<  BitDepthC ) − 1, inclusive.

nnpfc_cr_padding_val indicates the Cr value to be used for padding when nnpfc_padding_type is equal to 4. The value of nnpfc_cr_padding_val shall be in the range of 0 to ( 1  <<  BitDepthC ) − 1, inclusive.

The function InpSampleVal( y, x, picHeight, picWidth, croppedPic, cIdx ) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, sample array croppedPic, and component index cIdx (equal to 0 for luma, 1 for Cb, and 2 for Cr) returns the value of sampleVal derived as follows:

NOTE 7 – For the inputs to the function InpSampleVal( ), the vertical location is listed before the horizontal location for compatibility with input tensor conventions of some inference engines.

if( nnpfc_padding_type  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ x ][ y ]			(94)
else if( nnpfc_padding_type  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picWidth − 1, x ) ][ Clip3( 0, picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  2 ) 
	sampleVal = croppedPic[ Reflect( picWidth − 1, x ) ][ Reflect( picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  3 ) 
	if( y >= 0  &&  y < picHeight )
		sampleVal = croppedPic[ Wrap( picWidth − 1, x ) ][ y ] 
else if( nnpfc_padding_type  = =  4 ) 
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = ( cIdx  = =  0 ? nnpfc_luma_padding_val : 
			( cIdx  = =  1 ? nnpfc_cb_padding_val : nnpfc_cr_padding_val ) )
	else
		sampleVal = croppedPic[ x ][ y ]

When nnpfc_auxiliary_inp_idc is equal to 1, the variable strengthControlScaledVal is derived as follows:

for( i = 0; i < numInputPics; i++ )
	if( nnpfc_inp_format_idc  = =  1 )				(95)
		if( nnpfc_inp_order_idc  = =  0  | |  nnpfc_inp_order_idc  = =  2  | |
				nnpfc_inp_order_idc  = =  3 )
			strengthControlScaledVal[ i ] = 
					Floor ( StrengthControlVal[ i ] * ( ( 1  <<  inpTensorBitDepthY ) − 1 ) )
		else if( nnpfc_inp_order_idc  = =  1 )
			strengthControlScaledVal[ i ] = 
					Floor ( StrengthControlVal[ i ] * ( ( 1  <<  inpTensorBitDepthC ) − 1 ) )
	else
		strengthControlScaledVal[ i ] = StrengthControlVal[ i ]

A patch is a rectangular array of samples from a component (e.g., a luma or chroma component) of a picture.

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag )
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
				else
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = strengthControlScaledVal[ i ]
			}
	else if( nnpfc_inp_order_idc  = =  1 )				(96)
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal[ i ]
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yY = cTop + yP
				xY = cLeft + xP
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal[ i ]
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yTL = cTop + yP * 2
				xTL = cLeft + xP * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yP
				xC = cLeft / 2 + xP
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ], 2 ) )
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
					inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
					inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = strengthControlScaledVal[ i ]
			}
}

The process StoreOutputTensors( ), for deriving sample values in the sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, for the NNPF-generated pictures, from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numPicsInOutputTensor; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfcOutputPicHeight  &&  xY < nnpfcOutputPicWidth )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )				(97)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfcOutputPicHeight / outSubHeightC  &&
						xSrc < nnpfcOutputPicWidth / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfcOutputPicHeight  &&  xY < nnpfcOutputPicWidth )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfcOutputPicHeight / 2  &&
						xSrc < nnpfcOutputPicWidth / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

An NNPF PostProcessingFilter( ) is the target NNPF as derived in the semantics of the NNPFA SEI message. The following example process may be used, with the NNPF PostProcessingFilter( ), to generate, in a patch-wise manner, the filtered and/or interpolated picture(s), which contain Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfc_out_order_idc:

if( nnpfc_inp_order_idc  = =  0  | |  nnpfc_inp_order_idc  = =  2 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth ) {
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}
else if( nnpfc_inp_order_idc  = =  1 )
	for( cTop = 0; cTop < CroppedHeight / SubHeightC; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth / SubWidthC; cLeft  +=  inpPatchWidth ) {	(98)
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}
else if( nnpfc_inp_order_idc  = =  3 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth * 2 ) {
			inputTensor = DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( outputTensor )
		}

An NNPF-generated picture with index i contains sample arrays FilteredYPic[ i ], FilteredCbPic[ i ], and FilteredCrPic[ i ], when present, that are derived by Equation 98. An NNPF-generated picture does not include the overlap regions.

The NNPF process consists of the process defined by Equation 98 followed by outputting NNPF-generated pictures in their increasing index order, where all NNPF-generated pictures that were interpolated by the NNPF are output and those NNPF-generated pictures that correspond to any input pictures to the NNPF are output as specified in the semantics of the NNPFA SEI message.

nnpfc_complexity_info_present_flag equal to 1 specifies that one or more syntax elements that indicate the complexity of the NNPF associated with the nnpfc_id are present. nnpfc_complexity_info_present_flag equal to 0 specifies that no syntax elements that indicates the complexity of the NNPF associated with the nnpfc_id are present.

nnpfc_parameter_type_idc equal to 0 indicates that the neural network uses only integer parameters. nnpfc_parameter_type_idc equal to 1 indicates that the neural network may use floating point or integer parameters. nnpfc_parameter_type_idc equal to 2 indicates that the neural network uses only binary parameters. nnpfc_parameter_type_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_parameter_type_idc equal to 3.

nnpfc_log2_parameter_bit_length_minus3 equal to 0, 1, 2, and 3 indicates that the neural network does not use parameters of bit length greater than 8, 16, 32, and 64, respectively. When nnpfc_parameter_type_idc is present and nnpfc_log2_parameter_bit_length_minus3 is not present, the neural network does not use parameters of bit length greater than 1.

nnpfc_num_parameters_idc indicates the maximum number of neural network parameters for the NNPF in units of a power of 2 048. nnpfc_num_parameters_idc equal to 0 indicates that the maximum number of neural network parameters is unknown. The value nnpfc_num_parameters_idc shall be in the range of 0 to 52, inclusive. Values of nnpfc_num_parameters_idc greater than 52 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_num_parameters_idc greater than 52.

If the value of nnpfc_num_parameters_idc is greater than zero, the variable maxNumParameters is derived as follows:

maxNumParameters = ( 2 048  <<  nnpfc_num_parameters_idc ) − 1	(99)

It is a requirement of bitstream conformance that the number of neural network parameters of the NNPF shall be less than or equal to maxNumParameters.

nnpfc_num_kmac_operations_idc greater than 0 indicates that the maximum number of multiply-accumulate operations per sample of the NNPF is less than or equal to nnpfc_num_kmac_operations_idc * 1 000. nnpfc_num_kmac_operations_idc equal to 0 indicates that the maximum number of multiply-accumulate operations of the network is unknown. The value of nnpfc_num_kmac_operations_idc shall be in the range of 0 to 232 − 2, inclusive.

nnpfc_total_kilobyte_size greater than 0 indicates a total size in kilobytes required to store the uncompressed parameters for the neural network. The total size in bits is a number equal to or greater than the sum of bits used to store each parameter. nnpfc_total_kilobyte_size is the total size in bits divided by 8 000, rounded up. nnpfc_total_kilobyte_size equal to 0 indicates that the total size required to store the parameters for the neural network is unknown. The value of nnpfc_total_kilobyte_size shall be in the range of 0 to 232 − 2, inclusive.

nnpfc_num_metadata_extension_bits equal to 0 specifies that nnpfc_reserved_metadata_extension is not present. When nnpfc_num_metadata_extension_bits is greater than 0, let the variable numSpecifiedMetadataExtensionBits be the number of bits representing all syntax elements between nnpfc_num_metadata_extension_bits and nnpfc_reserved_metadata_extension. nnpfc_num_metadata_extension_bits greater than 0 specifies the sum of numSpecifiedMetadataExtensionBits and the length, in bits, of nnpfc_reserved_metadata_extension.

The value of nnpfc_num_metadata_extension_bits shall be in the range of numSpecifiedMetadataExtensionBits to 2 048, inclusive. Values in the range of numSpecifiedMetadataExtensionBits + 1 to 2 048, inclusive, for nnpfc_num_metadata_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of nnpfc_num_metadata_extension_bits in the range of 0 to numSpecifiedMetadataExtensionBits + 1 to 2 048, inclusive.

nnpfc_application_purpose_tag_uri_present_flag equal to 1 indicates that the nnpfc_application_purpose_tag_uri syntax element is present in this NNPFC SEI message. nnpfc_application_purpose_tag_uri_present_flag equal to 0 indicates that the nnpfc_application_purpose_tag_uri syntax element is not present in this NNPFC SEI message. When not present nnpfc_application_purpose_tag_uri_present_flag is inferred to be equal to 0.

nnpfc_application_purpose_tag_uri specifies a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the application determined purpose of the NNPF, when nnpfc_purpose is equal to 0.

NOTE 4 – nnpfc_application_purpose_tag_uri enables uniquely identifying the application determined purpose of NNPF without needing a central registration authority.

nnpfc_scan_type_idc equal to 0 indicates that the preferred display method for the pictures output by the NNPF is unknown or unspecified or specified by external means. nnpfc_scan_type_idc equal to 1 indicates that the pictures output by the NNPF are suitable for display using overscan. nnpfc_scan_type_idc equal to 2 indicates that the pictures output by the NNPF contain visually important information in the entire region out to the edges of the picture, such that the pictures output by the NNPF should not be displayed using overscan. Instead, they should be displayed using either an exact match between the display area and the edges, or using underscan. As used in this paragraph, the term "overscan" refers to display processes in which some parts near the borders of the pictures are not visible in the display area. The term "underscan" describes display processes in which the entire pictures are visible in the display area, but they do not cover the entire display area. For display processes that neither use overscan nor underscan, the display area exactly matches the area of the pictures. The value of nnpfc_scan_type_idc shall not be equal to 2. When not present, the value of nnpfc_scan_type_idc is inferred to be equal to 0.

nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset specify the spatial extrapolation area. The luma samples with horizontal picture coordinates from outSubWidthC * nnpfc_spatial_extrapolation_left_offset to nnpfcOutputPicWidth − ( outSubWidthC * nnpfc_spatial_extrapolation_right_offset ) and vertical picture coordinates from outSubWidthC * nnpfc_spatial_extrapolation_top_offset to nnpfcOutputPicWidth − ( outSubWidthC * nnpfc_spatial_extrapolation_bottom_offset ) correspond to the spatial area of the input picture. The value of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be in the range of 0 to 65536, inclusive. At least one of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be greater than 0.

nnpfc_reserved_metadata_extension shall not be present in bitstreams conforming to this version of this Specification. However, decoders conforming to this version of this Specification shall ignore the presence and value of nnpfc_reserved_metadata_extension. When present, the length, in bits, of nnpfc_reserved_metadata_extension is equal to nnpfc_num_metadata_extension_bits − numSpecifiedMetadataExtensionBits.

nnpfc_alignment_zero_bit_b shall be equal to 0.

nnpfc_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnpfc_payload_byte[ i ] for all present values of i shall be a complete bitstream that conforms to ISO/IEC 15938-17.

# Add clauses 8.30 to 8.34 as follows:

  - Encoder optimization information SEI message

    - Encoder optimization information SEI message syntax

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| if(EoiSpatialResamplingFlag) |  |
| eoi_spatial_resampling_type_flag | u(1) |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_type_idc | u(4) |
| eoi_privacy_protected_info_type | u(8) |
| } |  |
| } |  |
| } |  |

    - Encoder optimization information SEI message semantics

The encoder optimization information SEI message is used to indicate if the video has been optimized for human viewing or machine analysis and which types of optimization have been applied in pre-processing or encoding.

eoi_cancel_flag equal to 1 specifies that the persistence of the encoder optimization information SEI message included in any previous PU in output order is cancelled. eoi_cancel_flag equal to 0 indicates that information on optimization that has been applied in pre-processing or encoding follows.

eoi_persistence_flag specifies the persistence of the optimization information provided in this SEI message. eoi_persistence_flag equal to 0 specifies that the optimization information applies for the current picture only. eoi_persistence_flag equal to 1 specifies that the optimization information applies for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with an encoder optimization information SEI message is output that follows the current picture in output order.

eoi_for_human_viewing_idc equal to 3 specifies that purposes for the applied optimization include human viewing. eoi_for_human_viewing_idc equal to 2 specifies that the video is suitable but not specifically optimized for human viewing. eoi_for_huma_viewing_idc equal to 1 specifies that the video is unsuitable for human viewing. eoi_for_human_viewing_idc equal to 0 specifies that it is unknown if the video is suitable for human viewing.

eoi_for_machine_analysis_idc equal to 3 specifies that purposes for the applied optimization include machine analysis. eoi_for_machine_analysis_idc equal to 2 specifies that the video is suitable but not specifically optimized for machine analysis. eoi_for_machine_analysis_idc equal to 1 specifies that the video is unsuitable for machine analysis. eoi_for_machine_analysis_idc equal to 0 specifies that it is unknown if the video is suitable for machine analysis.

It is a requirement of bitstream conformance that the value of eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc shall not be both equal to 1.

eoi_type indicates the types of optimization method as specified in Table x1 where ( eoi_type & bitMask ) not equal to 0 indicates that the optimization type with the bitMask value in Table x1 has been applied. When eoi_type is greater than 0 and ( eoi_type & bitMask ) is equal to 0, the optimization type with the bitMask value has not been applied. When eoi_type is equal to 0, optimization as determined by the application has been used.

Table x1 – Definition of eoi_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| 0x02 | Temporal resampling optimization |
| 0x04 | Spatial resampling optimization |
| 0x08 | Temporal quality optimization in a manner that quality fluctuates temporally |
| 0x10 | Spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |
| 0x20 | Privacy protection optimization; the pictures for which this SEI message persists have been pre-processed or encoded to protect personal information. (e.g. removal or replacing of personal identifiable information, pseudonymization, anonymization) |

The variables EoiObjectBasedFlag, EoiTemporalResamplingFlag, EoiSpatialResamplingFlag, EoiTemporalQualityFlag, EoiSpatialQualityFlag, and EoiPrivacyProtectionFlag, specifying whether eoi_type indicates the type of the optimization to include object-based optimization, temporal resampling optimization, spatial resampling optimization, temporal quality optimization, spatial quality optimization, and privacy protection optimization, respectively, are derived as follows:

EoiObjectBasedFlag = ( ( eoi_type & 0x01 ) > 0 ) ? 1 : 0
EoiTemporalResamplingFlag = ( ( eoi_type & 0x02 ) > 0 ) ? 1 : 0
EoiSpatialResamplingFlag = ( ( eoi_type & 0x04 ) > 0 ) ? 1 : 0	(xx)
EoiTemporalQualityFlag = ( ( eoi_type & 0x08 ) > 0 ) ? 1 : 0
EoiSpatialQualityFlag = ( ( eoi_type & 0x10 ) > 0 ) ? 1 : 0
EoiPrivacyProtectionFlag = ( ( eoi_type & 0x20 ) > 0 ) ? 1 : 0

NOTE – For example, when certain highest temporal sublayers have been encoded with such coarse quantization that human viewers perceive the quality fluctuation annoying, but machine task performance is not compromised, eoi_for_human_viewing_flag and eoi_for_machine_analaysis_flag can be set equal to 0 and 1, respectively, and eoi_type can be set equal to a value that causes EoiTemporalQualityFlag to be equal to 1.

When eoi_persistence_flag is equal to 0, it is a requirement of bitstream conformance that EoiTemporalResamplingFlag shall be equal to 0 and EoiTemporalQualityFlag shall be equal to 0.

eoi_object_based_idc, when present, indicates the type of object-based optimization as specified in Table x2, where ( eoi_object_based_idc & bitMask ) not equal to 0 indicates that the object-based optimization type associated with the bitMask value in Table x2 has been applied. When eoi_object_based_idc is greater than 0 and ( eoi_object_based_idc & bitMask ) is equal to 0, the object-based optimization type associated with the bitMask value has not been applied. When eoi_object_based_idc is equal to 0, an application-defined type of object-based optimization has been applied. The value of eoi_object_based_idc shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 65 535, inclusive, for eoi_object_based_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_object_based_idc is in the range of 8 to 65 535, inclusive, decoders conforming to this version of this Specification shall ignore eoi_object_based_idc.

Table x2 – Definition of eoi_object_based_idc

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Areas outside the detected objects have been blurred prior to encoding. |
| 0x02 | Areas outside the detected objects have been encoded with coarser transform-domain quantization than the quantization used for the detected objects. |
| 0x04 | Areas outside the detected objects have been overwritten. For example, an encoding system can overwrite areas outside the detected objects with a constant sample value. |

eoi_temporal_resampling_type_flag equal to 0 specifies that the temporal resampling optimization is a subsampling operation. eoi_temporal_resampling_type_flag equal to 1 specifies that the temporal resampling optimization is an upsampling operation.

eoi_num_int_pics greater than 0 indicates that the count of pictures that the encoding system excluded between each pair of coded pictures in output order (when eoi_temporal_resampling_type_flag is equal to 0) or added between each pair of source pictures for encoding (when eoi_temporal_resampling_type_flag is equal to 1) within the persistence of this SEI message is constant. When eoi_temporal_resampling_type_flag is equal to 0 and eoi_num_int_pics is greater than 0, eoi_num_int_pics specifies the count of pictures that the encoding system excluded between each pair of coded pictures in output order. When eoi_temporal_resampling_type_flag is equal to 1 and eoi_num_int_pics is greater than 0, eoi_num_int_pics specifies the count of pictures that the encoding system added between each pair of source pictures for encoding.

eoi_num_int_pics equal to 0 indicates that the count of pictures that the encoding system excluded between each pair of coded pictures in output order (when eoi_temporal_resampling_type_flag is equal to 0) or added between each pair of source pictures for encoding (when eoi_temporal_resampling_type_flag is equal to 1) within the persistence of this SEI message is unknown or varying.

The value of eoi_num_int_pics shall be in the range of 0 to 63, inclusive.

eoi_spatial_resampling_type_flag equal to 0 specifies that the spatial resampling optimization is a subsampling operation. eoi_spatial_resampling_type_flag equal to 1 specifies that the spatial resampling optimization is an up-sampling operation.

eoi_privacy_protection_type_idc, when present, indicates the type of privacy protection optimization as specified in Table x3.

Table x3 – Definition of eoi_privacy_protection_type_idc

| eoi_privacy_protection_type_idc | Interpretation |
| --- | --- |
| 0 | Unknown or determined by the application. |
| 1 | Blurring; personal information is blurred to make it unidentifiable. |
| 2 | Replacing; personal information is replaced with something different from the original to make it unidentifiable. |
| 3 | Masking; personal information is masked so that it cannot be identified |
| 4…15 | Reserved for future use. |

eoi_privacy_protected_info_type, when present, indicates the types of protected information as specified in Table x4 where eoi_privacy_protected_info_type is greater than 0 and ( eoi_privacy_protected_info_type & bitMask ) not equal to 0 indicates that the information type with the bitMask value in Table x4 has been protected. When eoi_privacy_protected_info_type is equal to 0, an application-defined type of information has been protected. The value of eoi_privacy_protection_info_type shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 255, inclusive, for eoi_privacy_protected_info_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_privacy_protected_info_type is in the range of 8 to 255, inclusive, decoders conforming to this version of this Specification shall ignore eoi_privacy_protected_info_type.

Table x4 – Definition of eoi_privacy_protection_info_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Information that identifies a person is protected. For example, the face of the person. |
| 0x02 | Information that can identify vehicles is protected. For example, the license plate of the vehicle. |
| 0x04 | Information that can infer locations is protected. For example text or images on signs. |

  - Source picture timing information SEI message

    - Source picture timing information SEI message syntax

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_source_type_present_flag | u(1) |
| if( spti_source_type_present_flag ) |  |
| spti_source_type | u(16) |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_interval | u(32) |
| if( spti_persistence_flag ) |  |
| spti_max_sublayers_minus_1 | u(3) |
| for( i = 0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Source picture timing information SEI message semantics

The source picture timing information (SPTI) SEI message indicates the temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding, e.g., for camera-captured content, the temporal distance between source pictures is the difference between the time at which an image sensor was exposed to produce a source picture associated with the current decoded picture and the time at which the image sensor was exposed to produce the source picture associated with a previous decoded picture in output order. The information provided by the SPTI SEI message pertains only for picture(s) starting from the picture in the current layer in the access unit that contains the SPTI SEI message and all subsequent pictures of the current layer in output order based on its persistence. [Ed. Check phrasing of this. “Pertains” is not used in a similar way anywhere in the standard.]

spti_cancel_flag equal to 1 indicates that the SPTI SEI message cancels the persistence of any previous SPTI SEI message in output order that applies to the current layer. spti_cancel_flag equal to 0 indicates that source picture timing information follows.

spti_persistence_flag specifies the persistence of the SPTI SEI message for the current layer.

spti_persistence_flag equal to 0 specifies that the SPTI SEI message applies to the current decoded picture only.

spti_persistence_flag equal to 1 specifies that the SPTI SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an SPTI SEI message is output that follows the current picture in output order.

spti_source_timing_equals_output_timing_flag equal to 1 indicates the timing of source pictures is the same as the timing of corresponding decoded output pictures. spti_source_timing_equals_output_timing_flag equal to 0 indicates the timing of source pictures might not be the same as the timing of corresponding decoded output pictures.

When spti_source_timing_equals_output_timing_flag is equal to 1 and a picture timing SEI message is present for the current picture, source picture timing could be determined from information conveyed in the picture timing SEI message.

spti_source_type_present_flag equal to 1 indicates the syntax element spti_source_type is present in the SEI message. spti_source_type_present_flag equal to 0 indicates the syntax element spti_source_type is not present in the SEI message.

spti_source_type indicates the timing relationship between source pictures and corresponding decoded output pictures as specified in Table X, where ( spti_source_type & bitMask ) not equal to 0 indicates that the timing relationship has the interpretation associated with the bitMask value in Table X. When spti_source_type is greater than 0 and ( spti_source_type & bitMask ) is equal to 0, the interpretation associated with the bitMask value is not applicable to the SPTI SEI message. When spti_source_type is equal to 0, the timing relationship may be specified by the application.

The value of spti_source_type shall be in the range of 0 to 127, inclusive, in bitstreams conforming to this edition of this document. Values of 128 to 255, inclusive, for spti_source_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore SPTI SEI messages with spti_source_type in the range of 128 to 255, inclusive.

Table X – Interpretation of spti_source_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Slow motion: The absolute value of the temporal distance between consecutive source pictures is likely to be less than the temporal distance between corresponding decoded output pictures. |
| 0x02 | Sped-up motion: The absolute value of the temporal distance between consecutive source pictures is likely to be greater than the temporal distance between corresponding decoded output pictures. |
| 0x04 | High-speed imaging: The absolute value of the temporal distance between consecutive source pictures is likely to be less than 1/120 seconds. |
| 0x08 | Time-lapse imaging: The temporal distance between source pictures is likely to be greater than 1.001/24 seconds. |
| 0x10 | Temporal reversal: The absolute value of the temporal distance between consecutive source pictures is indicated to be negative (i.e., decoded pictures are output in reverse temporal order relative to the timing of the corresponding source pictures). |
| 0x20 | Still image / freeze frame: The temporal distance between source pictures is likely to be 0 (i.e., two or more decoded pictures are likely to represent the same source picture). |
| 0x40 | Sporadic or event-driven: The temporal distance between source pictures is likely to be non-constant. |

The value of ( spti_source_type & 0x04 ) & ( spti_source_type & 0x08 ) shall be zero (i.e., spti_source_type shall not simultaneously indicate high-speed imaging and time-lapse imaging).

spti_time_scale specifies the number of time units that pass in one second. The value of spti_time_scale shall not be equal to 0. For example, a time coordinate system that measures time using a 27 MHz clock has an spti_time_scale of 27 000 000.

spti_num_units_in_elemental_interval specifies the number of time units of a clock operating at the frequency spti_time_scale Hz that corresponds to the indicated elemental source picture interval of consecutive pictures in output order in the CLVS. The value of spti_num_units_in_elemental_interval shall not be equal to 0.

The indicated elemental source picture interval, also to be denoted by the variable ElementalSourcePictureInterval, in units of seconds, is equal to the quotient of spti_num_units_in_elemental_interval divided by spti_time_scale. For example, to represent an elemental source picture interval equal to 0.04 seconds, spti_time_scale may be equal to 27 000 000 and spti_num_units_in_elemental_interval may be equal to 1 080 000.

NOTE 1 – The method of indicating the elemental source picture interval is similar to that used for the timing and HRD parameters syntax used in several video coding standards such as Rec. ITU-T H.266 | ISO/IEC 23090-3, with spti_time_scale being similar to that syntax’s time_scale and spti_num_units_in_elemental_interval being similar to that syntax’s num_units_in_tick, and thus the variable ElementalSourcePictureInterval being similar to the variable ClockTick in Rec. ITU-T H.266 | ISO/IEC 23090-3.

spti_max_sublayers_minus_1 plus 1 specifies the maximum number of temporal sublayers for which picture interval scale factor (spti_sublayer_interval_scale_factor[ i ]) and synthesized flag (spti_sublayer_synthesized_picture_flag[ i ]) information is signalled. When spti_max_sublayers_minus_1 is not present, it is inferred to be equal to TemporalId..

spti_sublayer_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the source picture interval of corresponding pictures in the CLVS having TemporalId equal to i relative to the previous output picture with TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture with TemporalId less than or equal to i.

The indicated source picture interval associated with an output picture having TemporalId equal to i, relative to the previous output picture with TemporalId less than or equal to i, denoted by the variable SourcePictureInterval[ i ], in units of seconds, is derived as follows:

SourcePictureInterval[ i ] = ElementalSourcePictureInterval * spti_sublayer_interval_scale_factor[ i ] *
( 1 − 2 * temporalReversalFlag )		(8-X)

If spti_source_type_present_flag is equal to 1, the variable temporalReversalFlag is equal to ( spti_source_type & 0x10 )? 1 : 0. Otherwise (i.e. if spti_source_type_present_flag is equal to 0), the variable temporalReversalFlag is equal to 0.

NOTE 2 –Since ElementalSourcePictureInterval is multiplied by spti_sublayer_interval_scale_factor[ i ] when calculating SourcePictureInterval[ i ], it is possible to represent the same value of SourcePictureInterval[ i ] in multiple ways by applying a scale factor to the value of spti_time_scale and applying the same scale factor to spti_num_units_in_elemental_interval or spti_sublayer_interval_scale_factor[ i ]. There is no assumption that common scale factors have been removed or that the value of spti_sublayer_interval_scale_factor[ i ] is equal to 1 for the highest value of i. The reason to allow the same value to be represented in multiple ways is, at least in part, to allow spti_time_scale to be chosen to correspond with other timing-related elements used in the system environment, such as the clock rate of 27 MHz used in some multimedia communication systems.

spti_sublayer_synthesized_picture_flag[ i ], when present, equal to 1 indicates that decoded output pictures belonging to the ith temporal sublayer are synthesized and do not correspond to unmodified original source pictures. spti_sublayer_synthesized_picture_flag[ i ] equal to 0 provides no such indication. When not present, the value of spti_sublayer_synthesized_picture_flag[ i ] is inferred to be equal to 0.

NOTE 2 – When the TemporalId of an SPTI SEI message is greater than 0, and the SPTI SEI message persists for one or more pictures with lower TemporalId, an encoder can repeat the information of the SPTI SEI message by including it in one or more SPTI SEI messages with lower TemporalId, in order to avoid loss of information when pictures in temporal sublayer(s) are lost or removed.

  - Object mask information SEI message

    - Object mask information SEI message syntax

| object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_cancel_flag | u(1) |
| if( !omi_cancel_flag ) { |  |
| omi_persistence_flag | u(1) |
| omi_num_aux_pic_layer | ue(v) |
| omi_mask_id_length_minus1 | ue(v) |
| omi_mask_sample_value_length_minus8 | ue(v) |
| omi_mask_confidence_info_present_flag | u(1) |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence_length_minus1 | u(4) |
| omi_mask_depth_info_present_flag | u(1) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth_length_minus1 | u(4) |
| omi_mask_label_info_present_flag | u(1) |
| if( omi_mask_label_info_present_flag ) { |  |
| omi_mask_label_language_present_flag | u(1) |
| if( omi_mask_label_language_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_label_language | st(v) |
| } |  |
| } |  |
| for( i = 0; i < omi_num_aux_pic_layer; i++ ) { |  |
| omi_mask_pic_update_flag[ i ] | f(1) |
| if( omi_mask_pic_update_flag[ i ] ) { |  |
| omi_num_mask_in_pic_update[ i ] | ue(v) |
| for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ ) { |  |
| omi_mask_id[ i ][ j ] | u(v) |
| omi_aux_sample_value[ i ][ j ] | u(v) |
| omi_mask_cancel[ i ][ j ] | u(1) |
| if( !omi_mask_cancel[ i ][ j ] ) { |  |
| omi_mask_bounding_box_present_flag[ i ][ j ] | u(1) |
| if( omi_mask_bounding_box_present_flag[ i ][ j ] ) { |  |
| omi_mask_top[ i ][ j ] | u(16) |
| omi_mask_left[ i ][ j ] | u(16) |
| omi_mask_width[ i ][ j ] | u(16) |
| omi_mask_height[ i ][ j ] | u(16) |
| } |  |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence[ i ][ j ] | u(v) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth[ i ][ j ] | u(v) |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| if( omi_mask_label_info_present_flag ) |  |
| omi_mask_label[ i ][ j ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

    - Object mask information SEI message semantics

The object mask information (OMI) SEI message provides object mask information of object mask picture in the auxiliary picture layer associated with the primary picture layer, referred to as the current primary picture layer, in which the SEI message is present. If the OMI SEI message is present, it shall be present in a primary picture layer. One primary picture layer can be associated with one or more auxiliary picture layers. For each of the associated auxiliary picture layers containing the object mask picture with nuh_layer_id equal [Ed. (JB): Shouldn’t refer directly to nuh_layer_id syntax element in VSEI, but have an input variable. The same problem exists in depth representation info SEI and alpha channel info SEI.] to sdi_layer_id[ i ], the value of sdi_aux_id[ i ] shall be equal to AUX_OBJECT_MASK, for any value of i in range of 0 to sdi_max_layers_minus1, inclusive. [Ed. (JB): Is there a requirement that the SDI SEI be present when OMI SEI is present? Is there a requirement that the object mask aux pictures be associated in the SDI SEI with the primary picture carrying the OMI SEI?]

Use of this SEI message requires the definition of the following variables:

–	A cropped picture width and picture height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	A conformance cropping window left offset, ConfWinLeftOffset

–	A conformance cropping window top offset, ConfWinTopOffset

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

omi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous object mask information SEI message in the same layer, if present, in output order. omi_cancel_flag equal to 0 indicates that object mask information follows.

omi_persistence_flag specifies the persistence of the object mask information provided in this SEI message. omi_persistence_flag equal to 0 specifies that the object mask information applies to the current picture only. omi_persistence_flag equal to 1 specifies that the object mask information applies to the current picture and all subsequent pictures in the same layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in a PU containing an object mask information SEI message is output that follows the current picture in output order.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to AUX_OBJECT_MASK for at least one value of i, the OMI SEI message shall be ignored.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to AUX_OBJECT_MASK for at least one value of i and an OMI SEI message, the SDI SEI message shall precede the OMI SEI message in decoding order.

omi_num_aux_pic_layer indicates the number of auxiliary picture layers associated with the current primary picture layer. It is a requirement of bitstream conformance that the value of omi_num_aux_pic_layer shall be equal to numAuxLayer where the variable numAuxLayeris derived as follows.

Let omiPrimaryLayerId be the value of nuh_layer_id of the NAL unit that contains the SEI message.

numAuxLayer = 0;
for( i = 0; i <= sdi_max_layers_minus1; i++ )
	if( sdi_aux_id[ i ] = = AUX_OBJECT_MASK )
		for( j = 0; j <= sdi_num_associated_primary_layers_minus1[ i ]; j++ )			(xx)
			if( sdi_layer_id[ sdi_associated_primary_layer_idx[ i ][ j ] ]  = =  omiPrimaryLayerId )
				numAuxLayer++;

omi_mask_id_length_minus1 plus 1 specifies the length, in bits, of omi_mask_id[ i ][ j ] syntax elements.

omi_mask_sample_value_length_minus8 plus 8 specifies the length, in bits, of omi_aux_sample_value[ i ][ j ] syntax elements. The value of omi_mask_sample_value_length_minus8 shall be in the range of 0 to 8.

omi_mask_confidence_info_present_flag equal to 1 indicates that omi_mask_confidence[ i ][ j ] syntax elements are present. omi_mask_confidence_info_present_flag equal to 0 indicates that omi_mask_confidence[ i ][ j ]syntax elements are not present.

omi_mask_confidence_length_minus1 plus 1 specifies the length, in bits, of the omi_mask_confidence[ i ][ j ] syntax elements.

omi_mask_depth_info_present_flag equal to 1 indicates that omi_mask_depth[ i ][ j ] syntax elements are present. omi_mask_depth_info_present_flag equal to 0 indicates that omi_mask_depth[ i ][ j ]syntax elements are not present.

omi_mask_depth_length_minus1 plus 1 specifies the length, in bits, of the omi_mask_depth[ i ][ j ] syntax elements.

It is a requirement of bitstream conformance that the value of omi_num_aux_pic_layer, omi_mask_id_length_minus1, omi_mask_sample_value_length_minus8, omi_mask_confidence_info_present_flag, omi_mask_confidence_length_minus1, if present, omi_mask_depth_info_present_flag and omi_mask_depth_length_minus1, if present, shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omi_mask_label_info_present_flag equal to 1 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ][ j ] syntax elements are present. omi_mask_label_info_present_flag equal to 0 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ][ j ] syntax elements are not present.

omi_mask_label_language_present_flag equal to 1 indicates that omi_mask_label_language syntax element is present. omi_mask_label_language_present_flag equal to 0 indicates that omi_mask_label_language syntax element is not present.

omi_bit_equal_to_zero shall be equal to 0.

omi_mask_label_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the omi_mask_label_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When not present, the language of the label is unspecified.

omi_mask_pic_update_flag[ i ] equal to 1 indicates the object mask information of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer may be updated. omi_mask_pic_update_flag[ i ] equal to 0 indicates there is no change to the mask information of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. When omi_mask_pic_update_flag[ i ] is equal to 0, the persistence mechanism is used, that is the object mask information is inherited from the last OMI SEI message present in the same layer in decoding order which signals the mask information of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer.

omi_num_mask_in_pic_update[ i ] specifies the number of object masks in object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. omi_num_mask_in_pic_update [ i ] shall be in the range of 0 to (1<<(omi_mask_id_length_minus1 + 1)) – 1, inclusive.

omi_mask_id[ i ][ j ] indicates the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The length of the omi_mask_id[ i ][ j ] syntax element is omi_mask_id_length_minus1 + 1 bits.

The variable maskId[ i ][ j ] specifying the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer is derived as follows:

for( i = 0; i < omi_num_aux_pic_layer; i++ )
		for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ ) 	(xx)
			maskId[ i ][ j ] = omi_mask_id[ i ][ j ] + (1<<(omi_mask_id_length_minus1 + 1))*i

omi_aux_sample_value[ i ][ j ] specifies the value of the samples within the area of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer.

omi_mask_cancel[ i ][ j ] equal to 1 cancels the persistence scope of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture. omi_mask_cancel[ i ][ j ] equal to 0 specifies that the information of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer is signalled.

It is a requirement of bitstream conformance that when omi_mask_id[ i ][ j ] with a particular value is parsed for the first time in the current CLVS, the value of the corresponding omi_mask_cancel[ i ][ j ] shall be equal to 0.

omi_mask_bounding_box_present_flag[ i ][ j ] equal to1 specifies that the syntax elements omi_mask_top[ i ][ j ], omi_mask_left[ i ][ j ], omi_mask_width[ i ][ j ], and omi_mask_height[ i ][ j ], are present. omi_mask_bounding_box_present_flag[ i ][ j ] equal to 0 indicates syntax elements, omi_mask_top[ i ][ j ], omi_mask_left[ i ][ j ], omi_mask_width[ i ][ j ], and omi_mask_height[ i ][ j ], are not present.

omi_mask_top[ i ][ j ], omi_mask_left[ i ][ j ], omi_mask_width[ i ][ j ], and omi_mask_height[ i ][ j ] specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the j-th object mask in the cropped decoded object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer, relative to the conformance cropping window specified by the active SPS.

The value of omi_mask_left[ i ][ j ] shall be in the range of 0 to ( CroppedWidth / SubWidthC – 1 ), inclusive, CroppedWidth and SubWidthC being associated to the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. When it is not present, the value of omi_mask_left[ i ][ j ] is inferred to be 0.

The value of omi_mask_top[ i ][ j ] shall be in the range of 0 to ( CroppedHeight / SubHeightC – 1 ), inclusive, CroppedHeight  and SubHeightC  being associated to the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. When it is not present, the value of omi_mask_top[ i ][ j ] is inferred to be 0.

The value of omi_mask_width[ i ][ j ] shall be in the range of 0 to ( CroppedWidth / SubWidthC − omi_mask_left[ i ][ j ] ), inclusive. When it is not present, the value of omi_mask_width [ i ][ j ] is inferred to be ( CroppedWidth / SubWidthC − omi_mask_left[ i ][ j ] ).

The value of omi_mask_height[ i ][ j ] shall be in the range of 0 to ( CroppedHeight / SubHeightC − omi_mask_top[ i ][ j ] ), inclusive. When it is not present, the value of omi_mask_height [ i ][ j ] is inferred to be ( CroppedHeight / SubWidthC − omi_mask_top[ i ][ j ] ).

The identified object mask is within a bounding box containing luma samples with horizontal coordinates from SubWidthC * ( ConfWinLeftOffset + omi_mask_left[ i ][ j ] ) to SubWidthC * ( ConfWinLeftOffset + omi_mask_left[ i ][ j ] + omi_mask_width[ i ][ j ] ) − 1, inclusive, and vertical coordinates from SubHeightC * ( ConfWinTopOffset + omi_mask_top[ i ][ j ] ) to SubHeightC * ( ConfWinTopOffset + omi_mask_top[ i ][ j ] + omi_mask_height[ i ][ j ] ) − 1, inclusive.

Variable pI[ i ] [ x ][ y ] is the decoded value of the sample at the relative sample location (x, y) in the cropped object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The following process is to determine mask area in a auxiliary picture.

for( i = 0; i < omi_num_aux_pic_layer; i++ )
		for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ )	
			if( pI[ i ][ x ][ y ]  = =  omi_aux_sample_value [ i ][ j ]
					&&  x  >=  omi_mask_left[ i ][ j ]
					&&  x < omi_mask_left[ i ][ j ] + omi_mask_width[ i ][ j ]	(xx)
					&&  y  >=  omi_mask_top[ i ][ j ]
					&&  y < omi_mask_top[ i ][ j ] + omi_mask_height[ i ][ j ] )
				the sample at location (x, y)
			
				 is associated with the object mask with the identifier of 
				maskId[ i ][ j ]

omi_mask_confidence[ i ][ j ] specifies the degree of confidence associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer, in units of 2-( omi_mask_confidence_length_minus1 + 1 ), such that a higher value of omi_mask_confidence[ i ][ j ] indicates a higher degree of confidence. The length of the omi_mask_confidence[ i ][ j ] syntax element is omi_mask_confidence_length_minus1 + 1 bits.

omi_mask_depth[ i ][ j ] specifies the object depth associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. A smaller value of omi_mask_depth indicates a shorter distance to the object. The length of the omi_mask_depth[ i ][ j ] syntax element is omi_mask_depth_length_minus1 + 1 bits.

omi_mask_label[ i ][ j ] specifies the contents of the label associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The length of the omi_mask_label[ i ][ j ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

  - Modality information SEI message

    - Modality information SEI message syntax

| modality_info( payloadSize ) { | Descriptor |
| --- | --- |
| mi_modality_info_cancel_flag | u(1) |
| if(!mi_modality_info_cancel_flag) { |  |
| mi_modality_info_persistence_flag | u(1) |
| mi_modality_type | u(5) |
| mi_spectrum_range_present_flag | u(1) |
| if( mi_spectrum_range_present_flag ) { |  |
| mi_min_wavelength_mantissa | u(11) |
| mi_min_wavelength_exponent_plus15 | u(5) |
| mi_max_wavelength_mantissa | u(11) |
| mi_max_wavelength_exponent_plus15 | u(5) |
| } |  |
| mi_modality_type_extension_bits | ue(v) |
| if(mi_modality_type_extension_bits > 0 ) |  |
| mi_reserved_modality_type_extension | u(v) |
| } |  |
| } |  |

    - Modality information SEI message semantics

The modality information SEI message provides information about the source of optical radiation (such as visible light, infrared, or ultraviolet) used for generating the associated pictures and the wavelength of the spectrum band. As pictures of different modality types serve different purposes, the information conveyed in this SEI message can be used by a receiver to determine the purpose of the associated pictures.

NOTE 1– The interpretations of mi_modality_type and the wavelength of the spectrum band associated with mi_modality_type are specified by reference to the division of optical radiation specified in ISO 20473:2007.

mi_modality_info_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous modality information SEI message in output order. mi_modality_info_cancel_flag equal to 0 indicates that modality information follows.

mi_modality_info_persistence_flag specifies the persistence of the modality information SEI message for the current layer.

mi_modality_info_persistence_flag equal to 0 specifies that the modality information SEI message applies to the current decoded picture only.

mi_modality_info_persistence_flag equal to 1 specifies that the modality information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

– A picture in the current layer in an AU associated with a modality information SEI message is output that follows the current picture in output order.

mi_modality_type indicates the type of modality of the associated pictures as specified in Table 1. When not present, the value of mi_modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification. When mi_modality_type is equal to 2 or 3, decoders shall ignore vui_colour_primaries, vui_transfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag indicated in the VUI parameters

Table 1– Mapping of mi_modality_type to the type of picture modalities

| mi_modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Ultraviolet Picture |
| 4..31 | Reserved for future use |

NOTE 2– When a reserved value of mi_modality_type is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by mi_modality_type being equal to that value or any one of a set of values including that value.

mi_spectrum_range_present_flag equal to 1 specifies that the spectrum band of the optical radiation wavelength represented by the associated pictures is present in the modality information SEI message. mi_spectrum_range_present_flag equal to 0 specifies that the spectrum band of the optical radiation wavelength represented by the associated pictures is not present in the modality information SEI message.

mi_min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When mi_min_wavelength_mantissa is equal to 0 or is not present, the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures is unknown or unspecified or determined by other means not specified in this Specification

mi_min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When mi_min_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of mi_min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures is derived as follows:

MinWavelength = mi_min_wavelength_mantissa * 10 mi_min_wavelength_exponent_plus15-15

mi_max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When mi_max_wavelength_mantissa is equal to 0 or is not present, the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures is unknown or unspecified or determined by other means not specified in this Specification.

mi_max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When mi_max_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of mi_max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures is derived as follows:

MaxWavelength = mi_max_wavelength_mantissa * 10 mi_max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

mi_modality_type_extension_bits equal to 0 specifies that mi_reserved_modality_type_extension is not present. mi_modality_type_extension_bits greater than 0 specifies the length, in bits, of mi_reserved_modality_type_extension.

The value of mi_modality_type_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for mi_modality_type_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of mi_modality_type_extension_bits in the range of 0 to 2 048, inclusive, to appear in the syntax.

mi_reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of mi_reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of mi_reserved_modality_type_extension is equal to mi_modality_type_extension_bits.

  - Text description information SEI message

    - Text description information SEI message syntax

| text_description( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_id | u(14) |
| txt_cancel_flag | u(1) |
| if( !txt_cancel_flag ) { |  |
| txt_persistence_flag | u(1) |
| txt_descr_purpose | u(8) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |

    - Text description information SEI message semantics

The text description information SEI message provides text description about one or more pictures.

txt_descr_id indicates the identifier value of this text description information SEI message. The value of txt_descr_id shall be in the range of 1 to 16383, inclusive. Value 0 is reserved.

txt_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_id in output order that applies to the current layer. txt_cancel_flag equal to 0 indicates that text description information follows.

txt_persistence_flag specifies the persistence of the text information description SEI message for the current layer.

txt_persistence_flag equal to 0 specifies that the text description information applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text description information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same txt_descr_id is output that follows the current picture in output order.

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table xx. The value of text_descr_purpose shall be in the range of 0 to 5, inclusive. Values in the range of 6 to 255, inclusive, for text_descr_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of text_descr_purpose in the range of 0 to 255, inclusive.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6..255 | Reserved |

txt_num_strings_minus1 plus 1 indicates the number of entries for txt_descr_string_lang[ i ] and txt_descr_string[ i ] that follow.

txt_descr_string_lang[ i ] specifies the language of the txt_descr_string[ i ]. The language of the txt_descr_string[ i ]  shall be given by a language tag as defined by IETF RFC 5646. The length of txt_descr_string_lang[ i ]  shall be in the range of 0 to 49, inclusive.

txt_descr_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the txt_descr_purpose.

When txt_descr_purpose is equal to 0, the interpretation of what information is conveyed in the txt_descr_string is application-defined.

When txt_descr_purpose is equal to 1, the txt_descr_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 2, the txt_descr_string[ i ] specifies, when not a null string, AI marking information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 2 the string can contain information about the machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

When txt_descr_purpose is equal to 3, the txt_descr_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 4, the txt_descr_string[ i ] specifies content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 5, txt_descr_string[ i ] contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

# In the syntax table of subclause D.2.1, add payloadType values 215 to 219 to a future edition or ammendment of Recommendation ITU-T H.266 | International Standard ISO/IEC 23090-3 as follows:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| SeiExtensionBitsPresentFlag = 0 |  |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| … |  |
| else if( payloadType = = 213 ) |  |
| sei_processing_order( payloadSize ) |  |
| else if( payloadType = = 214 ) |  |
| processing_order_nesting( payloadSize ) |  |
| else if( payloadType = = 215 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| encoder_optimization_info( payloadSize ) |  |
| else if( payloadType = = 216 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| source_picture_timing_info( payloadSize ) |  |
| else if( payloadType = = 217 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| object_mask_info( payloadSize ) |  |
| else if( payloadType = = 218 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| modality_info( payloadSize ) |  |
| else if( payloadType = = 219 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| text_description( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 4 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 210 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_characteristics( payloadSize ) |  |
| else if( payloadType = = 211 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_activation( payloadSize ) |  |
| else if( payloadType = = 214 ) |  |
| processing_order_nesting( payloadSize ) |  |
| else if( payloadType = = 215 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| encoder_optimization_info( payloadSize ) |  |
| else if( payloadType = = 217 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| object_mask_info( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| if( SeiExtensionBitsPresentFlag ¦ ¦ more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| sei_reserved_payload_extension_data | u(v) |
| sei_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| sei_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

# Text to be added to a future edition or ammendment of Recommendation ITU-T H.266 | International Standard ISO/IEC 23090-3:

### Use of the source picture timing SEI message in VVC

For purposes of interpretation of the source picture timing SEI message, the following variable is specified:

–	TemporalId is set equal to TemporalId.

# Text to be added to a future edition or ammendment of Recommendation ITU-T H.265 | International Standard ISO/IEC 23008-2:

### Use of the source picture timing SEI message in HEVC

For purposes of interpretation of the source picture timing SEI message, the following variable is specified:

–	TemporalId is set equal to TemporalId.

# Text to be added to a future edition or ammendment of Recommendation ITU-T H.264 | International Standard ISO/IEC 14496-10:

### Use of the source picture timing SEI message in AVC

For purposes of interpretation of the source picture timing SEI message, the following variable is specified:

–	If the bitstream conforms to any of the profiles defined in Annex G, H, I, or J, TemporalId is set equal to temporal_id.

–	Otherwise (the bitstream conforms to a profile defined in Annex A), TemporalId is set equal to ( nal_ref_idc = = 0 ? 1 : 0 ).