| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 14–24 April 2024 | Document: JVET-AH2023-v1 |
| --- | --- |

| Title: | Exploration Experiments on Neural Network-based Video Coding (EE1) |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, R. Chang, F. Galpin, Y.Li, D. Rusanovskyy, M. Santamaria, J. Ström, and Z. Xie | Email: | elena.alshina@huawei.com renjiechang@tencent.com franck.galpin@interdigital.com yue.li@bytedance.com dmytror@qti.qualcomm.com maria.santamaria_gomez@nokia.com jacob.strom@ericsson.com xiezhihuang@oppo.com |
| Source: | EE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes Exploration Experiment 1 (EE1) tests to be performed between the JVET-AH and JVET-AI meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performances and complexity aspects.

# Introduction

Code base for the test should be NNVC9.0, anchor is default configuration of NNVC-9.0 (NN-Intra and LOP-3 filter enabled). NNVC common test conditions, results and complexity reporting template must be used.

For proposals in filter, Intra and super-resolution categories are encouraged to use existing AhG11 training set. Proponents in NN-Inter category encouraged to use extended training set. In all tests training set must be clearly specified.

For tests competing with technologies in NNVC-9 it is recommended to configure the proposed solution to have a complexity close to related NNVC(LOP/HOP), but not exceeding:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- Number of Parameters EE1 test   Number of Parameters NNVC (if possible).

NN architecture provided in this test description should not be changed, beside minor adjustment for parameters (such as channels number) in order to meet recommendation above. Exact parameters settings can be specified by 2nd AhG11/14 teleconference June 14.

# List of tests

This round of EE1 tests will include:

- EE1-1: LOP in-loop filter

  - EE1-1.1 – LOP with trainable DCT

    - JVET-AH0207 EE1 related: LOP input adjustment with trainable input transform [Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

  - EE1-1.2 – Partial Convolution and Over-Parameterization

    - JVET-AH0077 AHG11: A Low-Complexity Neural Network Loop Filter based on Partial Convolution and Over-Parameterization [A. Li, C. Zhu, L. Luo (UESTC), Y. Huo, Y. Liu (Transsion)]

  - EE1-1.3 – simplified residual groups with attention

    - JVET-AH0196 [AhG11] On Low Complexity Operational Point for In-Loop Filtering [T. Ryder, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

EE1-1.4 – content adaptive VLOP

    - JVET-AH0096 EE1-1.4: Content-adaptive loop-filter, [M.Santamaria, R. Yang, F. Cricri, M. M. Hannuksela, D. Bugdayci Sansli, A. Hallapuro, H. R. Tavakoli, J. Lainema, H. Zhang (Nokia)]

    - JVET-AH0051 EE1-5: Study of the NN architecture at Very Low Operational Point [D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

- EE1-2: HOP in-loop filter

  - EE1-2.1 – HOP with transformers and included luma/chroma balance

    - JVET-AH0205, EE1-2.3: Integer implementation of HOP In-loop filter with Transformer blocks and Attention blocks

    - JVET-AH0206 EE1 related: Additional inference test for EE1-2.3 to adjust luma-chroma balance [Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

- EE1-3: NN-inter prediction

  - EE1-3.1 – NN-Inter trained with extended training set

    - JVET-AH0107 EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement [W. Bao, N. Fu, X. Chen, J. Jia, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent)]

- EE1-4: NN-super-resolution

  - EE1-4.1 – NNSR multiple scaling ratios

    - JVET-AH0100 EE1-related: Multiple scaling ratios coding for NNVC with NNSR [Z. Lv, C. Zhou (vivo)]

EE1-4.1 – NNSR with improved training

    - JVET-AH0312 EE1-4 related: On training data of super resolution [C. Lin, Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance)] [late]

- EE1-5. Neural network based Intra coding

  - EE1-5.1 – NN-Intra with ISP

    - JVET-AH0165 AHG11: Combination of the neural network-based intra prediction mode and ISP [T. Dumas, F. Galpin, P. Bordes (InterDigital)]

# Tests description

## EE1-1: LOP in-loop filter

### Test EE1-1.1. JVET-AH0207 EE1 related: LOP input adjustment with trainable input transform [Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

A unified filter architecture for the Low-performance Operation Point (LOP) was defined in JVET-AF0043. Updated training strategy and configuration were presented in JVET-AH0042. Complete inference results for LOP training are reported in JVET-AH0014.

Based on the architecture of the LOP network, input transform with DCT were introduced in JVET-AG0069 and this model has improved the coding gain for RA luma over -0.4 percent compared to LOP with a new two-stage data extraction. This contribution aims at improving the coding gain with a trainable input transform and with the training on the existing LOP2 dataset.

It was commented that this would also be a reasonable extension on top of the new LOP model from EE1-1.2 which uses a DCT.

In this EE test, the DCT component will be replaced by the trainable input transform on top of the LOP3 network, and the network architecture will be adjusted accordingly. The configuration and parameters of the network are subject to further optimization.

### Test EE1-1.2.  JVET-AH0077 AHG11: A Low-Complexity Neural Network Loop Filter based on Partial Convolution and Over-Parameterization [A. Li, C. Zhu, L. Luo (UESTC), Y. Huo, Y. Liu (Transsion)]

This contribution proposes the use of partial convolution to reduce the inference complexity of neural-network-based loop filtering. To mitigate the performance loss associated with reducing complexity, this contribution further proposes an over-parameterization training strategy to improve performance while keeping complexity unchanged. LOP1 filter is performed on top of NNVC-8.0 with NnIntraPred enabled as a comparison baseline. Using block-level KMAC/Pixel as the complexity metric, the baseline model has a worst-case complexity of 16.2 KMAC/Pixel. The proposed low-complexity neural network loop filter has about 15.3 KMAC/Pixel, representing approximately a 5.5% complexity reduction. Additionally, this contribution significantly reduces the number of convolutional layers by about 24%. The new proposed model is implemented on top of the NNVC-8.0.

It was commented that from the information brought it is difficult to judge the benefit. Comparable training procedure should be used, should be based on new LOP3, also results with comparable complexity numbers.

Investigate in EE.

### Test EE1-1.3.  JVET-AH0196 [AhG11] On Low Complexity Operational Point for In-Loop Filtering [T. Ryder, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

In this test, a study on NN architecture proposed in JVET-AH0196 for LOP and VLOP is conducted. A candidate architecture for this test is shown in Figure EE1-3.a. It comprises utilization of unified BB across filter architecture and use of Residual Groups with Attention Block, shown in Figure EE1-3.b. The architecture to be modified to achieve target complexity levels for LOP and VLOP and better performance/complexity trade-off. Preliminary parameters of the NN architecture are shown below:

LOP:    { d1=16, d2=8, d3=4, d4=d5=2, Cy=28, Cuv=14, Ny=4, Nuv=2, nBBy=4, nBBuv=2}.

VLOP: { d1=4, d2=2, d3=1, d4=d5=1, Cy=16, Cuv=8, Ny=4, Nuv=2, nBBy=4, nBBuv=2}.

a.                                                                                           b.

Figure 1 Candidate EE1-1.3 architecture (a) and Residual Group (b).

Training to follow LOP training strategy and Stage 3 LOP dataset to be used. Training process may be modified to reflect change in the architecture and improve performance.

Training: Following LOP training strategy, Stage 3 dataset.

Inference: NNVC 9.0 software or latter, NNVC CTC, following LOP and VLOP anchor settings.

Proponent: Qualcomm

Crosscheck: TBD.

### Test EE1-1.4. Content Adaptive Loop-Filter for Very Low Operational Point

The approach of neural network weights adaptation, originally introduced in JVET-AH0096 for LOP architecture, will be applied to Very Low Operational Point filter. A set of multiplier parameters are overfitted to the coded test material. The generated parameter updates are coded with the MPEG-NNR standard and signalled within a NNFU APS.

Training: modified overfitting process adapted for VLOP architecture.

Inference: The NNVC-9.0 (supporting NNFU APS and other changes), NNVC CTC.

Proponent: Joint test of Nokia, Qualcomm

## EE1-2: HOP in-loop filter

### Test EE1-2.1  JVET-AH0206 EE1 related: Additional inference test for EE1-2.3 to adjust luma-chroma balance [Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

A unified filter architecture for the High-performance Operation Point (HOP) was defined in JVET-AD0380. Updated training strategy and configuration were presented in JVET-AG0041, and group convolution, rank reduction and flipping were adopted into HOP in JVET-AG0174. Complete inference results for HOP training are reported in JVET-AH0014.

Based on the architecture of the HOP network, Transformer blocks were introduced in JVET-AF0158, and integer implementation of the Transformer model was described in JVET-AH0205. This model has improved the coding gain significantly compared to HOP. This contribution aims to rebalance the luma-chroma coding gain during inference by adjusting chroma offset.

In this EE test, the issues with the complexity assessment for the full transformer model will be further addressed. In addition, the luma-chroma balance will be adjusted through training and with chroma offset. Other techniques of adjusting the luma-chroma balance may also be introduced in this test.

## EE1-3: NN-inter prediction

### Test EE1-3.1  JVET-AH0107 EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement [W. Bao, N. Fu, X. Chen, J. Jia, Z. Chen (Wuhan Univ.), Z. Liu, X. Xu, S. Liu (Tencent)]

This test is based on JVET-AH0107, aiming at network quantization and training dataset extension. Specifically, this test will implement network quantization, to align the coding performance between float inference and integer inference. Additionally, this test will continue to extend the existing training dataset under NNVC standardization.

### The test will be on top of the low operation point deep reference frame generation (LDRF) and high operation point deep reference frame generation (HDRF). The network architectures of both LDRF and HDRF are shown in Figure x.

Figure x network architecture of EE1-3.1

Training: Training strategy from JVET-AH0107.

Inference: The NNVC 9.0 software is used for inference under NNVC RA configuration.

Proponent: Wuhan University and Tencent.

Cross-check: Inference and training cross-checks are planned.

### Cross-checker: TBA.

## EE1-4: NN-super-resolution

### Test EE1-4.1  JVET-AH0100 EE1-related: Multiple scaling ratios coding for NNVC with NNSR [Z. Lv, C. Zhou (vivo)]

This test aims to investigate a more generic and flexible method for NNVC. Two subtests will be conducted:

- EE1-4.1.1: Adaptive resolution strategy to support multi-scaling ratios (s=2 && s=1.5) with RPR.

- EE1-4.1.2: Unified NNSR for adaptive resolution strategy (EE1-4.1.1) to support multi-scaling ratios (s=2 && s=1.5).

A candidate architecture is shown in Figure EE1-4.1, while the parameters may be changed to reduce complexity. In addition, loss function will be modified by considering subjective quality (introducing SSIM in the loss metric, for example).

Figure x Network structure of EE1-4.1

Inference: The NNVC 9.0 software is used for inference.

Proponent: vivo.

Cross-check: Inference and training cross-checks are planned.

Cross-checker: TBA.

### Test EE1-4.2   JVET-AH0312 EE1-4 related: On training data of super resolution [C. Lin, Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance)] [late]

In this test, a study on low complexity super resolution will be conducted. The candidate network architecture and preliminary parameters for this test are shown in Figure EE1-4.2. The network architecture and parameters may be modified to achieve better performance/complexity trade-off. Additionally, training parameters and loss function may be modified to improve coding performance and subjective quality.

Rec [3]

CONV 33,d1

CONV

1  1, d6

PReLU

Back Bone Block, CY

Back Bone Block, CY

...

PReLU

CONV

3  3, 4

Pixel Shuffle

Rec Y

Rec UV [2]

RecEXT UV [2]

RecEXTY

2

NY

sepCONV

13 CYCY21

sepCONV

31 CY21CY

CONV

1  1, CY

Back Bone Block, CUV

Back Bone Block, CUV

...

PReLU

CONV

3  3, 2

sepCONV

13 CUVCUV21

sepCONV

31 CUV21CUV

CONV

1  1, CUV

NC

Pred [3]

CONV  33,d2

BS [3]

QPbase

[1]

QPslice[1]

IPB [1]

CONV  11,d2

CONV  11,d2

CONV  11,d2

CONV  11,d2

[C,h,w]

CONV 1  1 C1+C

CONV 1  1 CC1

sepCONV 1  3 CC21

sepCONV 3  1 C21C

PReLU

Back Bone Block C

[C,h,w]

RPR Upsampling

CONV 1  1 CC

Figure x Network structure of EE1-4.2

Inference: The NNVC 9.0 software is used for inference.

Proponent: Bytedance.

Cross-check: Inference and training cross-checks are planned.

## Neural network based Intra coding

Test EE1-5.1  JVET-AH0165 AHG11: Combination of the neural network-based intra prediction mode and ISP [T. Dumas, F. Galpin, P. Bordes (InterDigital)]

This contribution proposes a combination of the Neural Network (NN)-based intra prediction mode and ISP inside NNVC. In addition, the objective function for training a NN for intra prediction now involves the SATD and the SAD between a given original block and its neural network prediction as this objective function has yielded mean BD-rate gains when combining the NN-based intra prediction mode and ISP.

NNVC-8.0 involving LOP-2, the low-complexity NN-based intra prediction mode, and the proposed combination of the NN-based intra prediction mode and ISP with respect to NNVC-8.0 involving LOP-2 and the default NN-based intra prediction mode (NNVC-8.0 CTC) gives:

- RA:  -0.13%, -0.78% and -0.76% BD-rate PSNR Y, U and V gains, respectively for a complexity of 112% and 113% at encoder and decoder respectively

- AI:   -0.17%, -0.63% and -0.56% BD-rate PSNR Y, U and V gains, respectively, for a complexity of 155% and 115% at encoder and decoder respectively.

NNVC-8.0 involving LOP-2, the low-complexity NN-based intra prediction mode, and the proposed combination of the NN-based intra prediction mode and ISP with respect to NNVC-8.0 involving LOP-2 and the low-complexity NN-based intra prediction mode provides:

- RA:  -0.25%, -0.85% and -0.80% BD-rate PSNR Y, U and V gains, respectively for a complexity of 107% and 103% at encoder and decoder respectively

- AI:   -0.54%, -0.93% and -0.93% BD-rate PSNR Y, U and V gains, respectively, for a complexity of 151% and 109% at encoder and decoder respectively.

Number of operations is decreased relative to current NN intra pred., but decoder runtime is increased due to more frequent usage.

Interesting gain and reduction in computation, but encoder runtime increased significantly, due to necessity of more checks.

In this EE test the  focus should be reduction of encoding time.

Figure 1: prediction of the current w×h block from its context of reference samples via the neural network fh, w . , θh,w, parametrized by θh,w, belonging to the low-complexity NN-based intra prediction mode. Here, w=8 and h=4. The definitions of grpIdx1, grpIdx2, repIdx, “preprocessing”, and “postprocessing” can be found in JVET-AG2019.

Table 1: architecture of each neural network belonging to the low-complexity NN-based intra prediction mode.

| architecture |  | description |
| --- | --- | --- |
| Number of hidden layers |  | For 16x16, 3. For the other six NNs, 2. |
| Number of neurons per hidden layer |  | For 4x4, 8x4, and 16x16, 1216 for the last hidden layer, and 640 for the other hidden layers. For the other four NNs, 1216. |
| Percentage of non-zero weights | hidden layers | 5% |
|  | layer returning repIdx | For 4x4, 10%. For the other six NNs, 50% |
|  | layer returning the predicted block | 50% |
|  | layer returning grpIdx1 and grpIdx2 | 100% |

Worst-case complexity of the low-complexity NN-based intra prediction mode: 4.8 kMAC/pixel.

Number of parameters of this mode (7 NNs together): 1333799.

# Timeline

T1 - 2 weeks after JVET-AG meeting (10-May-2024): EE description (JVET-AG2023) finalized and uploaded. NNVC-8.0 software is available, including anchor performance.

T2 – 3 weeks after JVET-AG meeting (17-May-2024) 1st teleconference. EE1-1.0 LOP training is over. Results discussed during teleconference. Training of EE1-1.1,2,3 started. EE1-4.1 adaptive resolution and QP selection strategy for RPR ready.

T2 – 7 weeks after JVET-AG meeting (14-June-2024) 2nd teleconference. Final setting for parameters to be announced, partial results discussed, combinational tests (if any) decided.

T3 – 2 weeks before T5 (27-June-2023): EE1 software is frozen (write access closed on 29-Mar-2024).

T4 – 3 days before T5 (10-July-2024): Cross-checkers report status to EE1 coordinators (sending e-mail).

T5 –10-July-2024: EE1 summary is uploaded as input contribution.

# SW location

https://vcgit.hhi.fraunhofer.de/jvet-ah-ee1/VVCSoftware_VTM/-/tree/EE1-X.X

# References

[1] E. Alshina, R.-L. Liao, S. Liu, A. Segall  Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AG2016.

[2] E. Alshina, F. Galpin BoG on NNVC trainign material JVET-AH0354