| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0011-v1 |
| --- | --- |

| Title: | JVET AHG report: Neural network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Shan Liu Andrew Segall Franck Galpin Junru Li Ru-Ling Liao Dmytro Rusanovskyy Maria Santamaria Tong Shao Mathias Wien Ping Wu | Email: | elena.alshina@huawei.com shanl@global.tencent.com asegall@amazon.com franck.galpin@InterDigital.com lijunru@bytedance.com ruling.lrl@alibaba-inc.com dmytror@qti.qualcomm.com maria.santamaria_gomez@nokia.com tshao@dolby.com wien@lfb.rwth-aachen.de pwuzte@gmail.com |
| Source: | AHG11 coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG11: Neural network-based video coding between the 34th meeting held in Rennes and 35th meeting in Sapporo.

# Introduction

The AHG was established with the following mandates:

- Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting.

- Discuss potential refinements of the test conditions for NN-based video coding in JVET-AF2016. Generate and distribute anchor encoding, and develop supporting software as needed.

- Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training.

- Analyze complexity characteristics for technologies under study, including transformers, perform complexity analysis, and develop complexity reductions of candidate technology.

- Finalize and discuss the EE on neural network-based video coding.

- Promote the call for training materials, distribute it, and actively communicate with content owners.

- Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. If possible, prepare encodings with combinations of tools included in the NNVC software for visual quality assessment at the next meeting.

- Coordinate with AHG14 on items related to NNVC software development and studying the impact of training set extension.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email. Four emails were exchanged on the reflector related to the AHG mandates.

## Common Test Conditions

### Document

During the cross-check and study of potential ECM & NNVC tools interaction is was found that one test sequence is different in AhG11 and AhG12 test conditions. One contribution reports this issue and requests adding md5 check sums into NNVC common test condition document.

| JVET-AI0052 | AHG11: Add checksums of sequences to NNVC CTC | J. Pardo (Huawei) |
| --- | --- | --- |

### Anchor Encoding

Anchor for the NN-based video coding activity made available on the Git repository used for the AHG activity: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Anchor%20performance?ref_type=heads  also distributed by AhG14.

## New training materials

JVET group received new training materials are response for the call issued during 33rd meeting. Before inclusion into common test conditions re-training of existing NNVC technology (namely LOP3 filter) has been performed.

| JVET-AI0150 |  | AHG11: Report of retraining LOP3 with new sequences from JVET-AH0120 | D. Liu, J. StrÃ¶m, M. Damghanian, P. Wennersten (Ericsson), D. Rusanovskyy (Qualcomm) |  |
| --- | --- | --- | --- | --- |
| JVET-AI0155 | AhG14: Report of HOP3 training using additional dataset from JVET-AH0120 |  |  | F.Galpin, T. Poirier (InterDigital), |  |

Two tests were conducted including the new dataset and show performance degradation for both LOP and HOP training. The root of the issue was found, and new training is needed to confirm the results.

## Dataset access

It has been reported by multiple proponents trying to retrain models from scratch that the original datasets used to train the current models have changed since their first use. Especially, the BVI and UGC datasets had some sequences missing. It was also noted that some md5sums are incorrect in the provided file. A new round to summarize available content should be discussed in the group.

## EE Coordination

The AHG finalized, conducted, and discussed the EE on NN based video coding. A summary report for the EE is available at this meeting as:

| JVET-AI0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Strom, R. Chang, Z. Xie (EE coordinators) |
| --- | --- | --- |

## Teleconferences

The AHG conducted two joint teleconferences with AHG14 and EE1 during the interim period. The teleconferences were held on May 17 and June 18. In those teleconferences, the following topics were discussed

- HOP-4 integration into NNVC SW

- integration status of NNVC-9,

- final parameter selection for all EE1 tests

For few rounds of EE1 activity group followed recommendation to configure EE1 test with complexity close but not exceeding the complexity of existing NNVC tools proposal targets to replace (anchor). If test configured following recommendation and shows performance gain then the complexity – performance trade-off obviously better than for anchor. During 34th meeting of JVET two changes have been adopted into NNVC: 1) wide activation, 2) attention block. Each individual proposal follows recommendation: lower than NNVC8.0 complexity and performance gain. But after merging two proposals together as HOP4 there was still gain, but complexity increased compared to the anchor (HOP3). In order not to delay EE1 tests NNVC-9.1 was release with HOP4. During teleconference it was suggested to try different configuration of HOP4 which would not exceed complexity of HOP3 (and so follow practice in AhG11/EE1).

This activity resulted in new proposal, which reports both gain and complexity reduction compared to HOP3.

| AI0172 | m68570 | EE1(AhG 11): On the complexity adjustment of HOP4 | Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm), F. Galpin (Interdigital), Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), |
| --- | --- | --- | --- |

It is recommended to review this proposal as part of EE1 study.

## Performance Evaluation

The performance and complexity of NN-based tools available in NNVC SW is summarized in the table below.  All test data provided by AhG14. Encoding and decoding run time is very dependent on cluster used for simulation. Run time data in this table are all from InterDigital.

NN-Intra is enabled in all tests.

LOP filter evolves toward better performance, computational complexity (measured in kMAC/pxl) constantly reduces. Decoding run time of LOP3 is slightly higher than LOP2 (likely because of 4 higher size of the model). More than 1% extra compression gain is possible with content adaptive version of neural-network based filter.

| Test vs NNVC (configured as VTM) | Random Access cfg |  |  |  |  | kMAC/pxl |  |  |  | Param (Mprm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Total | Filter | Intra | SR | Total | Filter | Intra | SR |
| NN-Intra & LOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-9.1(LOP3) | -7.3% | -13.1% | -11.3% | 1.2 | 81 | 24.8 | 16.9 | 7.9 | 0 | 1.7 | 0.21 | 1.5 | 0 |
| NNVC-8.0(LOP2) | -6.9% | -13.2% | -12.1% | 1.2 | 73 | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-7.1(LOP2) | -6.9% | -13.2% | -12.1% | 1.3 | 86 | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-9.1(LOP2CA) | -8.2% | -16.5% | -15.5% | 2.5 | 69 | 25.5 | 17.6 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NN-Intra & HOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-9.1(HOP4) | -14.1% | -19.2% | -19.6% | 2.9 | 1447 | 484 | 476 | 7.9 | 0 | 3.0 | 1.44 | 1.5 | 0 |
| NNVC-8.0(HOP3) | -13.7% | -13.9% | -14.5% | 2.5 | 1092 | 474 | 466 | 7.9 | 0 | 2.9 | 1.40 | 1.5 | 0 |
| NNVC-7.0(HOP2) | -13.6% | -12.5% | -14.2% | 4.1 | 2071 | 485 | 477 | 7.9 | 0 | 3.0 | 1.50 | 1.5 | 0 |
| NN-Intra & LOP filter & adaptive resolution coding (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-8.0 RPR | -7.5% | -10.9% | -9.7% |  |  | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-8.0 NNSR | -7.8% | -11.8% | -10.5% |  |  | 45.3 | 17.1 | 7.9 | 20.3 | 1.8 | 0.21 | 1.5 | 0.1 |
| NN-Intra & VLOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-9.1(VLOP) | -5.3% | -5.4% | -5.2% | 1.2 | 40 | 13.0 | 5.12 | 7.9 | 0 | 1.5 | 0.02 | 1.5 | 0 |

HOP filter performance also improves constantly. Unfortunately, both encoding and decoding run-time increase significantly from HOP3 to HOP4, partially due to higher computational complexity, but mostly due to new operations introduced in attention block.

The third tool added to LOP configuration – adaptive resolution – improves performance by 0.6% if RPR filter is used for resampling and by 0.9% in NNSR is used.

BD-rate reported is measured based on PSNR. In MS-SSIM gain is 1-2% higher.

More details expected in AhG14 report.

# Input contributions

There are 34 input contributions related to the AHG mandates. The list of input contributions is provided below.

| Reporting |  |  |  |
| --- | --- | --- | --- |
| JVET-AI0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Strom, R. Chang, Z. Xie (EE coordinators) |  |
| JVET-AI0041 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |  |
| NNVC test and training conditions |  |  |  |
| JVET-AI0052 | AHG11: Add checksums of sequences to NNVC CTC | J. Pardo (Huawei) |  |
| JVET-AI0235 | Crosscheck of JVET-AI0052 (AHG11: Add checksums of sequences to NNVC CTC) | F. Galpin (InterDigital) |  |
| JVET-AI0150 | AHG11: Report of retraining LOP3 with new sequences from JVET-AH0120 | D. Liu, J. Ström, M. Damghanian, P. Wennersten (Ericsson), D. Rusanovskyy (Qualcomm) |  |
| JVET-AI0155 | AhG14/AHG11: Report of HOP3 training using additional dataset from JVET-AH0120 | F.Galpin, T. Poirier (InterDigital), |  |
| EE1 contributions |  |  |  |
| JVET-AI0068 | EE1-1.2: Partial Convolution and Over-Parameterization | A. Li, J. Chi, C. Zhu, L. Luo, H. Guo(UESTC), Y. Huo, Y. Liu(Transsion) |  |
| JVET-AI0074 | EE1-4.1: Multiple scaling ratios coding for NNVC with NNSR | Z. Lv, C. Zhou (vivo) |  |
| JVET-AI0093 | EE1-4.2: On simplified super resolution model | C. Lin, Y. Li, J. Li, K. Zhang, L. Zhang (Bytedance) |  |
| JVET-AI0107 | EE1-1.5: VLOP using LOP3 architecture | D. Liu, J. Ström, M. Damghanian, P. Wennersten (Ericsson), D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm) |  |
| JVET-AI0130 | EE1-5.1 : combination of the neural network-based intra prediction mode and ISP | T. Dumas, F. Galpin, P. Bordes (Interdigital) |  |
| JVET-AI0157 | EE1-1.4: Content-adaptive VLOP-filter | R. Yang, M. Santamaria, F. Cricri, M.M. Hannuksela, D.Bugdayci Sansli, A.Hallapuro, J.Lainema, H. Zhang(Nokia), D.Rusanovskyy, Y.Li, M.Karczewicz(Qualcomm) |  |
| JVET-AI0172 | EE1(AhG 11): On the complexity adjustment of HOP4 | Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm), F. Galpin (Interdigital), Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), |  |
| JVET-AI0173 | EE1-1.1: LOP input adjustment with trainable components | Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm), |  |
| JVET-AI0176 | EE1-2.1: On the detailed analysis of integer Transformer ILF with luma-chroma component balancing and complexity trade-off | Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm) |  |
| JVET-AI0221 | EE1-1.3: On Low Complexity Operational Point for In-Loop Filtering | S.Eadie, T. Ryder, Y.Li, D.Rusanovskyy, M.Karczewicz (Qualcomm) |  |
| JVET-AI0239 | EE1-3: Deep Reference Frame Generation for Inter Prediction | S.Y.Lee, D.W.Kang, Y.H.Kim, C.J.Shin, S.J.Lee |  |
| New NNVC (AhG11 or EE1 related) contributions |  |  |  |
| JVET-AI0089 | AHG11: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding | Q. Qin, C. Jung (Xidian Univ.) |  |
| JVET-AI0090 | AHG11: Deep Reference Frame Generation for VVC Inter Coding | H. Zhang, C. Jung, Q. Qin (Xidian Univ.) |  |
| JVET-AI0092 | AHG11: QP Distance-Based Progressive Learning for Enhancing HOP In-Loop Filter | P. Fu, C. Jung, Q. Qin (Xidian Univ.) |  |
| JVET-AI0095 | EE1-related: Improved gain for NN in-loop filters using early cropping | J. Ström, M. Damghanian, D. Liu, P. Wennersten (Ericsson) |  |
| JVET-AI0105 | AHG11: Wavelet transform for super-resolution loss function | Jiedong Ye, Kejun Wu, Qiong Liu |  |
| JVET-AI0111 | AhG11: Content-adaptive LOP3 filter | M. Santamaria, R. Yang, F. Cricri, M. M. Hannuksela, D. Bugdayci Sansli, A. Hallapuro, J. Lainema, H. Zhang (Nokia) |  |
| JVET-AI0134 | AhG11: LOP3 filter with multiscale blocks | R. Yang, F. Cricri, M. Santamaria, H. Zhang, J. Lainema, M.M. Hannuksela(Nokia) |  |
| JVET-AI0151 | Crosscheck of JVET-AI0068 (EE1-1.2: Partial Convolution and Over-Parameterization) | D. Liu (Ericsson) |  |
| JVET-AI0175 | EE1 Related: Block-size invariant implementation of HOP4 | Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm) |  |
| JVET-AI0188 | AhG11: Block based QP information in NN loop filters | F. Galpin, T. Poirier, T. Dumast, P. Bordes |  |
| JVET-AI0204 | EE1-Related: EE1-1.5 with reduced complexity reduced complexity input feature extraction | D.Rusanovskyy, Y.Li, M.Karczewicz (Qualcomm), |  |
| Cross-checks |  |  |  |
| JVET-AI0241 | Crosscheck of JVET-AI0157 (EE1-1.4: Content-adaptive VLOP-filter) | D. Liu (Ericsson) |  |
| JVET-AI0258 | Crosscheck of JVET-AI0093 (EE1-4.2: On simplified super resolution model) | Z. Lv (vivo) |  |
| JVET-AI0264 | Cross-check of JVET-AI0074 (EE1-4.1: Multiple scaling ratios coding for NNVC with NNSR) | C. Lin (Bytedance) |  |
| JVET-AI0273 | Crosscheck of JVET-AI0130 (EE1-5.1: combination of the neural network-based intra prediction mode and ISP) | M. Santamaria (Nokia) |  |
| JVET-AI0278 | Crosscheck of JVET-AI0221 (EE1-1.3: On Low Complexity Operational Point for In-Loop Filtering) | J. StrÃ¶m (Ericsson) |  |
| JVET-AI0283 | Crosscheck of JVET-AI0130 (EE1-5.1: combination of the neural network-based intra prediction mode and ISP) | Y. Li (Bytedance) |  |

# Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

- Continue collecting training materials for neural network-based video coding tool development and investigate training stability.

- Doublecheck and fix training set description on GIT NNVC repository.