| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET-AI0014-v1 |
| --- | --- |

| Title: | JVET AHG report: NNVC software development AhG14 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin, R. Chang, Y. Li, Y. Li, J. N. Shingala, Z. Xie | Tel: Email: | franck.galpin@interdigital.com renjiechang@tencent.com yue.li@bytedance.com yli30@qti.qualcomm.com jay.shingala@ittiam.com xiezhihuang@oppo.com |
| Source: | AHG14 on NNVC software development |  |  |

# Abstract

This report summarizes the activities of the AHG14 on NNVC software development that have taken place between the 34th and 35th JVET meetings.

# Introduction

The mandates for the AhG are as follows:

- Coordinate development of the NNVC software and associated configuration files.

- Prepare and deliver NNVC-9.0 software version (and potential updates),  and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AF2016. Study the impact of the addition of new dataset on the already integrated models.

- Investigate and bridge coding performance of NNVC VTM anchor compared to the latest VTM version and/or VTM anchor in ECM software.

- Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) configurations.

- Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed.

- Coordinate with NNVC algorithm and software description (JVET-AH2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate.

- Coordinate with AHG11 on items related to NNVC activities.

# Software development

## Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

NNVC software is based on VTM-11.0 with enabled MCTF including the update from JVET-V0056, GOP32, enabling deblocking in the RDO and JVET-AH0054 with improved MCTF.

NNVC-9.1 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/Anchor%20performance/NNVC-current_Reporting_Template-with-anchor-data.xlsm?ref_type=heads is used for NNVC performance evaluation.

## Software changes

## NNVC-9.0

Several commits were merged in the NNVC repository.

The following changes were integrated:

- JVET-AH0080 (LOP3): training scripts + models + NNVC changes

- JVET-AH0081 (LOP3): models

- JVET-AH0064: improved MCTF, port from ECM

- JVET-AH0099 (rpr 1.5)

- JVET-AH0051 VLOP

- JVET-AH0096: LOP2 adaptive content: training scripts

- JVET-AH0088: new SADL

The following fixes and clean-up were performed:

- Build for ARM

- Fixes many implicit cast to pass MSVC strict build

- Clean model name

- Add support for Y4M

- Fix memory issue with padding

## NNVC-9.1

The following changes were integrated:

- JVET-AH0096: LOP2 adaptive content: models and NNVC inference

- JVET-AH00189 + JVET-AH0205: HOP4 models + training scripts + NNVC changes

- Extend SPS model ID to support new models

- SADL update for HOP4 support

The following fixes and clean-up were performed:

- Fixes on issues avec NNVC-9.0 integration

- Fixes legacy memory issues from VTM

- JVET-AH0096: fix parameter value for training

## Software version

NNVC-9.1 was tagged May 28th, 2024

NNVC-9.0 was tagged May 13th, 2024

NNVC-8.0 was tagged February 20th, 2024

NNVC-8.0rc3 was tagged February 20th, 2024.

NNVC-7.1 was tagged November 29th, 2023.

NNVC-7.0 was tagged November 3rd, 2023.

NNVC-6.1 was tagged September 25th, 2023 (fix).

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st, 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th, 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th, 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th, 2021 (VTM-11.0 base with MCTF enabled).

# CTC performance

See configurations section for naming convention.

## Comparison to VTM

### NNVC-8.0 VTM vs NNVC-9.1 VTM

AI results are unchanged compared to NNVC-8.0.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.07% | -0.44% | -0.23% | -0.10% | -0.28% | -0.27% | 103% | 100% |
| Class A2 | -0.07% | -0.25% | -0.38% | -0.14% | -0.17% | -0.41% | 100% | 102% |
| Class B | -0.27% | -0.49% | -0.52% | -0.31% | -0.63% | -0.48% | 101% | 98% |
| Class C | -0.01% | -0.22% | 0.00% | -0.03% | 0.03% | -0.03% | 102% | 98% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.12% | -0.36% | -0.29% | -0.16% | -0.29% | -0.30% | 101% | 99% |
| Class D | 0.10% | -0.58% | -0.33% | -0.33% | -1.43% | -0.67% | 99% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.12% | 0.18% | -0.04% | -0.14% | 0.38% | 0.71% | 103% | 112% |
| Class E | -0.85% | -1.11% | -1.17% | -0.57% | -0.79% | -0.59% | 103% | 109% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.36% | -0.52% | -0.82% | 0.23% | -0.42% | -1.37% | 103% | 108% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Note: Results from InterDigital, crosschecked by xxx

### NNVC-9.1 VTM vs NNVC-9.1 anchor

The NNVC-9.1 anchor includes LOP.3 filter and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.81% | -11.81% | -12.53% | -9.44% | -13.31% | -14.42% | 122% | 8345% |
| Class A2 | -7.55% | -12.21% | -8.04% | -8.14% | -12.90% | -7.89% | 120% | #NUM! |
| Class B | -6.90% | -14.15% | -12.10% | -7.52% | -15.40% | -14.41% | 124% | 8291% |
| Class C | -6.53% | -13.48% | -11.91% | -7.52% | -14.12% | -12.83% | 120% | 7846% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -7.31% | -13.11% | -11.32% | -8.03% | -14.14% | -12.69% | 122% | #NUM! |
| Class D | -6.71% | -11.81% | -11.15% | -6.82% | -12.00% | -12.02% | 122% | 7597% |
| Class F | -3.82% | -7.53% | -6.07% | -5.01% | -10.67% | -9.12% | 131% | 3845% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -5.32% | -11.84% | -9.04% | -6.95% | -10.18% | -9.07% | 113% | 6950% |
| Class E | -5.66% | -4.34% | -3.26% | -7.88% | -1.94% | -4.39% | 130% | 8302% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -6.05% | -9.91% | -7.88% | -7.22% | -4.80% | -9.28% | 111% | 6863% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.23% | -14.16% | -14.92% | -9.43% | -16.70% | -16.37% | 177% | 5844% |
| Class A2 | -7.53% | -13.55% | -10.64% | -8.25% | -14.07% | -10.25% | 174% | 5098% |
| Class B | -7.62% | -14.07% | -14.34% | -8.02% | -15.93% | -16.06% | 173% | 4941% |
| Class C | -7.45% | -13.40% | -13.11% | -8.33% | -16.12% | -15.73% | 164% | 3691% |
| Class E | -10.96% | -16.02% | -16.23% | -11.83% | -16.26% | -17.00% | 173% | 5427% |
| Overall | -8.39% | -14.17% | -13.86% | -9.00% | -15.85% | -15.23% | 172% | 4862% |
| Class D | -7.26% | -11.65% | -12.23% | -7.61% | -14.59% | -15.44% | 156% | 3266% |
| Class F | -5.20% | -9.48% | -8.66% | -5.59% | -12.06% | -11.70% | 136% | 4106% |

Note: Results from Interdigital, crosschecked by xxx.

### NNVC-9.1 VTM mode vs NNVC-9.1 HOP.4

The NNVC-9.1 where LOP.3 filter is replaced by HOP.4 and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -15.50% | -13.07% | -21.94% | -16.86% | -18.06% | -24.05% | 309% | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -12.86% | -23.89% | -16.58% | -11.95% | -21.96% | -15.22% | 311% | 157039% |
| Class C | -13.88% | -17.91% | -16.95% | -12.80% | -14.69% | -14.09% | 235% | 119442% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -14.88% | -17.91% | -18.12% | -11.21% | -13.74% | -12.83% | 234% | 125200% |
| Class F | -8.92% | -12.64% | -11.21% | -9.49% | -12.53% | -11.40% | 430% | 74404% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -12.09% | -9.21% | -5.43% | -11.14% | -3.23% | 2.09% | 233% | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -13.33% | -7.49% | -2.98% | -10.68% | 1.01% | 9.81% | 228% | 112300% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -13.50% | -11.49% | -19.38% | -15.11% | -16.89% | -21.60% | 360% | #NUM! |
| Class A2 | -13.07% | -17.24% | -21.11% | -13.81% | -18.17% | -18.36% | 272% | #NUM! |
| Class B | -11.80% | -17.15% | -14.49% | -11.73% | -17.33% | -14.85% | 253% | #NUM! |
| Class C | -12.82% | -14.55% | -16.57% | -12.58% | -13.97% | -15.75% | 192% | #NUM! |
| Class E | -16.51% | -20.23% | -19.51% | -16.28% | -19.24% | -20.02% | 264% | #NUM! |
| Overall | -13.31% | -16.16% | -17.71% | -13.59% | -16.97% | -17.62% | 257% | #NUM! |
| Class D | -12.10% | -13.61% | -17.02% | -11.17% | -13.72% | -15.80% | 171% | #NUM! |
| Class F | -9.63% | -13.10% | -12.71% | -9.43% | -13.31% | -13.59% | 166% | #NUM! |

Note: Results from InterDigital, crosschecked by xxx.

### NNVC-9.1 VTM mode vs NNVC-9.1 VLOP

The NNVC-9.1 where LOP.3 filter is replaced by VLOP and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.10% | -4.74% | -5.50% | -6.36% | -4.97% | -5.27% | 114% | 4133% |
| Class A2 | -5.24% | -4.48% | -3.39% | -5.19% | -4.01% | -1.87% | 115% | 3897% |
| Class B | -5.05% | -5.92% | -5.74% | -5.30% | -5.49% | -4.97% | 118% | 4154% |
| Class C | -5.07% | -6.05% | -5.52% | -5.47% | -5.72% | -4.13% | 114% | 3894% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -5.30% | -5.43% | -5.16% | -5.54% | -5.15% | -4.19% | 116% | 4027% |
| Class D | -5.39% | -5.88% | -6.18% | -5.27% | -4.80% | -4.98% | 115% | 3750% |
| Class F | -2.85% | -4.21% | -3.48% | -3.37% | -4.35% | -2.50% | 120% | 1663% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -3.74% | -4.15% | -2.68% | -4.77% | -6.62% | -3.88% | 112% | 3624% |
| Class E | -3.72% | 0.58% | -0.44% | -4.97% | -1.40% | -3.85% | 113% | 3580% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -4.46% | -2.04% | -0.73% | -5.27% | -5.46% | -3.49% | 108% | 3429% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.49% | -8.00% | -8.23% | -7.78% | -9.35% | -8.33% | 177% | 3252% |
| Class A2 | -6.26% | -7.29% | -6.16% | -6.41% | -6.81% | -4.63% | 187% | 2992% |
| Class B | -6.48% | -7.64% | -8.22% | -6.58% | -7.43% | -7.36% | 185% | 3030% |
| Class C | -6.37% | -7.86% | -7.91% | -6.89% | -8.04% | -7.10% | 170% | 2295% |
| Class E | -9.27% | -8.43% | -7.81% | -9.29% | -7.53% | -6.13% | 172% | 3074% |
| Overall | -7.05% | -7.82% | -7.74% | -7.27% | -7.80% | -6.80% | 178% | 2883% |
| Class D | -6.42% | -7.61% | -8.31% | -6.52% | -7.95% | -8.06% | 153% | 2121% |
| Class F | -4.39% | -6.21% | -5.97% | -4.40% | -5.66% | -4.67% | 136% | 2255% |

Note: Results from InterDigital, crosschecked by XX.

### NNVC-9.1 VTM vs NNVC-9.1 LOP2 adaptive content

xxx

Note: Results from xxx crosscheck by xxx,

### NNVC-9.1 VTM vs NNVC-9.1 NNSR

xxx

Note: Results from xxx crosscheck by xxx,

## Comparison to previous NNVC anchor

### NNVC-8.0 anchor vs NNVC-9.1 anchor

The results reflect LOP/nnvc progress between NNVC 8.0 (as anchor) and NNVC 9.1 (as test).

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.04% | -2.39% | -2.41% | -1.05% | -2.66% | -4.67% | 103% | 109% |
| Class A2 | -0.88% | 0.23% | 1.54% | -1.45% | -1.81% | -0.81% | 105% | #NUM! |
| Class B | -0.61% | -0.82% | 0.14% | -1.08% | -3.42% | -3.23% | 103% | 109% |
| Class C | 0.06% | 1.35% | 2.93% | -0.47% | -1.22% | 0.03% | 102% | 107% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.57% | -0.35% | 0.65% | -0.98% | -2.36% | -2.17% | 103% | #NUM! |
| Class D | 0.62% | 2.93% | 4.18% | -0.61% | -0.26% | 0.55% | 102% | 101% |
| Class F | -0.08% | 2.26% | 3.86% | -0.59% | -0.29% | 1.36% | 101% | 111% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.37% | 2.66% | 7.10% | 0.33% | 7.33% | 10.62% | 106% | 109% |
| Class E | -0.33% | 3.36% | 7.97% | -0.48% | 7.76% | 9.19% | 112% | 116% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.87% | 3.34% | 7.48% | 0.41% | 9.41% | 9.45% | 104% | 105% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.68% | -1.78% | -2.99% | -0.47% | -2.33% | -4.43% | 102% | 107% |
| Class A2 | -0.42% | -1.01% | 0.74% | -0.88% | -2.35% | -1.07% | 102% | 109% |
| Class B | -0.29% | -1.22% | -0.90% | -0.60% | -3.14% | -3.12% | 100% | 103% |
| Class C | 0.09% | 0.39% | 1.59% | -0.41% | -2.35% | -1.49% | 99% | 102% |
| Class E | -0.28% | -2.24% | -1.38% | -1.01% | -3.35% | -3.39% | 100% | 104% |
| Overall | -0.29% | -1.09% | -0.50% | -0.65% | -2.73% | -2.68% | 100% | 105% |
| Class D | 0.43% | 1.23% | 3.33% | -0.17% | -1.25% | -0.11% | 102% | 99% |
| Class F | 0.08% | 1.52% | 1.99% | -0.46% | -1.06% | -0.16% | 100% | 102% |

Note: Results from InterDigital, crosschecked by xxxx.

## Comparison to NNVC-9.1 anchor

### NNVC-9.1 vs NNVC-9.1 HOP.4

The results below reflect LOP/HOP performance differences (LOP set as anchor, HOP as test).

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 | -7.32% | -1.69% |  | -10.91% |  | -8.17% |  | -5.85% |  | -11.57% |  | 254% |  | #NUM! |  |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #DIV/0! |  |
| Class B | -6.44% | -11.72% |  | -5.26% |  | -4.78% |  | -8.28% |  | -0.97% |  | 250% |  | 1894% |  |
| Class C | -7.88% | -5.13% |  | -5.77% |  | -5.70% |  | -0.63% |  | -1.43% |  | 196% |  | 1522% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #DIV/0! |  |
| Class D | -8.77% | -6.96% |  | -8.00% |  | -4.70% |  | -1.95% |  | -1.19% |  | 192% |  | 1648% |  |
| Class F | -5.40% | -5.74% |  | -5.57% |  | -4.77% |  | -2.25% |  | -2.61% |  | 327% |  | 1935% |  |
| Class H | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class B | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class C | -7.16% | 2.91% |  | 3.66% |  | -4.53% |  | 7.72% |  | 11.87% |  | 207% |  | #NUM! |  |
| Class E | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class D | -7.75% | 2.21% |  | 5.28% |  | -3.83% |  | 6.27% |  | 19.44% |  | 206% |  | 1636% |  |
| Class F | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM | EncT | DecT CPU |  |
| Class A1 | -4.68% |  | 3.02% |  | -5.16% |  | -6.22% |  | -0.31% |  | -6.40% | 204% | #NUM! |  |
| Class A2 | -6.01% |  | -3.84% |  | -11.56% |  | -6.02% |  | -4.25% |  | -8.88% | 156% | #NUM! |  |
| Class B | -4.56% |  | -3.45% |  | -0.16% |  | -4.01% |  | -1.72% |  | 1.52% | 146% | #NUM! |  |
| Class C | -5.87% |  | -1.24% |  | -4.07% |  | -4.63% |  | 2.73% |  | -0.06% | 117% | #NUM! |  |
| Class E | -6.28% |  | -5.06% |  | -4.02% |  | -4.96% |  | -3.63% |  | -3.78% | 153% | #NUM! |  |
| Overall | -5.40% |  | -2.22% |  | -4.41% |  | -5.01% |  | -1.24% |  | -2.77% | 150% | #NUM! |  |
| Class D | -5.25% |  | -2.16% |  | -5.67% |  | -3.86% |  | 1.13% |  | -0.90% | 110% | #NUM! |  |
| Class F | -4.76% |  | -4.01% |  | -4.46% |  | -4.06% |  | -1.37% |  | -2.16% | 123% | #NUM! |  |

Note: Results from InterDigital, crosschecked by xxxx.

### NNVC-9.1 vs NNVC-9.1 VLOP

The results below reflect LOP/VLOP performance differences (LOP set as anchor, VLOP as test).

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 2.97% | 8.41% | 8.07% | 3.39% | 10.44% | 10.88% | 94% | 50% |
| Class A2 | 2.52% | 9.31% | 5.15% | 3.20% | 10.92% | 6.59% | 96% | #DIV/0! |
| Class B | 1.98% | 9.72% | 7.32% | 2.39% | 12.05% | 11.20% | 95% | 50% |
| Class C | 1.57% | 8.72% | 7.39% | 2.19% | 10.18% | 10.34% | 95% | 50% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 2.18% | 9.11% | 7.05% | 2.70% | 11.00% | 9.99% | 95% | #DIV/0! |
| Class D | 1.41% | 6.80% | 5.70% | 1.64% | 8.37% | 8.06% | 95% | 49% |
| Class F | 1.03% | 3.79% | 2.84% | 1.76% | 7.63% | 7.75% | 91% | 43% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 1.67% | 8.76% | 7.14% | 2.35% | 4.00% | 6.14% | 99% | 52% |
| Class E | 2.05% | 5.07% | 3.09% | 3.09% | 0.67% | 0.73% | 87% | 43% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 1.68% | 8.61% | 7.64% | 2.11% | 0.30% | 6.77% | 97% | 50% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1.93% | 7.48% | 8.02% | 1.80% | 9.39% | 9.84% | 100% | 56% |
| Class A2 | 1.39% | 7.55% | 5.10% | 1.98% | 8.84% | 6.33% | 107% | 59% |
| Class B | 1.24% | 7.54% | 7.17% | 1.53% | 10.28% | 10.44% | 107% | 61% |
| Class C | 1.17% | 6.61% | 6.17% | 1.56% | 10.09% | 10.56% | 104% | 62% |
| Class E | 1.90% | 8.98% | 10.11% | 2.80% | 10.45% | 13.08% | 100% | 57% |
| Overall | 1.47% | 7.57% | 7.23% | 1.87% | 9.88% | 10.12% | 104% | 59% |
| Class D | 0.91% | 4.68% | 4.67% | 1.19% | 8.02% | 9.01% | 98% | 65% |
| Class F | 0.88% | 3.78% | 3.02% | 1.25% | 7.71% | 8.03% | 100% | 55% |

Note: Results from InterDigital, crosschecked by xxxx.

## Other tools

Other results remain the same as tools were not changed. Please note that VTM anchor and NNVC variants results changed due to MCTF improvement.

# Contributions

We have xx contributions for AhG14.

# Configurations

The following configurations is used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration | Tested | Xcheck |
| --- | --- | --- | --- | --- |
| NNVC VTM mode | none | encoder_xxx_vtm.cfg | Y |  |
| NNVC Anchor/EE1 | Intra Pred + LOP.3 | encoder_xxx_nnvc.cfg | Y |  |
| NNVC. HOP | Intra Pred + HOP.4 | encoder_xxx_nnvc.cfg + nn-based/HOP4.cfg | Y |  |
| HOP only | HOP | encoder_xxx_vtm.cfg + nn-based/HOP4.cfg |  |  |
| Intra lower complexity | NN Intra lower complexity | nn-based/intra2.cfg |  |  |
| Sr | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg |  |  |
| Pf | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg |  |  |
| ALOP | Intra pred+adaptive LOP | encoder_xxx_vtm.cfg +nn-based/intra2.cfg+xxx |  |  |

Deprecated options:

| set0 | Loop filter set #0 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg | P |  |
| --- | --- | --- | --- | --- |
| set1 | Loop filter set #1 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg | P |  |
| set0+rdo | Loop filter set #0 + Rdo | nn-based/NnlfOption_1.cfg + --EncNnlfOpt=1 | P |  |
| set1+rdo+intra+temporal filter | Loop filter set #1 + Temporal filter | nn-based/NnlfOption_2.cfg + --NnlfSet1Multiframe=1 | P |  |

# Recommendations

The AHG recommends to:

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

Encourage people to submit merge requests fixing identified bugs.