| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0046 |
| --- | --- |

| Title: | EE2-3.1: High Precision Sample Based Bi-Directional Optical Flow |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yuwen He, Kai Zhang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Email: | mehdi.salehifar@bytedance.com yuwen.he@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports test results for EE2-3.1.

In test EE2-3.1 (JVET-AH0178), high precision parameter derivation, which is used for decoder side motion vector refinement (DMVR), i.e. BDOF DMVR, is reused to derive accurate sample adjustment.

In test a, the summations for deriving the parameters are weighted sum, however in test b, the weighted sum is not used.

On top of the ECM-13.0, simulation results of the proposed method are reported as:

Test 3.1a:

RA: -0.10%, -0.09%, -0.08%, EncT: 101.1%, DecT: 100.5%, EncVmPeak: 100.0%, DecVmPeak: 100.0%.

Test 3.1b:

RA: -0.03%, -0.03%, -0.00%, EncT: 101.0%, DecT: 100.6%, EncVmPeak: 100.0%, DecVmPeak: 100.0%.

# Introduction

A high-precision equation to derive the BDOF MV refinement parameters was adopted into ECM as JVET-AD0195 [1] based on a high precision formula.

Later an iterative BDOF was proposed and adopted into ECM as JVET-AE0065 [2], adding another BDOF-based motion refinement pass as the 4th pass of the multi-pass DMVR.

Finally, improvements for DMVR were proposed and adopted into ECM as JVET-AG0067 [3], using the mean removed equations to derive the BDOF DMVR parameters as well as adding the 16×16 BDOF DMVR layer.

# Proposed method

In this contribution, as it was reported in JVET-AH0178 [4], it is proposed to reuse the high precision formula used for BDOF DMVR for sample based BDOF too. Specifically:

(Gx.Gx+R1) * vx + Gx.Gy * vy = dI . Gx - dM . Gx

Gx.Gy * vx + (Gy.Gy+R1) * vy = dI . Gy - dM . Gy

where Gx/Gy are the summation of the two horizontal/vertical gradients derived for each reference block.

Summations () are weighted sums in test a, and not weighted sum in test b.

# Experimental results

The proposed method has been implemented on top of the ECM-13.0 [5]. Testing results under RA configuration is summarized in Tables I-II. The simulations were performed following the common test conditions [6]. More detailed results can be found in the attached spreadsheets.

Table I: Performance of Test EE2-3.1a

Table II: Performance of Test EE2-3.1b

# Conclusions

In this contribution, improvement for sample based BDOF is proposed. It provides -0.10% RA gain with 101% encoding, and 100% decoding time. It is recommended to adopt this method in ECM-14.0.

# References

[1] M. Salehifar, Y. He, K. Zhang, L. Zhang, “EE2-2.6: High-Precision MV Refinement for BDOF,” JVET-AD0195, Apr. 2023.

[2] 	M. Salehifar, Y. He, K. Zhang, H. Liu, L. Zhang, J. Chen, R.-L. Liao, X. Li, Y. Ye, “EE2-3.5: Iterative BDOF pass in multi-pass DMVR”, JVET-AE0065, July 2023.

[3] M. Salehifar, Y. He, K. Zhang, L. Zhang, “EE2-2.8: On DMVR Extensions,” JVET-AG0067, Jan. 2024.

[4] M. Salehifar, Y. He, K. Zhang, L. Zhang, “Non-EE2: High Accuracy Sample Based Bi-Directional Optical Flow,” JVET-AH0178, Apr. 2024.

[5] ECM-13.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

[6] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).