| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0084-v2 |
| --- | --- |

| Title: | EE2 test-5.2: ALF residuals scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Franck Galpin, Guillaume Boisson, Fabrice Urban (InterDigital), Nan Hu, Vadim Seregin, Marta Karczewicz (Qualcomm) | Email: | philippe.bordes@interdigital.com, nanh@qti.qualcomm.com |
| Source: | InterDigital, Qualcomm |  |  |

_____________________________

# Abstract

The EE2 test-5.2 proposes an ALF correction (residuals) scaling method. The scaling factors are coded per slice for the non-fixed APS-ALF. The proposed method was implemented on top of ECM-13.0, it reportedly provides {Y, U, V} BD-reduction with encoder and decoder runtimes as follows:

RA: -0.05%, -0.09%, -0.07%, 99.9% (EncT), 100.0 % (DecT),

LDB: -0.13%, -0.15%, -0.10%, 100.0% (EncT), 99.9% (DecT),

LDP: -0.16%, 0.01%, -0.01%, 100.0% (EncT), 100.1% (DecT).

# Introduction

The APS-ALF signals non-fixed coefficients that may be re-used for the subsequent pictures. However, these coefficients have been optimized by encoder for one frame and may not be perfectly tailored for other frames with different characteristics and coding parameters.

In NNVC software, when a NN filter is being applied to reconstructed pictures, a scaling factor is signaled for each color component in the picture header. The differences between the input samples and the NN filtered samples (residuals) are scaled by these scaling factors before being added to the input samples [1].

# Proposal

The EE2 test-5.2 proposes to signal in the slice header the scaling factors to be applied on the non-fixed APS-ALF correction residuals. Different luma scaling factors may be associated with different group of class indexes:

For each alternative filter set of each APS-ALF used in the current slice:

- A flag signals if the previous scaling factor values should be used for the current filter. If the TiD of the current slice is inferior or equal to the TiD of the APS, then the scaling factor values are up-dated.

- an index idxGroup signals the number num_sc of scaling factors (num_sc = 1 << idxGroup). One scaling factor is associated to one group of consecutive classes.

- num_sc scaling factors indexes ( idxScale[n] ) are coded. The scaling factor idxScale[n] is associated to nth group of classes. Then a scaling factor index sfi[c] is derived for each of the 25 class indexes ‘c’. For example:

sfi[.]=3333333333333333333333333	{ idxGroup = 0, idxScale={3}		}

sfi[.]=0000000000004444444444444	{ idxGroup = 1, idxScale={0,4}		}

sfi[.]=0000004444442222221111111	{ idxGroup = 2, idxScale={0,4,2,0}	}

- for each chroma component, one single scaling factor index is signaled.

- the mapping scaling factors indexes to scaling factors is derived using one pre-defined table signaled in SPS: tab(nbCorr=5)={ 8, 7, 6, 5, 9 } or tab(nbCorr=9)={ 8, 7, 6, 5, 4, 2, 9, 10, 12 }.

- the ALF residual correction corr(s) is scaled using the scaling factor associated to the class index of the sample (class(s)) and added to the reconstructed sample:

rec’(s) = rec(s) + ( corr(s) * tab[ sfi[ class(s) ] ] + 4 ) >> 3

# Results

The test-5.2 has been implemented in ECM-13.0 and tested with CTC with RA/LDB/LDP conditions.

Table 1: Performance results of proposal in RA.

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DncVmPeak |
| Class A1 | -0,05% | -0,21% | -0,12% | 100,1% | 100,1% | 100,2% | 100,0% |
| Class A2 | -0,05% | -0,16% | 0,07% | 99,8% | 99,8% | 100,2% | 99,7% |
| Class B | -0,02% | -0,04% | -0,13% | 99,9% | 100,0% | 100,0% | 100,0% |
| Class C | -0,07% | -0,01% | -0,05% | 100,1% | 100,2% | 100,0% | 100,2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,05% | -0,09% | -0,07% | 99,9% | 100,0% | 100,1% | 100,0% |
| Class D | -0,12% | -0,04% | -0,02% | 99,9% | 100,1% | 99,9% | 100,0% |
| Class F | -0,02% | 0,01% | 0,03% | 100,2% | 100,0% | 100,1% | 100,1% |
| Class TGM | -0,07% | 0,02% | 0,04% | 100,3% | 99,9% | 100,0% | 97,9% |

Table 2: Performance results of proposal in LDB.

|  | Low Delay B Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DncVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,11% | -0,60% | -0,01% | 99,8% | 100,0% | 100,0% | 100,2% |
| Class C | -0,12% | -0,14% | -0,40% | 100,1% | 99,9% | 100,0% | 100,0% |
| Class E | -0,17% | 0,60% | 0,14% | 100,2% | 99,8% | 100,4% | 100,3% |
| Overall | -0,13% | -0,15% | -0,10% | 100,0% | 99,9% | 100,1% | 100,1% |
| Class D | -0,23% | -1,14% | -0,67% | 100,2% | 99,5% | 100,6% | 100,0% |
| Class F | -0,21% | -0,08% | -0,78% | 100,0% | 100,8% | 100,4% | 99,9% |
| Class TGM | -0,19% | -0,05% | -0,14% | 99,9% | 100,0% | 100,0% | 95,5% |

Table 3: Performance results of proposal in LDP.

|  | Low Delay P Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DncVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,13% | 0,44% | 0,07% | 100,2% | 100,1% | 100,1% | 100,3% |
| Class C | -0,10% | -0,43% | 0,26% | 99,9% | 100,2% | 100,0% | 100,0% |
| Class E | -0,29% | -0,11% | -0,50% | 100,1% | 100,1% | 100,1% | 100,2% |
| Overall | -0,16% | 0,01% | -0,01% | 100,0% | 100,1% | 100,1% | 100,2% |
| Class D | -0,09% | 0,49% | -0,98% | 100,0% | 100,0% | 100,2% | 100,0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Conclusion

This EE2 test-5.2 signals scaling factors for non-fixed APS-ALF. The results show that the proposed method can increase the compression performance of ECM-13.0 without added encoding or decoding complexity. It is proposed to adopt this method in ECM.

# References

- Y.Li, K.Zhang, L.Zhang (Bytedance), H.Wang, M.Coban, A.Kotra, M. Karczewicz(Qualcomm), F.Galpin (InterDigital), “EE1-1.6: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling based on SADL implementation,” document JVET-Z0112, 26th Meeting, by teleconference, 20–29 April 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).