| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0086-v3 |
| --- | --- |

| Title: | EE2-2.1/2.5 related: History-based merge candidate for merge-based intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes a history-based merge candidate for merge-based intra prediction. The merge-based intra prediction is tested as EE2-2.1/2.5 in this meeting, which constructs an intra merge candidate list by adding intra modes used in TIMD-coded neighboring blocks and select a merge candidate to predict the current block. Apart from the common aspect, EE2-2.5 considers DIMD-coded blocks. To derive five intra merge candidates of the list, EE2-2.1/2.5a extends the number of checks for non-adjacent spatial merge candidates from 18 of ECM-13.0 to 87, whereas EE2-2.25c maintains the same number as ECM-13.0. To avoid the vast of checking non-adjacent spatial merge candidates in EE2-2.1/2.5a, it is proposed that history-based merge candidates are alternatively introduced without extending non-adjacent spatial merge candidates. Derived history-based merge candidates are added to the history buffer and inserted to the intra merge candidate lists when the list is vacant after deriving non-adjacent spatial merge candidates.

On top of the ECM-13.0, simulation results {Y, U, V, EncT, DecT} for overall test sequences in CTC are reported below:

Only TIMD-coded blocks are considered the same as EE2-2.1:

AI: {%, %, %, %, %};

TIMD and DIMD coded blocks are considered the same as EE2-2.5:

AI: {-0.07%, 0.03%, -0.01%, 104.3%, 101.0%};

Introduction

Merge-based intra prediction is tested as EE2-2.1/2.5 [2] in this meeting, which constructs an intra merge candidate list by adding intra modes used in TIMD-coded neighboring blocks and select a merge candidate to predict the current block. Apart from the common aspect, EE2-2.5 considers DIMD-coded blocks. EE2-2.1/2.5a [2] extends the number of checks for non-adjacent spatial merge candidates from 18 of ECM-13.0 to 87, whereas EE2-2.5c [2] maintains the same number as ECM-13.0 [3]. However, the vast checks of non-adjacent spatial merge candidates with pruning process in EE2-2.1/2.5a should be avoided for increasing complexity.

Proposed method

This contribution proposes to introduce a history-based merge candidate for merge-based intra prediction. Specifically, a set of intra prediction modes and weights of TIMD or DIMD coded blocks is added to history buffer as a history-based merge candidate. The history buffer updates the candidates when adding new candidate with pruning process and clear the candidates at the end of a CTU line. When the list is vacant after deriving non-adjacent spatial merge candidates, derived history-based merge candidates are inserted.

Experimental results

The proposed method was implemented on top of EE2-2.1 and EE2-2.5c [2], and tested according to the common test condition (CTC) of all intra and random access configurations as specified in the JVET-AF2017 [5] and JVET-AH2024 [2]. Experimental results of the proposed method over ECM-13.0 are summarized in Table 1 and Table 2. Table 1 and Table 2 correspond to the proposed method based on EE2-2.1 and EE2-2.1c.

Table 1 Experimental results of the proposed method considering only TIMD-coded blocks the same as EE2-2.1 over ECM-13.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.02% | 0.07% | 0.06% | 102.0% | 102.0% | 100.3% | 100.0% |
| Class F |  |  |  |  |  |  |  |

Table 2 Experimental results of the proposed method considering TIMD and DIMD coded blocks the same as EE2-2.5 over ECM-13.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% | -0.04% | 0.02% | 104.2% | 101.0% | 98.4% | 100.1% |
| Class A2 | -0.07% | -0.01% | -0.04% | 103.8% | 101.0% | 99.6% | 100.0% |
| Class B | -0.07% | -0.01% | 0.00% | 104.5% | 101.1% | 99.7% | 100.1% |
| Class C | -0.06% | 0.03% | -0.01% | 104.3% | 101.0% | 100.1% | 100.0% |
| Class E | -0.07% | 0.23% | -0.04% | 104.5% | 100.9% | 100.1% | 100.0% |
| Overall | -0.07% | 0.03% | -0.01% | 104.3% | 101.0% | 99.6% | 100.1% |
| Class D | -0.02% | 0.17% | -0.03% | 104.2% | 100.9% | 100.2% | 100.0% |
| Class F | -0.05% | -0.03% | -0.04% | 102.0% | 100.6% | 100.0% | 100.0% |

Supplemental results

Experimental results of EE2-2.1, EE2-2.5a and EE2-2.5c over ECM-13.0 obtained under the proponent’s computation environments are summarized in Table 3, Table 4, and Table 5, following CTC.

Table 3 Experimental results of EE2-2.1 over ECM-13.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.10% | -0.02% | 102.8% | 102.0% | 99.9% | 100.5% |
| Class A2 | -0.07% | 0.03% | -0.02% | 102.6% | 102.4% | 100.2% | 100.3% |
| Class B | -0.07% | -0.04% | 0.00% | 103.2% | 103.2% | 101.0% | 100.3% |
| Class C | -0.08% | 0.09% | -0.01% | 102.7% | 104.7% | 100.8% | 100.2% |
| Class E | -0.06% | 0.14% | 0.01% | 102.9% | 102.9% | 101.4% | 100.1% |
| Overall | -0.07% | 0.05% | -0.01% | 102.9% | 103.2% | 100.7% | 100.3% |
| Class D | -0.04% | 0.02% | -0.05% | 102.5% | 104.6% | 100.3% | 100.0% |
| Class F | -0.04% | 0.07% | 0.06% | 101.6% | 102.3% | 99.7% | 100.2% |

Table 4 Experimental results of EE2-2.5a over ECM-13.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | -0.01% | 0.03% | 104.1% | 101.0% | 101.6% | 100.1% |
| Class A2 | -0.08% | 0.02% | 0.06% | 103.7% | 100.8% | 99.0% | 100.0% |
| Class B | -0.06% | -0.03% | -0.09% | 104.6% | 100.8% | 99.9% | 100.0% |
| Class C | -0.05% | 0.10% | 0.11% | 104.1% | 100.9% | 100.1% | 100.1% |
| Class E | -0.06% | 0.04% | -0.01% | 104.7% | 100.7% | 100.9% | 100.0% |
| Overall | -0.07% | 0.02% | 0.01% | 104.3% | 100.8% | 100.2% | 100.0% |
| Class D | -0.03% | -0.04% | -0.08% | 104.1% | 100.7% | 100.1% | 100.0% |
| Class F | -0.06% | 0.12% | 0.05% | 102.1% | 100.3% | 101.5% | 100.0% |

Table 5 Experimental results of EE2-2.5c over ECM-13.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | 0.05% | 0.03% | 103.9% | 100.8% | 99.1% | 100.2% |
| Class A2 | -0.06% | 0.03% | 0.02% | 103.4% | 100.8% | 98.6% | 100.0% |
| Class B | -0.06% | -0.07% | -0.07% | 104.3% | 100.7% | 96.7% | 100.1% |
| Class C | -0.05% | 0.03% | 0.12% | 103.9% | 100.7% | 99.6% | 100.1% |
| Class E | -0.05% | 0.10% | 0.01% | 104.3% | 100.7% | 100.1% | 100.0% |
| Overall | -0.06% | 0.02% | 0.02% | 104.0% | 100.8% | 98.6% | 100.1% |
| Class D | -0.04% | 0.20% | 0.06% | 104.0% | 100.8% | 100.1% | 100.0% |
| Class F | -0.03% | -0.09% | 0.06% | 101.8% | 100.2% | 100.0% | 100.0% |

Experimental results of the proposed method over ECM-13.0 are summarized in Table 6 and Table 7, where the subsampling factor is increased 8 times of CTC, i.e., only every 64th frame is coded. Table 6 and Table 7 correspond to the proposed method based on EE2-2.1 and EE2-2.1c.

Table 6 Experimental results of the proposed method considering only TIMD-coded blocks the same as EE2-2.1 over ECM-13.0, where subsampling factor is increased by 8 times of CTC.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.13% | 0.14% | 102.3% | 101.1% | 99.9% | 100.4% |
| Class A2 | -0.05% | 0.23% | 0.10% | 102.1% | 101.0% | 99.9% | 100.5% |
| Class B | -0.06% | 0.13% | -0.18% | 102.7% | 101.6% | 99.4% | 100.3% |
| Class C | -0.06% | 0.01% | 0.06% | 102.0% | 101.6% | 100.8% | 100.2% |
| Class E | 0.00% | 0.36% | -0.56% | 102.2% | 101.3% | 100.2% | 100.1% |
| Overall | -0.05% | 0.12% | -0.09% | 102.3% | 101.4% | 100.0% | 100.3% |
| Class D | 0.02% | -0.17% | -0.11% | 101.8% | 101.9% | 100.2% | 100.1% |
| Class F | 0.00% | 0.11% | -0.31% | 101.3% | 101.0% | 100.3% | 100.2% |

Table 7 Experimental results of the proposed method considering TIMD and DIMD coded blocks the same as EE2-2.5 over ECM-13.0, where subsampling factor is increased by 8 times of CTC.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.13% | 0.14% | 102.3% | 101.1% | 99.9% | 100.4% |
| Class A2 | -0.05% | 0.23% | 0.10% | 102.1% | 101.0% | 99.9% | 100.5% |
| Class B | -0.06% | 0.13% | -0.18% | 102.7% | 101.6% | 99.4% | 100.3% |
| Class C | -0.06% | 0.01% | 0.06% | 102.0% | 101.6% | 100.8% | 100.2% |
| Class E | 0.00% | 0.36% | -0.56% | 102.2% | 101.3% | 100.2% | 100.1% |
| Overall | -0.05% | 0.12% | -0.09% | 102.3% | 101.4% | 100.0% | 100.3% |
| Class D | 0.02% | -0.17% | -0.11% | 101.8% | 101.9% | 100.2% | 100.1% |
| Class F | 0.00% | 0.11% | -0.31% | 101.3% | 101.0% | 100.3% | 100.2% |

Conclusion

This contribution proposes a history-based merge candidate for intra merge mode. The proposed method brings additional coding gains over EE2-2.5c. It is recommended that the proposed method be adopted into the next ECM or next EE.

Reference

- M. Abdoli, R. G. Youvalari, A. Tissier, Y.-J. Chang, V. Seregin, M. Karczewicz, “EE2-2.1/EE2-2.5: Merge-based intra prediction,” JVET-AI0110, July 2024.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AH2024, April 2024.

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 13 (ECM 13),” JVET-AH2025, April 2024.

- https://vcgit.hhi.fraunhofer.de/ecm/jvet-ah-ee2/ECM/-/tree/TEST_2.5

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October. 2023.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).